=======================================================================DICTIN

DICTIN

PROGRAM DICTIN (Renamed from DICTION to eliminate conflict with   DICTIN

UNIX diction command - 12/22/02)                  DICTIN

==============                                                    DICTIN

VERSION 81-1 (SEPTEMBER 1981)                                     DICTIN

VERSION 82-1 (JANUARY 1982)                                       DICTIN

VERSION 83-1 (JANUARY 1983)  *KEEP ORIGINAL MOD. NUMBER           DICTIN

*NEW, MORE COMPATIBLE I/O UNITS.     DICTIN

VERSION 84-1 (SEPTEMBER 1984)*UPDATED TO HANDLE ENDF/B-VI FORMAT. DICTIN

(PROGRAM WILL NOW WORK ON ALL       DICTIN

VERSIONS OF THE ENDF/B FORMAT).    DICTIN

VERSION 85-1 (AUGUST 1985)   *FORTRAN-77/H VERSION                DICTIN

VERSION 86-1 (JANUARY 1986)  *MAT ORDER CHECK.                    DICTIN

*IF NO HOLLERITH SECTION COPY MAT.   DICTIN

VERSION 88-1 (JULY 1988)     *OPTION...INTERNALLY DEFINE ALL I/O  DICTIN

FILE NAMES (SEE, SUBROUTINE FILEIO  DICTIN

FOR DETAILS).                       DICTIN

*IMPROVED BASED ON USER COMMENTS.    DICTIN

VERSION 89-1 (JANUARY 1989)  *PSYCHOANALYZED BY PROGRAM FREUD TO  DICTIN

INSURE PROGRAM WILL NOT DO ANYTHING DICTIN

CRAZY.                              DICTIN

*IMPROVED BASED ON USER COMMENTS.    DICTIN

*ADDED LIVERMORE CIVIC COMPILER      DICTIN

CONVENTIONS.                        DICTIN

*UPDATED TO USE NEW PROGRAM CONVERT  DICTIN

KEYWORDS.                           DICTIN

VERSION 92-1 (JANUARY 1992)  *UPDATED BASED ON USER COMMENTS.     DICTIN

*UP TO 6000 SECTIONS PER TAPE.       DICTIN

*CHANGED DEFAULT MOD NUMBER FOR NEW  DICTIN

SECTIONS FROM 0 TO 1                DICTIN

VERSION 94-1 (JANUARY 1994)  *VARIABLE ENDF/B DATA FILENAMES      DICTIN

TO ALLOW ACCESS TO FILE STRUCTURES  DICTIN

(WARNING - INPUT PARAMETER FORMAT   DICTIN

HAS BEEN CHANGED)                   DICTIN

*CLOSE ALL FILES BEFORE TERMINATING  DICTIN

(SEE, SUBROUTINE ENDIT)             DICTIN

*ADDED FORTRAN SAVE OPTION           DICTIN

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    DICTIN

*IMPROVED COMPUTER INDEPENDENCE       DICTIN

*ALL DOUBLE PRECISION                 DICTIN

*ON SCREEN OUTPUT                     DICTIN

*UNIFORM TREATMENT OF ENDF/B I/O      DICTIN

*IMPROVED OUTPUT PRECISION            DICTIN

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      DICTIN

POINT READ FOR MORE DIGITS           DICTIN

*UPDATED TEST FOR ENDF/B FORMAT       DICTIN

VERSION BASED ON RECENT FORMAT CHANGEDICTIN

*GENERAL IMPROVEMENTS BASED ON        DICTIN

USER FEEDBACK                        DICTIN

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON        DICTIN

USER FEEDBACK                        DICTIN

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS            DICTIN

*RENAMED dictin TO ELIMINATE CONFLICT DICTIN

WITH UNIX diction COMMAND.           DICTIN

*ADDED DOCUMENTATION LINE TO COMMENTS.DICTIN

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACKDICTIN

*UP TO 100,000 SECTIONS PER TAPE.     DICTIN

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.      DICTIN

*UP TO 500,000 SECTIONS PER TAPE.     DICTIN

VERS. 2007-2 (DEC. 2007)    *72 CHARACTER FILE NAMES.             DICTIN

VERS. 2010-1 (Apr. 2010)    *General update based on user feedbackDICTIN

VERS. 2012-1 (Aug. 2012)    *Added CODENAME                       DICTIN

*32 and 64 bit Compatible             DICTIN

*Added ERROR stops                    DICTIN

VERS. 2015-1 (Jan. 2015)    *Corrected END = it was saying ERROR. DICTIN

*Replaced ALL 3 way IF Statements.    DICTIN

VERS. 2015-2 (Mar. 2015)    *Corrected Seqence Numbers            DICTIN

1) Restart at 1 for each MAT/MF/MT.  DICTIN

2) 99999 on section end, MT=0        DICTIN

3)     0 on MF = 0                   DICTIN

VERS. 2017-1 (May  2017)    *Updated based on user feedback.      DICTIN

VERS. 2018-1 (Jan. 2018)    *Added on-line output for ALL ENDERRORDICTIN

VERS. 2020-1 (Feb. 2020)    *Identical to 2018-1.                 DICTIN

DICTIN

2015-2 Acknowledgment                                             DICTIN

=====================                                             DICTIN

I thank Jean-Christophe Sublet (UKAEA) for contributing MAC       DICTIN

executables and Bojan Zefran (IJS, Slovenia) for contributing     DICTIN

LINUX (32 or 64 bit) executables. And most of all I must thank    DICTIN

Andrej Trkov (NDS, IAEA) for overseeing the entire PREPRO project DICTIN

at IAEA, Vienna. This was a truly International team who worked   DICTIN

together to produce PREPRO 2015-2.                                DICTIN

DICTIN

OWNED, MAINTAINED AND DISTRIBUTED BY                              DICTIN

------------------------------------                              DICTIN

THE NUCLEAR DATA SECTION                                          DICTIN

INTERNATIONAL ATOMIC ENERGY AGENCY                                DICTIN

P.O. BOX 100                                                      DICTIN

A-1400, VIENNA, AUSTRIA                                           DICTIN

EUROPE                                                            DICTIN

DICTIN

ORIGINALLY WRITTEN BY                                             DICTIN

------------------------------------                              DICTIN

Dermott E. Cullen                                                 DICTIN

DICTIN

PRESENT CONTACT INFORMATION                                       DICTIN

---------------------------                                       DICTIN

Dermott E. Cullen                                                 DICTIN

1466 Hudson Way                                                   DICTIN

Livermore, CA 94550                                               DICTIN

U.S.A.                                                            DICTIN

Telephone  925-443-1911                                           DICTIN

E. Mail    RedCullen1@Comcast.net                                 DICTIN

Website    RedCullen1.net/HOMEPAGE.NEW                            DICTIN

DICTIN

AUTHORS MESSAGE                                                   DICTIN

---------------                                                   DICTIN

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENATION   DICTIN

FOR THIS PROGRAM INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ   DICTIN

ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                      DICTIN

DICTIN

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTERDICTIN

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE    DICTIN

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECTDICTIN

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY      DICTIN

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO DICTIN

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF   DICTIN

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR        DICTIN

COMPUTER.                                                         DICTIN

DICTIN

PURPOSE                                                           DICTIN

-------                                                           DICTIN

THIS PROGRAM IS DESIGNED TO CREATE A REACTION INDEX FOR EACH      DICTIN

MATERIAL ON AN ENDF/B FORMATTED TAPE AND TO INSERT THIS REACTION  DICTIN

INDEX IN FILE 1, SECTION 451 OF EACH MATERIAL.                    DICTIN

DICTIN

IN THE DESCRIPTION THAT FOLLOWS FOR SIMPLICITY THE ENDF/B         DICTIN

TERMINOLOGY---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL      DICTIN

MEDIUM MAY BE TAPE, CARDS, DISK, OR ANY OTHER MEDIUM.             DICTIN

DICTIN

ENDF/B FORMAT                                                     DICTIN

-------------                                                     DICTIN

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS    DICTIN

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION  DICTIN

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT).DICTIN

DICTIN

THIS PROGRAM WILL AUTOMATICALLY DETERMINE WHICH VERSION OF THE    DICTIN

ENDF/B FORMAT EACH MAT IS IN AND WILL THEN PROPERLY REPLACE THE   DICTIN

REACTION INDEX FOR EACH MAT. DIFFERENT MATS ON THE SAME TAPE MAY  DICTIN

EVEN BE IN DIFFERENT VERSIONS OF THE ENDF/B FORMAT.               DICTIN

DICTIN

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B      DICTIN

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS    DICTIN

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE DICTIN

NUMBERS (COLUMNS 76-80) NEED NOT BE PRESENT ON INPUT, BUT WILL BE DICTIN

CORRECTLY OUTPUT ON ALL LINES.                                    DICTIN

DICTIN

ENDF/B FORMAT VERSION                                             DICTIN

---------------------                                             DICTIN

THE ENDF/B FORMAT CAN BE DETERMINED FROM THE SECOND LINE OF       DICTIN

THE HOLLERITH SECTION (MF=1, MT=451).                             DICTIN

ENDF/B-IV = N1 - LINE COUNT (POSITIVE)                            DICTIN

ENDFB/-V  = N1 = N2 =0                                            DICTIN

ENDF/B-VI = N1 =0, N2= VERSION NUMBER (6 OR MORE)                 DICTIN

DICTIN

SECTION SIZE                                                      DICTIN

------------                                                      DICTIN

SINCE THIS PROGRAM ONLY READS THE DATA ONE LINE AT A TIME THERE   DICTIN

IS NO LIMIT TO THE SIZE OF ANY GIVEN SECTION, E.G. THE TOTAL      DICTIN

CROSS SECTION MAY BE DESCRIBED BY 200,000 DATA POINTS.            DICTIN

DICTIN

NUMBER OF SECTIONS PER TAPE                                       DICTIN

---------------------------                                       DICTIN

IT IS ASSUMED THAT THE ENDF/B TAPE CONTAINS 100,000 OR FEWER      DICTIN

SECTIONS = 100,000 OR FEWER MAT,MF,MT COMBINATIONS. IF THIS LIMIT DICTIN

IS EXCEEDED THIS PROGRAM WILL TERMINATE EXECUTION. IF NEED BE THISDICTIN

LIMIT CAN EASILY BE CHANGED BY CHANGING THE DIMENSION STATEMENT   DICTIN

BELOW AND RE-DEFINING THE VARIABLE MAXTAB IN THE BELOW DATA       DICTIN

STATEMENT. ALTERNATIVELY THE ENDF/B TAPE MAY BE DIVIDED INTO A    DICTIN

NUMBER SMALLER TAPES EACH CONTAINING 100,000 OR FEWER SECTIONS.   DICTIN

EACH ENDF/B TAPE CAN THEN RUN THROUGH THIS PROGRAM AND THE OUTPUT DICTIN

FOR EACH ENDF/B TAPE CAN THEN BE RE-COMBINED (I.E., MERGED BACK   DICTIN

TOGETHER).                                                        DICTIN

DICTIN

HOLLERITH SECTION                                                 DICTIN

-----------------                                                 DICTIN

IF ANY MATERIAL DOES NOT INITIALLY CONATIN A SECTION MF=1, MT=451 DICTIN

A WARNING MESSAGE WILL BE PRINTED AND THE MATERIAL WILL BE COPIED.DICTIN

DICTIN

IF ANY MATERIAL INITIALLY CONTAINS A SECTION MF=1, MT=451 A NEW   DICTIN

REACTION INDEX WILL BE CREATED AND INSERTED. THE INITIAL SECTION  DICTIN

MF=1, MT=451 MAY OR MAY NOT CONTAIN A REACTION INDEX.             DICTIN

DICTIN

IF THE MATERIAL INITIALLY CONTAINS A REACTION INDEX IT WILL BE    DICTIN

USED TO DEFINE THE MOD NUMBER FOR CORRESPONDING SECTIONS IN THE   DICTIN

NEW REACTION INDEX (I.E. IF A SECTION FROM THE ORIGINAL REACTION  DICTIN

INDEX HAS THE SAME MF/MT NUMBERS AS A SECTION IN THE NEW REACTION DICTIN

INDEX THE MOD NUMBER FROM THE ORIGINAL REACTION INDEX WILL BE USEDDICTIN

IN THE NEW REACTION INDEX). OTHERWISE THE MOD NUMBER IN THE NEW   DICTIN

REACTION INDEX WILL BE SET EQUAL TO ZERO.                         DICTIN

DICTIN

PROGRAM OPERATION                                                 DICTIN

-----------------                                                 DICTIN

THE ENTIRE ENDF/B TAPE IS FIRST READ AND A DICTIONARY ENTRY IS    DICTIN

CREATED FOR EACH SECTION OF THE TAPE. THE ENDF/B TAPE IS THEN     DICTIN

REWOUND AND READ A SECOND TIME. DURING THIS SECOND PASS THE       DICTIN

DICTIONARY OF EACH MAT IS REPLACED. THIS VERSION OF DICTIN        DICTIN

DOES NOT USE SCRATCH FILES AND IS MORE EFFICIENT THAN EARLIER     DICTIN

VERSIONS OF DICTIN.                                               DICTIN

DICTIN

INPUT LINES                                                       DICTIN

-----------                                                       DICTIN

LINE   COLS.  DESCRIPTION                                       DICTIN

----   -----  ------------------------------                    DICTIN

1   1-60   ENDF/B INPUT DATA FILENAME                        DICTIN

(STANDARD OPTION = ENDFB.IN)                      DICTIN

2   1-60   ENDF/B OUTPUT DATA FILENAME                       DICTIN

(STANDARD OPTION = ENDFB.OUT)                     DICTIN

DICTIN

EXAMPLE INPUT NO. 1                                               DICTIN

-------------------                                               DICTIN

READ \ENDFB6\K300\ENDFB.IN AND WRITE \ENDFB\K300\ENDFB.OUT. THE   DICTIN

FOLLOWING 2 INPUT LINES ARE REQUIRED,                             DICTIN

DICTIN

\ENDFB6\K300\ENDFB.IN                                                 DICTIN

\ENDFB6\K300\ENDFB.OUT                                                DICTIN

DICTIN

EXAMPLE INPUT NO. 2                                               DICTIN

-------------------                                               DICTIN

USE THE DEFAULT FILENAMES TO READ ENDFB.IN AND WRITE ENDFB.OUT.   DICTIN

2 BLANK INPUT LINES ARE REQUIRED                                  DICTIN

DICTIN

INPUT FILES                                                       DICTIN

-----------                                                       DICTIN

UNIT  DESCRIPTION                                                 DICTIN

----  -----------                                                 DICTIN

2  INPUT PARAMETERS (BCD - 80 CHARACTERS/RECORD)               DICTIN

10  ORIGINAL TAPE OF ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)   DICTIN

DICTIN

OUTPUT FILES                                                      DICTIN

------------                                                      DICTIN

UNIT  DESCRIPTION                                                 DICTIN

----  -----------                                                 DICTIN

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                 DICTIN

11  FINAL TAPE OF ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)      DICTIN

DICTIN

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)   DICTIN

---------------------------------------------------------------   DICTIN

UNIT  FILE NAME                                                   DICTIN

----  ----------                                                  DICTIN

2   DICTIN.INP                                                  DICTIN

3   DICTIN.LST                                                  DICTIN

10   ENDFB.IN                                                    DICTIN

11   ENDFB.OUT                                                   DICTIN

DICTIN

=======================================================================DICTIN