=======================================================================FIXUP

FIXUP

PROGRAM FIXUP                                                     FIXUP

=============                                                     FIXUP

VERSION 84-1 (NOVEMBER 1984)                                      FIXUP

VERSION 86-1 (JANUARY 1986) *IMPROVED BASED ON USER COMMENTS      FIXUP

*FORTRAN-77/H VERSION                 FIXUP

VERSION 86-2 (JUNE 1986)    *ALLOW CREATION OF SECTIONS OF CROSS  FIXUP

SECTIONS WHICH ARE NOT PRESENT IN    FIXUP

THE ORIGINAL EVALUATION              FIXUP

VERSION 88-1 (JULY 1988)    *OPTION...INTERNALLY DEFINE REA I/O   FIXUP

FILE NAMES (SEE, SUBROUTINE FILEIO   FIXUP

FOR DETAILS).                        FIXUP

*IMPROVED BASED ON USER COMMENTS.     FIXUP

VERSION 89-1 (JANUARY 1989) *PSYCHOANALYZED BY PROGRAM FREUD TO   FIXUP

INSURE PROGRAM WILL NOT DO ANYTHING  FIXUP

CRAZY.                               FIXUP

*UPDATED TO USE NEW PROGRAM CONVERT   FIXUP

KEYWORDS.                            FIXUP

*ADDED LIVERMORE CIVIC COMPILER       FIXUP

CONVENTIONS.                         FIXUP

VERSION 89-2 (MARCH 1989)   *ADDED ENDF-6 SUMMATION RULES AND     FIXUP

DEFINED MF AND MT NUMBERS. PROGRAM   FIXUP

WILL NOW USE MF=1, MT=451 TO DEFINE  FIXUP

THE ENDF FORMAT OF THE DATA (E.G.,   FIXUP

ENDF-6 OR EARLIER) AND USE THE       FIXUP

CORRECT SUMMATION RULES FOR EACH     FIXUP

VERSION OF THE ENDF FORMAT. IF       FIXUP

MF=1, MT=451 IS NOT PRESENT PROGRAM  FIXUP

WILL USE ENDF-6 SUMMATION            FIXUP

CONVENTIONS AS A DEFAULT.            FIXUP

VERSION 90-1 (JUNE 1990)    *UPDATED BASED ON USER COMMENTS       FIXUP

*ADDED PHOTON INTERACTION, MF=23      FIXUP

VERSION 91-1 (JUNE 1991)    *ADDED FORTRAN SAVE OPTION            FIXUP

*NEW MORE CONSISTENT ENERGY OUTPUT    FIXUP

ROUTINE                              FIXUP

VERSION 92-1 (JANUARY 1992) *ADDED OPTION TO CALCULATE RATIOS,    FIXUP

E.G., CAPTURE/FISSION AND PRODUCTS,  FIXUP

NU-BAR*FISSION - AND OUTPUT THE      FIXUP

RESULTS IN THE ENDF FORMAT (SEE,     FIXUP

BELOW - CREATING RATIOS AND PRODUCTS)FIXUP

*ALLOW TOTAL NU-BAR (MF=1, MT=452) TO FIXUP

BE USED IN DEFINING RATIOS OR        FIXUP

PRODUCTS.                            FIXUP

*ALLOW ALL CROSS SECTIONS TO BE PUT   FIXUP

ON A UNIFORM ENERGY GRID.            FIXUP

*NOTE, CHANGE IN INPUT FORMAT FOR     FIXUP

RANGES OF MT NUMBERS                 FIXUP

*COMPLETELY CONSISTENT I/O ROUTINES - FIXUP

TO MINIMIZE COMPUTER DEPENDENCE.     FIXUP

VERSION 93-1 (JULY 1993)    *CORRECTED ALGORITHM TO CREATE UNIFORMFIXUP

ENERGY GRID.                         FIXUP

VERSION 94-1 (JANUARY 1993) *VARIABLE ENDF/B DATA FILENAMES       FIXUP

TO ALLOW ACCESS TO FILE STRUCTURES   FIXUP

(WARNING - INPUT PARAMETER FORMAT    FIXUP

HAS BEEN CHANGED)                    FIXUP

*INCREASED PAGE SIZE FROM 1002 TO     FIXUP

12000 DATA POINTS.                   FIXUP

*CLOSE ALL FILES BEFORE TERMINATING   FIXUP

(SEE, SUBROUTINE ENDIT)              FIXUP

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    FIXUP

*IMPROVED COMPUTER INDEPENDENCE       FIXUP

*ALL DOUBLE PRECISION                 FIXUP

*ON SCREEN OUTPUT                     FIXUP

*UNIFORM TREATMENT OF ENDF I/O        FIXUP

*IMPROVED OUTPUT PRECISION            FIXUP

*DEFINED SCRATCH FILE NAMES           FIXUP

*INCREASED PAGE SIZE FROM 12000 TO    FIXUP

36000 DATA POINTS.                   FIXUP

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      FIXUP

POINT READ FOR MORE DIGITS           FIXUP

*UPDATED TEST FOR ENDF FORMAT         FIXUP

VERSION BASED ON RECENT FORMAT CHANGEFIXUP

*GENERAL IMPROVEMENTS BASED ON        FIXUP

USER FEEDBACK                        FIXUP

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF-6, NOT 5, IF MISSING     FIXUP

MF=1, MT-451.                        FIXUP

*FIXED CREATION OF SECTIONS           FIXUP

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON        FIXUP

USER FEEDBACK                        FIXUP

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS            FIXUP

*SUMMATION RULES ARE DEFINED BASED    FIXUP

ON CONTENTS OF TABLES.               FIXUP

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACKFIXUP

*INCREASED PAGE SIZE FROM 36000 TO    FIXUP

60000 DATA POINTS.                   FIXUP

VERS. 2005-1 (JAN. 2005)    *UPDATED MT CREATION TO ALLOW MAT =0  FIXUP

INDICATING CREATE FOR ALL MATS.      FIXUP

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII DATA  FIXUP

*INCREASED PAGE SIZE FROM 60,000 TO   FIXUP

600,000 DATA POINTS.                 FIXUP

VERS. 2007-2 (OCT. 2007)    *ADDED MT=16 AS SUM MT=875 THRU 891   FIXUP

*72 CHARACTER FILE NAMES              FIXUP

VERS. 2010-1 (Apr. 2010)    *Defining cross sections by summation FIXUP

to now mandatory - either build-in   FIXUP

rules or by user input.              FIXUP

VERS. 2011-1 (March 2011)   *Added new MT # to allowed and        FIXUP

summation rules.                     FIXUP

VERS. 2012-1 (Aug.  2012)   *Corrected definition of MT=3 to avoidFIXUP

double counting of MT=18.            FIXUP

*Extended incident particle list to   FIXUP

include photon (ZA = 0).             FIXUP

*Added CODENAME                       FIXUP

*32 and 64 bit Compatible             FIXUP

*Added ERROR stops.                   FIXUP

VERS. 2015-1 (Jan.  2015)   *Extended OUT9.                       FIXUP

*Replaced ALL 3 way IF Statements     FIXUP

FIXUP

VERS. 2015-2 (Oct.  2015)   *Threshold Correction no longer       FIXUP

allowed = TOO DANGEROUS!!!           FIXUP

VERS. 2017-1 (May   2017)   *Updated based on user feekback       FIXUP

*Increased tables to 3,000,000.       FIXUP

*All floating input parameters changedFIXUP

to character input + IN9 conversion. FIXUP

*Ignore attempts to "correct" reactionFIXUP

threshold = cannot be done for       FIXUP

temperature dependent (MF=3) data.   FIXUP

VERS. 2017-2 (Oct.  2017)   *Updated to insure sharp edges for    FIXUP

photon interaction cross sections    FIXUP

MF=23.                               FIXUP

*Updated for ELECTRONS to create,     FIXUP

MF/MT=23/501 = Total                 FIXUP

MF/MT=23/522 = Total ionization      FIXUP

*Updated to define MF=26 and electron FIXUP

Cross Sections MT=526, 527, 528 as   FIXUP

LEGAL MF/MT Combinations.            FIXUP

VERS. 2018-1 (Jan.  2018)   *Decreased PAGE size from 2,700,000   FIXUP

to 1,800,000 - PAGE was too BIG for  FIXUP

many computers - forcing the code    FIXUP

to run VERY SLOWLY - smaller size    FIXUP

improves running time.               FIXUP

*Added on-line output for ALL ENDERRORFIXUP

VERS. 2019-1 (June  2019)   *Additional Interpolation Law Tests   FIXUP

*Print WARNING if ALL MTs in any      FIXUP

evaluation DO NOT ALL EXTEND to the  FIXUP

same Maximum Tabulated Energy =      FIXUP

in this case data above the lowest   FIXUP

common energy is identied as being   FIXUP

UNRELIABLE.                          FIXUP

VERS. 2019-2 (Oct.  2019)   *Corrected ERROR defining first point FIXUP

of each MT = first point was being   FIXUP

ERRONEOUSLY skipped (due to an ERROR FIXUP

in removing the previous THRESHOLD   FIXUP

"correction" test - which is no      FIXUP

longer allowed).                     FIXUP

VERS. 2020-1 (Dec.  2020)   *ZA & AWRE Correction Required.       FIXUP

*Extenstion to 20 MeV NOT ALLOWED.    FIXUP

*Threshold correction NOT ALLOWED.    FIXUP

*Added Target isomeric state          FIXUP

VERS. 2021-1 (Kan.  2021)   *Updated for FORTRAN 2018             FIXUP

*DELETED MT=3 = Nonelastic by adding  FIXUP

it to the built-in DELETION table -  FIXUP

to select set option 3 = 2 (use      FIXUP

built-in DELETION table). MT=3 is    FIXUP

never used in applications, and yet  FIXUP

can add an enormous volume to ENDF   FIXUP

format files: (MT=3) = (MT-1)-(MT-2),FIXUP

included ALL of the resonances from  FIXUP

capture, fission,.....               FIXUP

FIXUP

2019-2 Acknowledgment                                             FIXUP

=====================                                             FIXUP

I thank Jean-Christophe Sublet (NDS, IAEA, Vienna, Austria) for   FIXUP

reporting the ERROR in FIXUP (2019-1) that led to the update in   FIXUP

FIXUP (2019-2) to correctly define the first point in each MT.    FIXUP

FIXUP

OWNED, MAINTAINED AND DISTRIBUTED BY                              FIXUP

------------------------------------                              FIXUP

THE NUCLEAR DATA SECTION                                          FIXUP

INTERNATIONAL ATOMIC ENERGY AGENCY                                FIXUP

P.O. BOX 100                                                      FIXUP

A-1400, VIENNA, AUSTRIA                                           FIXUP

EUROPE                                                            FIXUP

FIXUP

ORIGINALLY WRITTEN BY                                             FIXUP

------------------------------------                              FIXUP

Dermott E. Cullen                                                 FIXUP

FIXUP

PRESENT CONTACT INFORMATION                                       FIXUP

---------------------------                                       FIXUP

Dermott E. Cullen                                                 FIXUP

1466 Hudson Way                                                   FIXUP

Livermore, CA 94550                                               FIXUP

U.S.A.                                                            FIXUP

Telephone  925-443-1911                                           FIXUP

E. Mail    RedCullen1@Comcast.net                                 FIXUP

Website    RedCullen1.net/HOMEPAGE.NEW                            FIXUP

FIXUP

PURPOSE                                                           FIXUP

=======                                                           FIXUP

THIS PROGRAM IS DESIGNED TO READ EVALUATED DATA IN THE ENDF       FIXUP

FORMAT, PERFORM CORRECTIONS AND OUTPUT THE RESULT IN THE ENDF     FIXUP

FORMAT. TWO TYPES OF CORRECTIONS ARE POSSIBLE (1) AUTOMATIC AND   FIXUP

(2) OPTIONAL (BASED ON USER INPUT) CORRECTIONS.                   FIXUP

FIXUP

ONE OF THE MOST IMPORTANT FUNCTIONS OF THIS PROGRAM IS TO         FIXUP

RE-DEFINE ALL REDUNDANT CROSS SECTIONS (E.G. TOTAL) TO BE EXACTLY FIXUP

EQUAL TO THE SUM OF ITS PARTS. THIS PROCEDURE ELIMINATES THE      FIXUP

PROBLEM WITH MANY ENDF EVALUATIONS, WHERE DUE TO THE USE OF       FIXUP

NON-LINEAR INTERPOLATION LAWS THE TOTAL MAY BE EQUAL TO THE SUM   FIXUP

OF ITS PARTS AT ALL TABULATED ENERGIES, BUT BASED ON THE          FIXUP

INTERPOLATION LAWS IT CAN BE QUITE DIFFERENT AT ENERGIES BETWEEN  FIXUP

TABULATED ENERGIES.                                               FIXUP

FIXUP

AUTOMATIC CHECKS/CORRECTIONS                                      FIXUP

============================                                      FIXUP

(1) CHECK THAT MAT/MF/MT DOES NOT CHANGE UNLESS A MEND/FEND/SEND  FIXUP

LINE IS READ. IF MAT/MF/MT CHANGES A WARNING MESSAGE IS       FIXUP

PRINTED BUT NO CORRECTIVE ACTION IS TAKEN.                    FIXUP

(2) ALL LINES WITHIN A GIVEN MAT WILL BE SEQUENTIALLY NUMBERED    FIXUP

ON OUTPUT.                                                    FIXUP

FIXUP

OPTIONAL CHECKS/CORRECTIONS                                       FIXUP

===========================                                       FIXUP

THE FOLLOWING NUMBERS CORRESPOND TO THE INPUT DATA OPTION COLUMNS FIXUP

(SEE THE DESCRIPTION OF THE INPUT BELOW)                          FIXUP

FIXUP

(1) CORRECT ZA AND AWR IN ALL SECTIONS. CHECK TO INSURE THAT THE  FIXUP

C1 AND C2 VALUES (ZA AND AWR) ARE THE SAME IN ALL SECTIONS.   FIXUP

THE C1 AND C2 OF THE FIRST SECTION READ ARE ASSUMED TO BE     FIXUP

CORRECT AND ARE USED FOR COMPARISON. IF THE C1 AND/OR C2 OF   FIXUP

THE FIRST SECTION ARE NOT POSITIVE AN ERROR MESSAGE IS OUTPUT FIXUP

AND THE MATERIAL IS COPIED WITHOUT CHANGE.                    FIXUP

NOTE....TO CHANGE THE ZA AND/OR AWR OF ANY MATERIAL IT IS     FIXUP

MERELY NECESSARY TO CHANGE THE ZA AND/OR AWR IN THE FIRST     FIXUP

SECTION OF THE MATERIAL AND USE THIS OPTION TO AUTOMATICALLY  FIXUP

CHANGE ALL OTHER SECTIONS.                                    FIXUP

FIXUP

2017/5/20 - This option (2) is no longer allowed              FIXUP

FIXUP

WARNING: Threshold Correction is no longer allowed.           FIXUP

This option has resulted in far too much             FIXUP

misinterpretation and as such it is judged to        FIXUP

be too dangerous to be allowed in this code.         FIXUP

For example, the Laboratory frame of reference       FIXUP

threshold is temperature dependent = it is not       FIXUP

uniquely defined by Q value and atomic weight.       FIXUP

ThIS OPTION is IGNORED.                              FIXUP

FIXUP

(2) CORRECT CROSS SECTION (MF=3) THRESHOLDS. THE Q-VALUE AND AWR  FIXUP

ARE USED TO DERIVE THE REACTION THRESHOLD USING THE RELATION, FIXUP

FIXUP

E-THRESHOLD = -(Q-VALUE)*(AWRE+1.0)/AWRE                      FIXUP

FIXUP

IF THE THRESHOLD IS POSITIVE THE CROSS SECTION IS CHECKED TO  FIXUP

INSURE THAT THE FIRST TABULATED POINT IS AT THE THRESHOLD AND FIXUP

HAS A ZERO CROSS SECTION. IF NOT, THE CROSS SECTION WILL BE   FIXUP

CHANGED.                                                      FIXUP

(A) IF THE FIRST TABULATED POINT IS ABOVE THE THRESHOLD AND   FIXUP

HAS A ZERO CROSS SECTION, THE POINT IS DELETED AND A POINTFIXUP

IS INSERTED AT THE THRESHOLD.                             FIXUP

(B) IF THE FIRST TABULATED POINT IS ABOVE THE THRESHOLD AND   FIXUP

HAS A NON-ZERO CROSS SECTION, A POINT WITH ZERO CROSS     FIXUP

SECTION IS INSERTED AT THE THRESHOLD.                     FIXUP

(C) IF THE FIRST TABULATED POINT IS BELOW THE THRESHOLD AND   FIXUP

HAS A NON-ZERO CROSS SECTION, ALL POINTS BELOW THE        FIXUP

THRESHOLD ARE DELETED AND A POINT WITH ZERO CROSS SECTION FIXUP

IS INSERTED AT THE THRESHOLD.                             FIXUP

FIXUP

2017/5/20 - This option (2) is no longer allowed              FIXUP

FIXUP

(3) EXTEND ALL CROSS SECTIONS (MF=3) TO 20 MEV. IF THE TABULATED  FIXUP

CROSS SECTION ENDS BELOW 20 MEV IT WILL BE EXTENDED TO 20 MEV FIXUP

AS EITHER ZERO (IMOPS(3)=1) OR CONSTANT (IMOPS(3)=2) EQUAL    FIXUP

TO THE LAST TABULATED VALUE.                                  FIXUP

(4) ALLOW REACTION (MF=3, ANY MT) DELETION. ALL SPECIFIED         FIXUP

REACTIONS WILL BE DELETED WHEN THE DATA IS READ FROM THE      FIXUP

INPUT ENDF DATA FILE AND WILL NOT BE IN THE OUTPUT ENDF       FIXUP

DATA FILE. WARNING DELETED REACTIONS MAY NOT BE USED TO DEFINEFIXUP

ANY RECONSTRUCTED REACTIONS (I.E. REACTIONS DEFINED BY SUMMINGFIXUP

OTHER REACTIONS). SINCE DELETED REACTIONS ARE DELETED DURING  FIXUP

READING IT IS AS IF THEY NEVER EXISTED AND IF ANY DELETED     FIXUP

REACTION IS REQUIRED LATER TO DEFINE ANY SUM AN ERROR WILL    FIXUP

RESULT. THE USER MAY SPECIFY THAT THE DELETION RULES ARE TO BEFIXUP

READ FROM INPUT (IMOPS(4)=1) OR THAT THE BUILT IN SUMMATION   FIXUP

RULES ARE TO BE USED (MOPS(4)=2). AT THE PRESENT TIME THE     FIXUP

BUILT-IN DELETION RULES ARE THAT NO SECTIONS SHOULD BE DELETEDFIXUP

(THE USER MAY OVERRIDE THIS CONVENTION BY INPUT).             FIXUP

(5) ALLOW REACTION (MF=3, ANY MT) RECONSTRUCTION BY SUMMING OTHER FIXUP

REACTIONS. IN ORDER TO OPTIMIZE THE RUNNING TIME OF THIS      FIXUP

PROGRAM CARE SHOULD BE EXERCISED TO MINIMIZE THE NUMBER OF    FIXUP

TIMES THAT EACH CONTRIBUTING CROSS SECTION MUST BE USED.      FIXUP

THE USED MAY SPECIFY THAT THE SUMMATION RULES ARE TO BE READ  FIXUP

AS INPUT (IMOPS(5)=1) OR THAT THE BUILT IN SUMMATION RULES    FIXUP

ARE TO BE USED (IMOPS(5)=2). THE BUILT IN SUMMATION RULES ARE FIXUP

DESIGNED TO USE ENDF CONVENTIONS AND TO MINIMIZE THE NUMBER   FIXUP

OF TIMES THAT EACH CROSS SECTION IS USED.                     FIXUP

(6) INSURE THAT ALL CROSS SECTIONS ARE NON-NEGATIVE (I.E. ARE     FIXUP

ZERO OR POSITIVE). DURING READING ALL NEGATIVE CROSS SECTIONS FIXUP

WILL BE SET EQUAL TO ZERO AND TREATED AS SUCH DURING ALL      FIXUP

SUBSEQUENT SUMMATIONS AND ENDF OUTPUT.                        FIXUP

NOTE...THIS OPTION SHOULD NEVER BE USED WITH DATA CONTAINING  FIXUP

BACKGROUND CROSS SECTIONS WHICH MAY BE NEGATIVE. ONLY AFTER   FIXUP

THE RESONANCE CONTRIBUTION HAS BEEN ADDED TO THE BACKGROUND   FIXUP

TO DEFINE THE ACTUAL CROSS SECTION IS IT VALID TO ELIMINATE   FIXUP

NEGATIVE CROSS SECTIONS.                                      FIXUP

NOTE...THIS OPTION MAY BE USED TO DELETE NEGATIVE ELASTIC     FIXUP

CROSS SECTIONS THAT MAY RESULT FROM RECONSTRUCTING CROSS      FIXUP

SECTIONS FROM SINGLE LEVEL BREIT-WIGNER PARAMETERS. IF THE    FIXUP

TOTAL CROSS SECTION IS THEN RECONSTRUCTED USING THE CORRECTED FIXUP

ELASTIC CROSS SECTION THE TOTAL WILL BE POSITIVE DUE TO THE   FIXUP

CONTRIBUTIONS OF CAPTURE AND FISSION (THUS AVOIDING NUMERICAL FIXUP

INSTABILITY PROBLEMS DURING SELF-SHIELDING CALCULATIONS).     FIXUP

(7) WITHIN EACH SECTION OF CROSS SECTIONS DELETE ENERGIES THAT    FIXUP

ARE NOT IN ASCENDING ENERGY ORDER (ENERGY REPETITION IS O.K.) FIXUP

(8) WITHIN EACH SECTION OF CROSS SECTIONS ELIMINATE DUPLICATE     FIXUP

POINTS (SUCCESSIVE POINTS WITH THE SAME ENERGY-CROSS SECTION).FIXUP

(9) TEST THAT ALL SECTIONS ARE IN ASCENDING MAT/MF/MT ORDER.      FIXUP

IF NOT, NO CORRECTIVE ACTION WILL BE TAKEN, ONLY AN ERROR     FIXUP

MESSAGE WILL BE OUTPUT.                                       FIXUP

(10) CHECK MF/MT FOR EACH SECTION TO INSURE THAT THEY ARE DEFINED  FIXUP

IN THE ENDF FORMAR MANUAL. IF THEY ARE NOT DEFINED AN ERROR   FIXUP

MESSAGE IS PRINTED, BUT NO CORRECTIVE ACTION IS TAKEN.        FIXUP

(11) ALLOW SECTIONS WHICH ARE NOT PRESENT IN THE ORIGINAL (INPUT)  FIXUP

EVALUATION TO BE CREATED. NORMALLY THIS PROGRAM WILL ONLY     FIXUP

RECONSTRUCT AND OUTPUT SECTIONS IF THE SECTION IS PRESENT     FIXUP

IN THE ORIGINAL EVALUATION. THIS PROCEDURE IS FOLLOWED BECAUSEFIXUP

NORMALLY THE PROGRAM DOES NOT KNOW HOW TO DEFINE THE CONTENTS FIXUP

OF THE FIRST TWO LINES OF THE SECTION (E.G., Q-VALUE,         FIXUP

TEMPERATURE, INITIAL AND FINAL STATES). THIS OPTION MAY BE    FIXUP

USED TO ALLOW THE PROGRAM TO READ AND SAVE A TABLE DEFINING   FIXUP

THE CONTENTS OF THE FIRST TWO LINES OF EACH SECTION TO BE     FIXUP

CREATED.                                                      FIXUP

NOTE...IF A SECTION IS PRESENT ANY COMMAND TO CREATE IT WILL  FIXUP

BE IGNORED.                                                   FIXUP

(12)ALLOW ENERGY POINTS TO BE INSERTED. THE PROGRAM CAN READ UP   FIXUP

TO 50, ENERGIES, MAT, MT AND USE LINEAR INTERPOLATION TO      FIXUP

INSERT ENERGY POINTS INTO TABLES AS THEY ARE READ, E.G.,      FIXUP

INSERT AN ENERGY POINT AT THERMAL ENERGY (0.0253 EV). IF      FIXUP

AN MAT AND/OR MT IS ZERO THIS IMPLIES = ALL - INSERT THE      FIXUP

ENERGY IN ALL TABLES.                                         FIXUP

(13)PUT ALLOW CROSS SECTIONS ON A UNIFORM ENERGY GRID = EACH      FIXUP

SECTION (MT) OF CROSS SECTIONS WILL INCLUDE ALL ENERGIES      FIXUP

WHICH APPEAR IN AT LEAST ONE SECTION OF DATA. PARAMETERS      FIXUP

(MT=251 THROUGH 255) ARE NOT INCLUDED IN THE UNIFORM ENERGY   FIXUP

GRID.                                                         FIXUP

(14)DELETE SECTION IF CROSS SECTION = 0 AT ALL ENERGIES. THIS     FIXUP

SOUNDS LIKE AN ABSURD OPTION, BUT IS REQUIRED BECAUSE SUCH    FIXUP

SECTIONS EXIST IN ENDF/B-VI DATA.                             FIXUP

FIXUP

CREATING RATIOS AND PRODUCTS                                      FIXUP

============================                                      FIXUP

IN ORDER TO CREATE RATIOS AND PRODUCTS = NEW MT NUMBERS, YOU MUST FIXUP

DO TWO THINGS,                                                    FIXUP

FIXUP

1) DEFINE EACH NEW MT NUMBER AS A RATIO OR PRODUCT OF TWO MT      FIXUP

NUMBERS.                                                       FIXUP

FIXUP

2) USE THE CREATE MT NUMBER OPTION AND INPUT THE FIRST TWO LINES  FIXUP

OF THE SECTION                                                 FIXUP

FIXUP

WARNING - UNLESS YOU DO BOTH OF THESE YOU WILL NOT OBTAIN OUTPUT  FIXUP

IN THE ENDF FORMAT.                                               FIXUP

FIXUP

TWO SPECIAL MT NUMBERS HAVE BEEN DEFINED BY CSEWG INVOLVING       FIXUP

RATIOS AND PRODUCTS,                                              FIXUP

FIXUP

ALPHA (MT=254)= CAPTURE (MT=102)/FISSION (MT=18)                  FIXUP

FIXUP

ETA (MT=255)  = NU-BAR (MT=452)*FISSION (MT=18)/ABSORPTION (MT=27)FIXUP

FIXUP

ABSORPTION (MT=27) = FISSION (MT=18) + SUM (MT=102 THROUGH 116)   FIXUP

FIXUP

AS YET THERE IS NO STANDARD DEFINITION OF MT NUMBERS FOR RATIO    FIXUP

OR PRODUCT DATA. YOU ARE FREE TO USE ANY MT NUMBERS NORMALLY NOT  FIXUP

USED IN THE ENDF. HOWEVER, IT WILL THEN BE YOUR RESPONSIBILITY    FIXUP

TO PROPERLY INTERPRET THE RESULTS, I.E., NOBODY ELSE WILL HAVE    FIXUP

ANY IDEA HOW TO INTERPRET A TABLE OF DATA ASSOCIATED WITH THE MT  FIXUP

NUMBERS YOU HAVE USED.                                            FIXUP

FIXUP

THIS PROGRAM CAN BE ONLY DIRECTLY DEFINE RATIOS AND PRODUCTS      FIXUP

USING TWO MT NUMBERS = BINARY OPERATIONS, E.G., DEFINE THE CAPTUREFIXUP

TO FISSION RATIO, OR DEFINE THE PRODUCT NU-BAR*FISSION.           FIXUP

FIXUP

THIS PROGRAM CANNOT DIRECTLY DEFINE RATIO OR PRODUCT OF A SUM OF  FIXUP

SECTIONS TO THE SUM OF ANOTHER SET OF SECTIONS. HOWEVER, THIS CAN FIXUP

BE DONE INDIRECTLY BY FIRST DEFINING A DUMMY MT NUMBER (ANY MT    FIXUP

NUMBER NOT NORMALLY USED IN ENDF) TO BE A SUM OF SECTIONS AND     FIXUP

A SECOND DUMMY MT NUMBER TO BE A SECOND SUM OF SECTIONS. YOU CAN  FIXUP

THEN DEFINE RATIO OR PRODUCT YOU REQUIRE TO BE THE RATIO OF THESE FIXUP

TWO DUMMY MT NUMBERS.                                             FIXUP

FIXUP

FOR EXAMPLE, TO DEFINE ETA,                                       FIXUP

1) FIRST DEFINE (MT=27) = (MT=27) + (SUM OF MT=102 THROUGH 116)   FIXUP

2) NEXT DEFINE (MT=333) = (MT=452)*(MT=18)                        FIXUP

3) LAST DEFINE (MT=255) = (MT=333)/(MT=27)                        FIXUP

DO NOT FORGET TO TURN ON THE CREATE SECTION OPTION (ON THE FIRST  FIXUP

INPUT LINE) AND INPUT THE FIRST TWO LINES OF SECTION MT=255 -     FIXUP

OTHERWISE YOU WILL NOT GET ANY ENDF FORMATTED OUTPUT.             FIXUP

FIXUP

THE ONLY SPECIAL CONVENTIONS USED BY THIS PROGRAM IN CALCULATING  FIXUP

RATIOS ARE WHEN THE DENOMINATOR OF THE RATIO IS ZERO. IN THIS     FIXUP

CASE IF THE NUMERATOR IS ALSO ZERO THE RATIO IS DEFINED TO BE ONE.FIXUP

IN THIS CASE IF THE NUMERATOR IS NOT ZERO THE RATIO IS DEFINED    FIXUP

TO BE ZERO.                                                       FIXUP

FIXUP

ENDF FORMAT                                                       FIXUP

===========                                                       FIXUP

THIS PROGRAM MAY BE USED WITH DATA IN ANY VERSION OF THE ENDF     FIXUP

FORMAT (I.E. ENDF-1, 2, 3, 4, 5 OR 6 FORMAT). SINCE A             FIXUP

PAGING SYSTEM IS USED STORE CROSS SECTION TABLES ON SCRATCH FILES FIXUP

THERE IS NO LIMIT TO THE SIZE OF TABLES (E.G. THE TOTAL CROSS     FIXUP

SECTION MAY BE REPRESENTED BY 200,000 TABULATED POINTS).          FIXUP

FIXUP

WARNING                                                           FIXUP

=======                                                           FIXUP

(1) FOR EACH SECTION OF CROSS SECTIONS (I.E. EACH MT, MF=3) IN    FIXUP

THE ORIGINAL EVALUATION (I.E. ENDF/B DATA READ) ONE SECTION   FIXUP

OF DATA WILL BE OUTPUT, UNLESS THE SECTION HAS BEEN DELETED.  FIXUP

THIS INCLUDES ANY SECTIONS WHICH ARE NOT PRESENT IN THE       FIXUP

ORIGINAL EVALUATION, BUT THE USER INDICATES (BY INPUT) SHOULD FIXUP

BE CREATED.                                                   FIXUP

FIXUP

THE PROGRAM WILL NOT OUTPUT ANY SECTION RECONSTRUCTED BY      FIXUP

SUMMATION UNLESS THE CORRESPONDING SECTION (MT NUMBER) IS     FIXUP

PRESENT IN THE ORIGINAL EVALUATION OR USER INPUT INDICATES    FIXUP

SHOULD BE CREATED AND OUTPUT. THIS IS (A) BECAUSE THE         FIXUP

PROGRAM CANNOT DEFINE THE PARAMETERS TO APPEAR ON THE FIRST   FIXUP

TWO LINES OF THE SECTION, (B) TO AVOID OUTPUTTING TOO MUCH    FIXUP

DATA WHICH THE USER MAY NOT BE INTERESTED IN.                 FIXUP

FIXUP

(2) FOR ANY SECTIONS THAT DO NOT APPEAR IN THE ORIGINAL DATA THE  FIXUP

USER MAY SPECIFY THAT THEY BE DEFINED BY SUMMATION. ANY SUCH  FIXUP

SECTION MAY BE USED BE DEFINE SUBSEQUENT SUMS, BUT THE SECTIONFIXUP

ITSELF WILL NOT BE OUTPUT (E.G. GENERALLY MT=27 AND 101 ARE   FIXUP

NOT PRESENT IN EVALUATIONS. HOWEVER, THE BUILT-IN SUMMATION   FIXUP

RULES OF THIS PROGRAM USES THE ENDF SUMMATION RULES TO        FIXUP

DEFINE MT=27 AND 101, WHICH IN TURN ARE USED TO DEFINE THE    FIXUP

NON-ELASTIC CROSS SECTION, MT=3. SECTIONS MT=27 AND 101 ARE   FIXUP

NOT OUTPUT).                                                  FIXUP

FIXUP

(3) ALL DATA IN FILE 3 AND 23 MUST BE LINEARLY INTERPOLABLE. IF   FIXUP

THE DATA IS NOT LINEARLY INTERPOLABLE THIS PROGRAM WILL       FIXUP

TERMINATE.                                                    FIXUP

FIXUP

PROGRAM OPERATION                                                 FIXUP

=================                                                 FIXUP

ALL MAT NUMBER ON AN ENDF TAPE ARE PROCESSED. EACH MAT IS         FIXUP

TREATED SEPARATELY. WITHIN EACH MAT, EACH SECTION BEFORE MF=3     FIXUP

IS READ, CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND OUTPUT.   FIXUP

WHEN MF=3 IS LOCATED ALL CROSS SECTIONS ARE READ, SECTIONS TO BE  FIXUP

DELETED ARE DELETED, SECTIONS WHICH ARE NOT PRESENTED AND USER    FIXUP

INPUT INDICATES SHOULD BE CREATED ARE CREATE, SECTIONS TO BE KEPT FIXUP

ARE CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND WRITTEN TO A   FIXUP

SCRATCH FILE. NEXT, IF THE USER SPECIFIES THAT THEY SHOULD,       FIXUP

SECTIONS ARE RECONSTRUCTED. FINALLY ALL CROSS SECTIONS (OLD AND   FIXUP

NEW) ARE OUTPUT. WITHIN THE SAME MAT, EACH SECTION AFTER MF=3 IS  FIXUP

READ, CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND OUTPUT.      FIXUP

FIXUP

MF=3                                                              FIXUP

====                                                              FIXUP

THE TREATMENT OF THE CROSS SECTIONS REQUIRES UP TO 4 PASSES FOR   FIXUP

CROSS SECTIONS. IN THE PROGRAM THEY CORRESPOND TO SUBROUTINES     FIXUP

PASS1, PASS2, PASS3 AND PASS4. THE ORIGINAL AND FINAL ENDF DATA   FIXUP

FILES, 5 SCRATCH FILES AND 3 IN CORE ARRAYS ARE USED. OPERATIONS  FIXUP

PERFORMED DURING EACH PASS ARE,                                   FIXUP

FIXUP

PASS1                                                             FIXUP

=====                                                             FIXUP

READ ALL CROSS SECTIONS FROM ITAPE. DELETED ANY SECTIONS. CREATE  FIXUP

ANY SECTIONS. CHECK/CORRECT THEM AND WRITE THEM TO SCRATCH FILE.  FIXUP

DATA IS READ INTO ARRAY A, TRANSFERRED TO ARRAY C (AFTER EDITING) FIXUP

AND OUTPUT TO ISCRC FROM ARRAY C.                                 FIXUP

ITAPE  - UNIT ORIGINAL ENDF DATA IS READ FROM.                    FIXUP

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS WRITTEN ON.             FIXUP

TABA   - ARRAY INTO WHICH ORIGINAL DATA IS READ.                  FIXUP

TABC   - ARRAY INTO WHICH EDITED DATA IS TRANSFERRED TO AND       FIXUP

FROM WHICH IT IS WRITTEN TO ISCRC.                       FIXUP

FIXUP

PASS2                                                             FIXUP

=====                                                             FIXUP

IF A UNIFORM ENERGY GRID IS REQUESTED IT IS CREATED DURING THIS   FIXUP

PASS. FIRST ALL OF THE CROSS SECTIONS FROM PASS1 ARE READ AND A   FIXUP

UNIFORM ENERGY GRID IS CREATED = ALL ENERGIES THAT ARE INCLUDED   FIXUP

IN AT LEAST ONE SECTION (MT) OF CROSS SECTIONS.                   FIXUP

ISCRA  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.             FIXUP

ISCRB  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.             FIXUP

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS READ FROM.              FIXUP

TABA   - ARRAY CONTAINING UNIFORM ENERGY GRID.                    FIXUP

TABB   - ARRAY CONTAINING UNIFORM ENERGY GRID.                    FIXUP

TABC   - ARRAY CONTAINING EDITED DATA.                            FIXUP

FIXUP

THE UNIFORM ENERGY GRID ENDS UP ON ISCRB. NEXT EACH SECTION OF    FIXUP

CROSS SECTIONS FROM PASS1 IS READ FROM ISCRC, INTERPOLATED TO     FIXUP

THE UNIFORM ENERGY GRID AND OUTPUT TO ISCRA. FINALLY ISCRA AND    FIXUP

ISCRC ARE SWITCH, SO THAT AT THE END OF THIS PASS THE DATA WILL   FIXUP

AGAIN BE ON ISCRC (EXACTLY AS AT THE END OF PASS1), WITH UPDATED  FIXUP

POINT COUNTS.                                                     FIXUP

ISCRA  - SCRATCH UNIT THAT UNIFORM ENERGY GRID DATA IS WRITTEN ON.FIXUP

ISCRB  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.             FIXUP

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS READ FROM.              FIXUP

TABA   - ARRAY CONTAINING UNIFORM ENERGY GRID DATA.               FIXUP

TABB   - ARRAY CONTAINING UNIFORM ENERGY GRID.                    FIXUP

TABC   - ARRAY CONTAINING EDITED DATA.                            FIXUP

FIXUP

PASS3                                                             FIXUP

=====                                                             FIXUP

SUMMATION CROSS SECTIONS ARE DEFINED BY READING DATA FROM ISCRC   FIXUP

AND MERGING THEM ONTO ISCRA. THE FIRST SECTION THAT CONTRIBUTES   FIXUP

TO A SUM IS MERELY COPIED FROM C TO A. IF MORE SECTIONS WILL      FIXUP

CONTRIBUTE TO THE SUM THE DATA IN A IS TRANSFERRED TO B, A        FIXUP

SECTION OF DATA FROM C IS ADDED TO THE DATA IN B AND STORED IN    FIXUP

A. THE CYLE OF ADDED C AND B TO A, FOLLOWED BY MOVING A TO B      FIXUP

IS CONTINUED UNTIL ALL CONTRIBUTING SECTIONS HAVE BEEN ADDED.     FIXUP

THE SUM IS THEN COPIED FROM A TO D. IF NEWLY CONSTRUCTED SECTION  FIXUP

IS REQUIRED FOR ANY LATER SUMMUATIONS IT IS ALSO COPIED TO E.     FIXUP

THE CYCLE OF ADDED SECTIONS FROM C AND B TO A IS REPEATED FOR     FIXUP

EACH REQUIRED SUMMATION REACTION. IN ADDITION TO SECTIONS FROM    FIXUP

C, AFTER THE FIRST SUMMATION SECTIONS MAY ALSO BE ADDED TO A      FIXUP

FROM E (THE CONTRIBUTION OF NEW RECONSTRUCTED CROSS SECTIONS).    FIXUP

WHEN ALL REQUIRED SECTIONS HAVE BEEN RECONSTRUCTED THE NEW        FIXUP

SECTIONS WILL BE ON E AND THE ORIGINAL SECTIONS ON C.             FIXUP

ISCRC  - SCRATCH FILE FROM WHICH ORIGINAL DATA IS READ.           FIXUP

ISCRA  - SCRATCH FILE ONTO WHICH SUM FOR ONE SECTION IS WRITTEN.  FIXUP

ISCRD  - SCRATCH FILE ONTO WHICH ALL SUM CROSS SECTIONS ARE       FIXUP

WRITTEN.                                                 FIXUP

ISCRE  - SCRATCH FILE ONTO WHICH ALL SUM CROSS SECTIONS WHICH     FIXUP

ARE REQUIRED FOR LATER SUMS ARE WRITTEN.                 FIXUP

ISCRB  - UTILITY SCRATCH FILE USED TO CREATE SUM CROSS SECTIONS.  FIXUP

TABA   - ARRAY INTO WHICH SUMS ARE WRITTEN.                       FIXUP

TABB   - ARRAY INTO WHICH PARTIAL SUMS ARE WRITTEN.               FIXUP

TABC   - ARRAY INTO WHICH ORIGINAL DATA IS READ.                  FIXUP

FIXUP

PASS4                                                             FIXUP

=====                                                             FIXUP

CROSS SECTIONS ARE READ FROM ISCRC (ORIGINAL) AND ISCRD (NEW)     FIXUP

AND ARE WRITTEN IN THE ENDF FORMAT ON OTAPE. THE BEGINNING OF     FIXUP

EACH SECTION OF ORIGINAL DATA IS READ FROM ISCRC (TO DEFINE       FIXUP

SECTION HEADER INFORMATION). IF THIS MT HAS NOT BEEN RECOSTRUCTED FIXUP

ON ISCRD THE ORIGINAL SECTION IS OUTPUT. IF THE SECTION HAS BEEN  FIXUP

RECONSTRUCTED THE ORIGINAL SECTION IS SKIPPED AND THE NEW SECTION FIXUP

IS OUTPUT.                                                        FIXUP

OTAPE  - OUTPUT DATA IN THE ENDF FORMAT.                          FIXUP

ISCRC  - SCRATCH FILE FROM WHICH ORIGINAL DATA IS READ.           FIXUP

ISCRD  - SCRATCH FILE FROM WHICH NEW DATA IS READ.                FIXUP

TABC   - ARRAY INTO WHICH CROSS SECTIONS ARE READ FROM SCRATCH    FIXUP

AND WRITTEN TO OTAPE                                     FIXUP

FIXUP

I/O FILE DEFINITIONS                                              FIXUP

====================                                              FIXUP

UNIT   DESCRIPTION                                                FIXUP

====   ===========                                                FIXUP

2    INPUT PARAMETERS.                                          FIXUP

3    OUTPUT REPORT.                                             FIXUP

10    ORIGINAL DATA IN THE ENDF FORMAT.                          FIXUP

11    FINAL DATA IN THE ENDF FORMAT.                             FIXUP

12    SCRATCH FILE                                               FIXUP

14    SCRATCH FILE                                               FIXUP

15    SCRATCH FILE                                               FIXUP

16    SCRATCH FILE                                               FIXUP

17    SCRATCH FILE                                               FIXUP

FIXUP

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)   FIXUP

===============================================================   FIXUP

UNIT  FILE NAME   FORMAT                                          FIXUP

====  ==========  ======                                          FIXUP

2   FIXUP.INP    BCD                                            FIXUP

3   FIXUP.LST    BCD                                            FIXUP

10   ENDFB.IN     BCD                                            FIXUP

11   ENDFB.OUT    BCD                                            FIXUP

12-17  (SCRATCH)   BINARY                                          FIXUP

FIXUP

INPUT LINES                                                       FIXUP

===========                                                       FIXUP

LINE  COLUMNS  FORMAT  DESCRIPTION                                FIXUP

====  =======  ======  ===========                                FIXUP

1    1-14     14I1   INPUT OPTIONS AS DESCRIBED ABOVE.          FIXUP

EACH COLUMN OF THE INPUT LINE CONTROLS     FIXUP

ONE OF THE TESTS/CORRECTIONS DESCRIBED     FIXUP

ABOVE. TESTS/CORRECTION 1-14 (NOT ALL      FIXUP

IMPLEMENTED YET) CORRESPOND TO COLUMNS     FIXUP

1-14 OF THIS INPUT LINE AND ARE TREATED    FIXUP

AS FOLLOWS,                                FIXUP

= 0 - DO NOT PERFORM TEST/CORRECTION.      FIXUP

= 1 - PERFORM TEST/CORRECTION.             FIXUP

FOR MT EXCLUSION FROM THRESHOLD TESTS      FIXUP

(COLUMN 2), DELETION (COLUMN 4), OR        FIXUP

SUMMATION (COLUMN 5) THE INPUT OPTION      FIXUP

MAY BE,                                    FIXUP

= 1 - READ RULES FROM INPUT                FIXUP

= 2 - USE BUILT-IN RULES                   FIXUP

2    1-72      A72   ENDF INPUT DATA FILENAME                   FIXUP

(STANDARD OPTION = ENDFB.IN)               FIXUP

3    1-72      A72   ENDF OUTPUT DATA FILENAME                  FIXUP

(STANDARD OPTION = ENDFB.OUT)              FIXUP

4-M    1-5      FREE   CHARACTER (S,D,T,R,*) FOLLOWED BY BLANK OR FIXUP

FORM   MT NUMBER                                  FIXUP

- THE ALLOWED CHARACTERS ARE,              FIXUP

- S OR BLANK = SUM (OR DIFFERENCES)        FIXUP

- D = DELETE                               FIXUP

- T = NO THRESHOLD ENERGY CORRECTIONS      FIXUP

- R = RATIO                                FIXUP

- * = PRODUCT                              FIXUP

6-72      FREE   UP TO 10 LOWER AND UPPER MT RANGES WHICH   FIXUP

FORM   WILL BE USED TO DEFINE THE RECONSTRUCTED   FIXUP

CROSS SECTION OR TO DEFINE MT RANGES WHICH FIXUP

ARE EXCLUDED FROM THRESHOLD TESTS.         FIXUP

FIXUP

EACH MT NUMBER IS DEFINED BY A CONTINUOUS  FIXUP

STRING OF DIGITS, POSSIBILITY PRECEEDED BY FIXUP

A - (MINUS SIGN). EACH MT NUMBER MUST BE   FIXUP

BLANK OR OTHERWISE (NOT A DIGIT) DELIMITED.FIXUP

FIXUP

COLUMNS 6-72 MAY CONTAIN STRINGS OF DIGITS FIXUP

THE FIRST DIGIT STRING OF EACH PAIR MAY BE FIXUP

PRECEEDED BY A - (MINUS SIGN).             FIXUP

FIXUP

EACH LINE WILL BE INTERPRETED AS FOLLOWS,  FIXUP

FIXUP

*SUMMATION (OR DIFFERENCES)                 FIXUP

--------------------------                 FIXUP

COLUMNS 1-5 = S OR BLANK FOLLOWED BY THE   FIXUP

MT NUMBER TO BE DEFINED BY SUMMATION       FIXUP

FIXUP

COLUMNS 6-72 = UP TO 10 MT RANGE (PAIRS OF FIXUP

MT NUMBERS) TO BE USED TO DEFINED THE SUM. FIXUP

IF THE FIRST MT NUMBER OF A PAIR IS        FIXUP

NEGATIVE THE RANGE OF MT NUMBERS IS        FIXUP

SUBTRACTED - AT LEAST ONE RANGE MUST BE    FIXUP

SPECIFIED.                                 FIXUP

FIXUP

*DELETIONS                                  FIXUP

---------                                  FIXUP

COLUMNS 1-5 = D FOLLOWED BY BLANKS         FIXUP

FIXUP

COLUMNS 6-72 CONTAIN UP TO 10 MT RANGE     FIXUP

(PAIRS OF MT NUMBERS), EACH RANGE DEFINING FIXUP

A RANGE OF MT NUMBERS TO BE DELETED - AT   FIXUP

LEAST ONE RANGE MUST BE SPECIFIED.         FIXUP

FIXUP

*EXCLUSION FROM THRESHOLD TESTS             FIXUP

------------------------------             FIXUP

COLUMNS 1=5 = T FOLLOWED BY BLANKS         FIXUP

FIXUP

COLUMNS 6-72 CONTAIN UP TO 10 MT RANGE     FIXUP

(PAIRS OF MT NUMBERS), EACH RANGE DEFINING FIXUP

A RANGE OF MT NUMBERS WHOSE THRESHOLD      FIXUP

ENERGY WILL NOT BE CHECKED - AT LEAST ONE  FIXUP

RANGE MUST BE SPECIFIED.                   FIXUP

FIXUP

*RATIO                                      FIXUP

-----                                      FIXUP

COLUMNS 1-5 = R FOLLOWED BY THE MT NUMBER  FIXUP

TO BE DEFINED BY A RATIO                   FIXUP

FIXUP

COLUMNS 6-72 CONTAINS 2 MT NUMBERS TO BE   FIXUP

USED TO DEFINE THE RATIO.                  FIXUP

FIXUP

*PRODUCT                                    FIXUP

-----                                      FIXUP

COLUMNS 1-5 = * FOLLOWED BY THE MT NUMBER  FIXUP

TO BE DEFINED BY A PRODUCT                 FIXUP

FIXUP

COLUMNS 6-72 CONTAINS 2 MT NUMBERS TO BE   FIXUP

USED TO DEFINE THE PRODUCT.                FIXUP

FIXUP

CONVENTIONS                                FIXUP

-----------                                FIXUP

*UP TO 20 DELETIONS AND 20 SUMMATIONS OR    FIXUP

RATIOS OR PRODUCTS MAY BE SPECIFIED.       FIXUP

*ONLY 1 EXCLUSION FROM THRESHOLD TESTS      FIXUP

MAY BE SPECIFIED (THE 1 LINE MAY CONTAIN   FIXUP

UP TO 10 MT RANGES TO EXCLUDE FROM TESTS). FIXUP

*INPUT IS TERMINATED BY INPUTTNG 0 OR       FIXUP

BLANK IN COLUMNS 1-72 (I.E. THE LAST       FIXUP

INPUT LINE MUST BE BLANK).                 FIXUP

*THE UPPER LIMIT OF EACH RANGE MUST BE AT   FIXUP

LEAST AS BIG AS THE LOWER LIMIT (IN        FIXUP

ABSOLUTE VALUE).                           FIXUP

*FOR RECONSTRUCTION POSITIVE MT RANGES WILL FIXUP

BE ADDED TO THE SUM AND NEGATIVE MT RANGES FIXUP

WILL BE SUBTRACTED.                        FIXUP

*IF INPUT OPTION 2 (FIRST INPUT LINE) IS    FIXUP

0 THRESHOLD EXCLUSION IS NOT ALLOWED.      FIXUP

*IF INPUT OPTION 4 (FIRST INPUT LINE) IS    FIXUP

0 DELETIONS ARE NOT ALLOWED.               FIXUP

*IF INPUT OPTION 5 (FIRST INPUT LINE) IS    FIXUP

0 SUMMATIONS AND RATIOS ARE NOT ALLOWED.   FIXUP

N-K                    IF THE USER SPECIFIES THAT SECTIONS WHICH  FIXUP

ARE NOT PRESENT IN THE ORIGINAL EVALUATION FIXUP

MAY BE CREATED, TWO LINES MUST BE INPUT FORFIXUP

EACH SECTION TO BE CREATED. THE TWO LINES  FIXUP

DEFINE (C1, C2, L1 AND L2) FOR EACH OF THE FIXUP

FIRST TWO LINES OF THE SECTION TO BE       FIXUP

CREATED. THE FIRST LINE ALSO DEFINES (MAT  FIXUP

AND MT). (N1, N2) ARE ALWAYS ZERO ON THE   FIXUP

FIRST LINE AND WILL BE CALCULATED BY THE   FIXUP

PROGRAM FOR THE SECOND LINE.               FIXUP

FIRST    1-11     E11.4  ZA OF SECTION TO BE CREATED                FIXUP

LINE    12-22     E11.4  AWRE OF SECTION TO BE CREATED              FIXUP

23-33      I11   L1 OF SECTION TO BE CREATED                FIXUP

34-44      I11   L2 OF SECTION TO BE CREATED                FIXUP

45-48      I4    MAT OF SECTION TO BE CREATED               FIXUP

49-51      I3    MT OF SECTION TO BE CREATED                FIXUP

SECOND    1-11     E11.4  C1 OF SECTION TO BE CREATED                FIXUP

LINE    12-22     E11.4  C2 OF SECTION TO BE CREATED                FIXUP

23-33      I11   L1 OF SECTION TO BE CREATED                FIXUP

34-44      I11   L2 OF SECTION TO BE CREATED                FIXUP

*PAIRS OF LINES MAY BE IN ANY MAT/MT ORDER  FIXUP

(E.G., THEY NEED NOT BE IN ASCENDING       FIXUP

MAT/MT ORDER).                            FIXUP

*UP TO 50 PAIRS OF LINES MAY BE USED TO     FIXUP

DEFINE SECTIONS TO BE CREATED. THE LIST    FIXUP

IS TERMINATED WHEN THE FIRST LINE OF A     FIXUP

PAIR CONTAINS A ZERO (OR BLANK) MAT AND/OR FIXUP

MT.                                        FIXUP

M-N                    IF THE USER SPECIFIES THAT ENERGIES WHICH  FIXUP

ARE NOT PRESENT IN THE ORIGINAL EVALUATION FIXUP

MAY BE INSERTED, ONE LINE MUST BE INPUT FORFIXUP

EACH ENERGY TO BE INSERTED.                FIXUP

1-11    E11.4   ENERGY TO BE INSERTED                      FIXUP

12-15     I4     MAT IN WHICH TO INSERT ENERGY = 0 = ALL    FIXUP

16-18     I3     MT IN WHICH TO INSERT ENERGY = 0 = ALL     FIXUP

*UP TO 50 (ENERGY, MAT, MT) LINES MAY BE    FIXUP

USED. THE LIST IS TERMINATED BY A BLANK    FIXUP

LINE.                                      FIXUP

*INPUT MAY BE IN ANY (ENERGY, MAT, MT)      FIXUP

ORDER.                                     FIXUP

*ENERGY POINTS CAN ONLY BE INSERTED WITHIN  FIXUP

THE ORIGINAL ENERGY RANGE OF A SECTION -   FIXUP

THIS OPTION CANNOT BE USED TO EXTEND THE   FIXUP

CROSS SECTION EITHER BELOW OR ABOVE THE    FIXUP

ORIGINAL TABULATED ENERGY RANGE.           FIXUP

FIXUP

EXAMPLE INPUT NO. 1                                               FIXUP

===================                                               FIXUP

(1) USE OPTIONS 1-11 (ALL OPTIONS, EXCEPT INSERT ENERGY POINTS)   FIXUP

(2) DELETE MT=900 (FOR EXAMPLE PURPOSES ONLY)                     FIXUP

(3) DEFINE THE FOLLOWING MT NUMBERS TO BE RECONSTRUCTED,          FIXUP

(MT=  4) = THE SUM OF MT= 51 THROUGH  91                      FIXUP

(MT=103) = THE SUM OF MT=700 THROUGH 718 (NOT 719)            FIXUP

(MT=104) = THE SUM OF MT=720 THROUGH 738 (NOT 739)            FIXUP

(MT=105) = THE SUM OF MT=740 THROUGH 758 (NOT 759)            FIXUP

(MT=106) = THE SUM OF MT=760 THROUGH 778 (NOT 779)            FIXUP

(MT=107) = THE SUM OF MT=780 THROUGH 798 (NOT 799)            FIXUP

NEW     (MT= 16) = THE SUM OF MT=875 THROUGH 891                      FIXUP

(MT=101) = THE SUM OF MT=102 THROUGH 114                      FIXUP

(MT= 18) = (MT=19) + (MT=20 AND 21) + (MT=38)                 FIXUP

(IF TOTAL FISSION, MT=18, IS NOT PRESENT, DEFINE   FIXUP

IT BY SUMMING FIRST, SECOND, ETC. CHANCE - NOTE    FIXUP

THAT THIS MUST BE DONE IN THIS ORDER, SINCE THE    FIXUP

NEXT SUM INVOLVES USING MT=18.                     FIXUP

(MT= 27) = THE SUM OF MT= 18 AND 101                          FIXUP

(MT=101 RECONSTRUCTED ABOVE USED IN SUM).          FIXUP

(MT=  3) = THE SUM OF (MT=4)+(MT=6-9)+(MT=16-17)+(MT=22-37)+  FIXUP

(MT=41-45)                                         FIXUP

(MT=4 AND 27 RECONSTRUCTED ABOVE USED IN SUM).     FIXUP

(MT= 19) = (MT=18) - (MT=20 AND 21) - (MT=38)                 FIXUP

(DEFINE FIRST CHANGE FISSION BY SUBTRACTION TO     FIXUP

ALLOW RESONANCE CONTRIBUTION FROM MT=18 TO BE      FIXUP

INCLUDED IN MT=19).                                FIXUP

(MT=  1) = THE SUM OF MT=2 AND 3                              FIXUP

(MT=3 RECONSTRUCTED ABOVE USED IN SUM).            FIXUP

(4) THRESHOLD ENERGIES OF THE FOLLOWING MT NUMBERS WILL NOT BE     FIXUP

TESTED OR CORRECTED.                                          FIXUP

MT=1, 4, 18, 19, 91, 103 THROUGH 114.                         FIXUP

(5) DEFINE MT=254 TO BE THE CAPTURE TO FISSION RATIO (MT=102/18)   FIXUP

(6) CREATE MAT=1300/MT=254 - NOTE, THIS IS NECESSARY IN ORDER TO   FIXUP

HAVE THE CAPTURE TO FISSION RATIO OUTPUT IN THE ENDF FORMAT    FIXUP

FIXUP

NOTE, ON THE FOLLOWING INPUT LINES THE CHARACTERS = ( ) + , HAVE  FIXUP

BEEN USED ONLY TO MAKE THE INPUT MORE READABLE - THESE CHARACTERS FIXUP

WILL BE SKIPPED BY THE PROGRAM IN READING INPUT - THE RESULTS     FIXUP

WOULD BE THE SAME IF THESE CHARACTERS WERE OMITTED, AS LONG AS    FIXUP

ALL OF THE MT NUMBERS ARE DELIMITED, I.E., THERE IS AT LEAST ONE  FIXUP

NON-DIGITAL CHARACTER BETWEEN MT NUMBERS. NOTE, THAT - (MINUS     FIXUP

SIGN) IS IMPORTANT AND IS USED DURING INPUT TO DEFINE MT RANGES   FIXUP

WHICH SHOULD BE SUBTRACTED, E.,G., SEE THE DEFINITION OF MT=19.   FIXUP

FIXUP

READ FILE /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT    FIXUP

FIXUP

THE FOLLOWING 21 INPUT LINES ARE REQUIRED.                        FIXUP

FIXUP

11111111111                                                       FIXUP

/ENDFB6/K300/LEAD.IN                                              FIXUP

/ENDFB6/K300/LEAD.OUT                                             FIXUP

D900                                                              FIXUP

4=( 51, 91)                                                    FIXUP

103=(700,718)                                                    FIXUP

104=(720,738)                                                    FIXUP

105=(740,758)                                                    FIXUP

106=(760,778)                                                    FIXUP

107=(780,798)                                                    FIXUP

16=(875,891)                                                    FIXUP

101=(102,114)                                                    FIXUP

18=( 19, 19)+( 20, 21)+( 38, 38)                                FIXUP

27=( 18, 18)+(101,101)                                          FIXUP

3=(  4,  4)+(  6,  9)+( 16, 17)+( 22, 37)+( 41, 45)            FIXUP

19=( 18, 18)-( 20, 21)-( 38, 38)                                FIXUP

1=(  2,  3)                                                    FIXUP

T    (  1,  1)+(  4,  4)+( 18, 18)+( 91, 91)+(103,114)            FIXUP

R254=(102/ 18)                                                    FIXUP

(BLANK LINE TO TERMINATE SUMMATION/DELETION RULES)   FIXUP

2.00400+ 3 0.00000+ 0          0          01300254               FIXUP

0.00000+ 0 0.00000+ 0          0          0                      FIXUP

(BLANK LINE TO TERMINATE SECTION CREATION RULES)     FIXUP

FIXUP

NOTE, THE DELETION AND THRESHOLD EXCLUSION LINES MAY APPEAR IN    FIXUP

IN ANY ORDER. HOWEVER, SUMMATION AND RATIO RULES MUST APPEAR IN   FIXUP

THE ORDER IN WHICH YOU WANT THEM TO BE EXECUTED - E.G., THE       FIXUP

ABOVE INPUT WILL FIRST RECONSTRUCT MT=4, WHICH CAN THEN BE USED   FIXUP

TO CONTRIBUTE TO THE FOLLOWING SUM TO DEFINE MT=3, WHICH IN TURN  FIXUP

CAN THEN BE USED TO CONTRIBUTE TO THE FOLLOWING SUM TO DEFINE     FIXUP

MT=1. IF THE ORDER OF THE INPUT LINES IS CHANGED SUCH THAT MT=3   FIXUP

IS RECONSTRUCTED BEFORE MT=4, THE ORIGINAL MT=4 WILL BE USED IN   FIXUP

THE SUMMATION TO DEFINE MT=3. THE SAME RULES APPLY TO CALCULATING FIXUP

RATIOS, IF EITHER THE NUMERATOR OR DENOMINATOR IS TO BE DEFINED   FIXUP

BY SUMMATION, THIS SHOULD BE DONE BEFORE DEFINING THE RATIO BY    FIXUP

INPUT PARAMETERS.                                                 FIXUP

FIXUP

EXAMPLE INPUT NO. 2                                               FIXUP

===================                                               FIXUP

(1) USE OPTIONS 1-11 (ALL OPTIONS, EXCEPT INSERT ENERGY POINTS)   FIXUP

(2) USE BUILT-IN TABLES FOR SUMMATION/DELETION/THRESHOLD EXCLUSIONFIXUP

(THIS ONLY REQUIRES COLUMNS 2, 4 AND 5 TO BE SET =2 ON THE    FIXUP

FIRST INPUT LINE. THE BUILT-IN RULES EXACTLY CORRESPOND TO    FIXUP

THE INPUT ABOVE UNDER EXAMPLE NO. 1, EXCEPT THAT NO MT NUMBERSFIXUP

WILL BE DELETED.                                              FIXUP

(3) IF NOT PRESENT, CREATE MAT=1300/MT=1                          FIXUP

FIXUP

USE THE STANDARD FILE NAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE   FIXUP

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).          FIXUP

FIXUP

THE FOLLOWING 6 INPUT LINES ARE REQUIRED.                         FIXUP

FIXUP

12122111111                                                       FIXUP

FIXUP

FIXUP

2.00400+ 3 0.00000+ 0          0          01300  1               FIXUP

0.00000+ 0 0.00000+ 0          0          0                      FIXUP

(BLANK LINE TO TERMINATE SECTION CREATION RULES)     FIXUP

FIXUP

EXAMPLE INPUT NO. 3                                               FIXUP

===================                                               FIXUP

(1) USE OPTIONS 1-10 (ALL OPTIONS PRESENTLY IMPLEMENTED, EXCEPT   FIXUP

DO NOT ALLOW SECTION CREATION AND INSERT ENERGY POINTS).      FIXUP

(2) USE BUILT-IN TABLES FOR SUMMATION/DELETION/THRESHOLD EXCLUSIONFIXUP

(THIS ONLY REQUIRES COLUMNS 2, 4 AND 5 TO BE SET =2 ON THE    FIXUP

FIRST INPUT LINE. THE BUILT-IN RULES EXACTLY CORRESPOND TO    FIXUP

THE INPUT ABOVE UNDER EXAMPLE NO. 1, EXCEPT THAT NO MT NUMBERSFIXUP

WILL BE DELETED.                                              FIXUP

(3) DO NOT CREATE ANY SECTIONS.                                   FIXUP

FIXUP

READ FILE /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT    FIXUP

FIXUP

THE FOLLOWING 3 INPUT LINES ARE REQUIRED.                         FIXUP

FIXUP

1212211111                                                        FIXUP

/ENDFB6/K300/LEAD.IN                                              FIXUP

/ENDFB6/K300/LEAD.OUT                                             FIXUP

FIXUP

EXAMPLE INPUT NO. 4                                               FIXUP

===================                                               FIXUP

SAME AS EXAMPLE NO. 3, ABOVE, EXCEPT INSERT AN ENERGY POINT AT    FIXUP

THERMAL FOR ALL REACTIONS WHICH SPAN THE THERMAL ENERGY RANGE.    FIXUP

FIXUP

USE THE STANDARD FILE NAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE   FIXUP

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).          FIXUP

FIXUP

THE FOLLOWING 5 INPUT LINES ARE REQUIRED.                         FIXUP

FIXUP

121221111101                                                      FIXUP

FIXUP

FIXUP

2.53000- 2   0  0                                                FIXUP

(BLANK LINE TO TERMINATE ENERGY INSERTS)             FIXUP

FIXUP

WARNING                                                           FIXUP

=======                                                           FIXUP

ALTHOUGH THIS PROGRAM IS DESIGNED TO ALLOW REACTIONS TO BE DEFINEDFIXUP

BY ADDING OR SUBTRACTING REACTIONS THE USER SHOULD ALWAYS TRY TO  FIXUP

DEFINE REACTIONS BY SUMMING TO AVOID NEGATIVE CROSS SECTIONS. FOR FIXUP

EXAMPLE, IT IS POSSIBLE TO CALCULATE MT=3 AND DEFINE MT=1 AS THE  FIXUP

SUM OF MT=2 AND 3 (THE RECOMMENDED APPROACH AS USED IN THE ABOVE  FIXUP

INPUT). ALTERATIVELY IT IS POSSIBLE TO CALCULATE MT=1 AND DEFINE  FIXUP

MT=3 AS MT=1 MINUS MT=2 (THIS APPROACH IS NOT RECOMMENDED).       FIXUP

FIXUP

THE ONLY BUILT-IN SUMMATION RULE THAT USES SUBTRACTION IS THE     FIXUP

CALCULATION OF THE FIRST CHANGE FISSION (MT=19) AS THE TOTAL      FIXUP

FISSION (MT=18) MINUS THE SECOND, THIRD AND FOURTH CHANGE FISSION FIXUP

(MT=20, 21, 38). THIS HAS BEEN DONE TO ALLOW THE RESONANCE        FIXUP

CONTRIBUTION, CALCULATED BY MANY CODES AND INCLUDED IN MT=18,     FIXUP

TO BE CONSISTENTLY INCLUDED IN THE FIRST CHANCE FISSION.          FIXUP

FIXUP

=======================================================================FIXUP