====================================================================   GROUPIE

GROUPIE

PROGRAM GROUPIE                                                   GROUPIE

===============                                                   GROUPIE

VERSION 76-1 (NOVEMBER 1976)                                      GROUPIE

VERSION 79-1 (OCTOBER 1979) CDC-7600 AND CRAY-1 VERSION.          GROUPIE

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION                 GROUPIE

VERSION 81-1 (JANUARY 1981) EXTENSION TO 3000 GROUPS              GROUPIE

VERSION 81-2 (MARCH 1981) IMPROVED SPEED                          GROUPIE

VERSION 81-3 (AUGUST 1981) BUILT-IN 1/E WEIGHTING SPECTRUM        GROUPIE

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY       GROUPIE

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                      GROUPIE

*ELIMINATED COMPUTER DEPENDENT CODING. GROUPIE

*NEW, MORE COMPATIBLE I/O UNIT NUMBERS.GROUPIE

*NEW MULTI-BAND LIBRARY BINARY FORMAT. GROUPIE

VERSION 83-2 (OCTOBER 1983) ADDED OPTION TO ALLOW SIGMA-0 TO BE   GROUPIE

DEFINED EITHER AS MULTIPLES OF        GROUPIE

UNSHIELDED TOTAL CROSS SECTION IN EACHGROUPIE

GROUP, OR POWERS OF 10 IN ALL GROUPS. GROUPIE

VERSION 84-1 (APRIL 1984)   ADDED MORE BUILT IN MULTIGROUP ENERGY GROUPIE

STRUCTURES.                           GROUPIE

VERSION 85-1 (APRIL 1985)  *UPDATED FOR ENDF/B-VI FORMATS.        GROUPIE

*SPECIAL I/O ROUTINES TO GUARANTEE     GROUPIE

ACCURACY OF ENERGY.                   GROUPIE

*DOUBLE PRECISION TREATMENT OF ENERGY  GROUPIE

(REQUIRED FOR NARROW RESONANCES).     GROUPIE

*MINIMUM TOTAL CROSS SECTION TREATMENT GROUPIE

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                  GROUPIE

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMAT                      GROUPIE

VERSION 86-2 (JUNE 1986)   *BUILT-IN MAXWELLIAN, 1/E AND FISSION  GROUPIE

WEIGHTING SPECTRUM.                   GROUPIE

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O    GROUPIE

FILE NAMES (SEE, SUBROUTINES FILIO1   GROUPIE

FILIO2 FOR DETAILS).                  GROUPIE

*IMPROVED BASED ON USER COMMENTS.      GROUPIE

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO    GROUPIE

INSURE PROGRAM WILL NOT DO ANYTHING   GROUPIE

CRAZY.                                GROUPIE

*UPDATED TO USE NEW PROGRAM CONVERT    GROUPIE

KEYWORDS.                             GROUPIE

*ADDED LIVERMORE CIVIC COMPILER        GROUPIE

CONVENTIONS.                          GROUPIE

VERSION 91-1 (JUNE 1991)   *INCREASED PAGE SIZE FROM 1002 TO 5010 GROUPIE

POINTS                                GROUPIE

*UPDATED BASED ON USER COMMENTS        GROUPIE

*ADDED FORTRAN SAVE OPTION             GROUPIE

*COMPLETELY CONSISTENT ROUTINE TO READ GROUPIE

FLOATING POINT NUMBERS.               GROUPIE

VERSION 92-1 (JANUARY 1992)*ADDED RESONANCE INTEGRAL CALCULATION -GROUPIE

UNSHIELDED AND/OR SHIELDED - FOR      GROUPIE

DETAILS SEE BELOW                     GROUPIE

*INCREASED NUMBER OF ENERGY POINTS     GROUPIE

IN BUILT-IN SPECTRA - TO IMPROVE      GROUPIE

ACCURACY.                             GROUPIE

*ALLOW SELECTION OF ZA/MF/MT OR        GROUPIE

MAT/MF/MT RANGES - ALL DATA NOT       GROUPIE

SELECTED IS SKIPPED ON INPUT AND      GROUPIE

NOT WRITTEN AS OUTPUT.                GROUPIE

*COMPLETELY CONSISTENT I/O ROUTINES -  GROUPIE

TO MINIMIZE COMPUTER DEPENDENCE.      GROUPIE

*NOTE, CHANGES IN INPUT PARAMETER      GROUPIE

FORMAT - FOR ZA/MF/MT OR MAT/MF/MT    GROUPIE

RANGES.                               GROUPIE

VERSION 92-2 (JUNE 1992)   *MULTIBAND PARAMETERS OUTOUT AS        GROUPIE

CHARACTER (RATHER THAN BINARY) FILE.  GROUPIE

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 5010 TO      GROUPIE

30000 POINTS                          GROUPIE

*ELIMINATED COMPUTER DEPENDENCE.       GROUPIE

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES        GROUPIE

TO ALLOW ACCESS TO FILE STRUCTURES    GROUPIE

(WARNING - INPUT PARAMETER FORMAT     GROUPIE

HAS BEEN CHANGED)                     GROUPIE

*CLOSE ALL FILES BEFORE TERMINATING    GROUPIE

(SEE, SUBROUTINE ENDIT)               GROUPIE

VERSION 95-1 (JANUARY 1994)*CORRECTED MAXWELLIAN WEIGHTING        GROUPIE

*CHANGING WEIGHTING SPECTRUM FROM      GROUPIE

0.1 TO 0.001 % UNCERTAINTY            GROUPIE

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    GROUPIE

*IMPROVED COMPUTER INDEPENDENCE       GROUPIE

*ALL DOUBLE PRECISION                 GROUPIE

*ON SCREEN OUTPUT                     GROUPIE

*UNIFORM TREATMENT OF ENDF/B I/O      GROUPIE

*IMPROVED OUTPUT PRECISION            GROUPIE

*DEFINED SCRATCH FILE NAMES           GROUPIE

*UP TO 1000 GROUP MULTI-BAND          GROUPIE

CALCULATION (PREVIOUSLY 175)         GROUPIE

*MAXIMUM NUMBER OF GROUPS REDUCED     GROUPIE

FROM 3,000 TO 1,000                  GROUPIE

*UP TO 1000 MATERIALS                 GROUPIE

(PREVIOUSLY 100)                     GROUPIE

*CORRECTED USE OF MAXWELLIAN +        GROUPIE

1/E + FISSION SPECTRUM               GROUPIE

*ONLY 2 BAND VERSION DISTRIBUTED      GROUPIE

(CONTACT AUTHOR FOR DETAILS)         GROUPIE

*DEFINED SCRATCH FILE NAMES           GROUPIE

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      GROUPIE

POINT READ FOR MORE DIGITS           GROUPIE

*UPDATED TEST FOR ENDF/B FORMAT       GROUPIE

VERSION BASED ON RECENT FORMAT CHANGEGROUPIE

*GENERAL IMPROVEMENTS BASED ON        GROUPIE

USER FEEDBACK                        GROUPIE

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING  GROUPIE

MF=1, MT-451.                        GROUPIE

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF=10, ACTIVATION CROSS SECTIONGROUPIE

PROCESSING.                          GROUPIE

*GENERAL IMPROVEMENTS BASED ON        GROUPIE

USER FEEDBACK                        GROUPIE

VERS. 2002-1 (FEBRUARY 2002)*ADDED TART 700 GROUP STRUCTURE       GROUPIE

*ADDED VARIABLE SIGMA0 INPUT OPTION   GROUPIE

(MAY 2002)     *OPTIONAL INPUT PARAMETERS            GROUPIE

(NOV. 2002)    *ADDED SAND-II EXTENDED DOWN TO       GROUPIE

1.0D-5 EV.                           GROUPIE

(JUNE 2003)    *CORRECTED SAND-II 620 AND 640 GROUP  GROUPIE

ENERGY BOUNDARIES DEFINITIONS.       GROUPIE

VERS. 2004-1 (SEPT. 2004)  *INCREASED PAGE SIZE FROM 30000 TO     GROUPIE

120000 POINTS                         GROUPIE

*ADDED "OTHER" AS ADDITIONAL REACTION  GROUPIE

TO IMPROVE MULTI-BAND FITTING         GROUPIE

*ADDED ITERATION FOR "BEST" PARTIAL    GROUPIE

PARAMETERS.                           GROUPIE

*DO NOT SKIP LOW TOTAL ENERGY RANGES   GROUPIE

WHEN DEFINING AVERAGE CROSS SECTIONS -GROUPIE

THIS MAKES OUTPUT COMPATIBLE WITH     GROUPIE

ANY STANDARD AVERAGING PROCEDURE      GROUPIE

VERS. 2005-1 (JAN. 2005)   *ADDED OPTION TO CHANGE TEMPERATURE OF GROUPIE

BUILT-IN STANDARD SPECTRUM.           GROUPIE

VERS. 2007-1 (JAN. 2007)   *CHECKED AGAINST ALL ENDF/B-VII.       GROUPIE

*INCREASED PAGE SIZE FROM 120,000 TO   GROUPIE

600,000 POINTS                        GROUPIE

VERS. 2008-1 (JAN. 2008)   *72 CHARACTER FILE NAMES.              GROUPIE

*GENERAL UPDATES                       GROUPIE

VERS. 2010-1 (Apr. 2010)   *INCREASED WEIGHTING SPECTRUM TO 30,000GROUPIE

FROM 3,000 ENERGY POINTS.             GROUPIE

*ADDED OUTPUT TO PLOT/COMPARE SHIELDED GROUPIE

AND UNSHIELDED CROSS SECTIONS.        GROUPIE

VERS. 2011-1 (June 2011)   *Corrected TART 700 groups to extend upGROUPIE

to 1 GeV (1,000 MeV) - previously it  GROUPIE

was ERRONEOUSLY cutoff at 20 MeV.     GROUPIE

VERS. 2011-2 (Nov. 2011)   *Corrected TART 616 groups lowest      GROUPIE

energy from 1.0D-4 eV to 1.0D-5 eV.   GROUPIE

*Added TART 666 to 200 MeV (for TENDL).GROUPIE

*Optional high energy cross section    GROUPIE

extension above tabulated energy rangeGROUPIE

(either = 0 = standard, or constant)  GROUPIE

WARNING - ENDF/B standard convention  GROUPIE

is that the cross section = 0 where itGROUPIE

is not explicitly defined - extension GROUPIE

= 0 is standard, constant is NOT, so  GROUPIE

constant extension is NOT RECOMMENDED.GROUPIE

VERS. 2012-1 (Aug. 2012)   *Added CODENAME                        GROUPIE

*32 and 64 bit Compatible              GROUPIE

*Added ERROR stop.                     GROUPIE

VERS. 2013-1 (Nov. 2013)   *Extended OUT9.                        GROUPIE

*Uses OUTG, not OUT10 for energies.    GROUPIE

VERS. 2015-1 (Jan. 2015)   *Corrected SPECTM - handle ALL includedGROUPIE

group structures, i.e., even those    GROUPIE

that start above thremal range by     GROUPIE

ALWAYS constructing weigthing spectrumGROUPIE

to be AT LEAST 1.0D-5 eV to 20 MeV.   GROUPIE

*Extended OUTG                         GROUPIE

*Replaced ALL 3 way IF Statements.     GROUPIE

*Generalized TART Group Strructures.   GROUPIE

*Generalized SAND-II Group Structures. GROUPIE

*Extended SAND-II to 60, 150, 200 MeV. GROUPIE

VERS. 2015-2 (Mar. 2015)   *Deleted 1P from formats reading input GROUPIE

parameters, causing incorrect scaling GROUPIE

*Changed ALL data to "D" instead of    GROUPIE

"E" to insure it is REAL*8 and avoid  GROUPIE

Truncation ERRORS.                    GROUPIE

VERS. 2015-3 (July 2015)   *Insure no 10 digit output - not       GROUPIE

needed for multi-group and this makes GROUPIE

listings simpler.                     GROUPIE

*Corrected High Energy Extension =     GROUPIE

Can effect highest energy group.      GROUPIE

VERS. 2016-1 (July 2016)   *Added UKAEA 1102 Group Structure.     GROUPIE

*Increased storage to accommodate      GROUPIE

much larger group structures =        GROUPIE

up to 20,000 Groups.                  GROUPIE

*Added output listing of the complete  GROUPIE

input parameters for URRFIT, includingGROUPIE

the NJOY parameters LSSF and ICOMP.   GROUPIE

*Changed multiple IF statements to     GROUPIE

accommodate compiler optimizer        GROUPIE

*Cosmetic changes based on FREUD       GROUPIE

psychoanalysis.                       GROUPIE

*Updated multi-band treatment to       GROUPIE

explcitly handle small shielding      GROUPIE

limit - without this update the small GROUPIE

limit becomes numerically unstable.   GROUPIE

VERS. 2017-1 (May  2017)   *Increased max. points to 3,000,000.   GROUPIE

*METHODB was incorrecctly named        GROUPIE

METHOD  in one routine = corrected.   GROUPIE

*Default multi-band is method #2 =     GROUPIE

conserve <x>, <1/(x+<x>>, <1/x>.      GROUPIE

*Definition of built-in group structureGROUPIE

using SUBROUTINE GROPE is identical   GROUPIE

for GROUPIE and VIRGIN.               GROUPIE

*All floating input parameters changed GROUPIE

to character input + IN9 conversion.  GROUPIE

*Output report identfies MF now that   GROUPIE

this code does more than just MF=3.   GROUPIE

*Added NRO = energy dependent scatter  GROUPIE

radius to copying FILE2 parameters    GROUPIE

to define unresolved energy range.    GROUPIE

*Corrected energy dependent scatter    GROUPIE

for all resonance types (see, above   GROUPIE

comments) = for multi-band output     GROUPIE

VERS. 2018-1 (Jan. 2018)   *Added on-line output for ALL ENDERROR GROUPIE

VERS. 2019-1 (June 2019)   *Major re-write to re-order output to  GROUPIE

include Unresolved Resonance Region   GROUPIE

self-shielding.                       GROUPIE

*Added Unresolved self-shielding by    GROUPIE

Extrapolating cross section moments   GROUPIE

from Resolved (supersedes URRDO and   GROUPIE

URRFIT codes).                        GROUPIE

*Added entire self-shielding array to  GROUPIE

memory - previously only one group    GROUPIE

results were in memory - saving ALL   GROUPIE

greatly simplifies the logic.         GROUPIE

*Additional Interpolation Law Tests    GROUPIE

*Check maximum Tabulated Energy of MTs GROUPIE

to insure they ALL end at the same    GROUPIE

energy.                               GROUPIE

*Multi-band = 1 no longer allowed.     GROUPIE

The only allowed values are,          GROUPIE

0 = no multi-band calculations, or,   GROUPIE

2 = Conserve 1/[total + <total>]      GROUPIE

*Unresolved Resonance Region           GROUPIE

Self-Shielding Requires all of these, GROUPIE

1) Unresolved data with ENDF input    GROUPIE

2) 616 TART Groups (input -11)        GROUPIE

3) Define Sigma0 standard (input = 0) GROUPIE

*Unresolved Resonance Region           GROUPIE

Self-Shielding Always Outputs,        GROUPIE

1) LSSF   = 0 = Output cross sections GROUPIE

2) INTUNR = 2 = Interpolation law     GROUPIE

*Added ZAzzzaaa to filenames.          GROUPIE

VERS. 2020-1 (Aug. 2020)   *Major re-write to update for new URR  GROUPIE

self-shielding, MF/MT=2/152 and 2/153.GROUPIE

*Corrected BOTH ends of unresolved     GROUPIE

for MF/MT=2/152 and 2/153 output.     GROUPIE

*Unresolved extrapolation ONLY to      GROUPIE

groups completely inside the URR +    GROUPIE

per ends for MF/MT=2/152 & 153 output.GROUPIE

*Small shielding < 0.1 % = accuracy    GROUPIE

of reconstructed data.                GROUPIE

*Forced no self-shielding at upper end GROUPIE

of unresolved = match high energy     GROUPIE

tabulated.                            GROUPIE

*Corrected PLOTTAB output if no URR    GROUPIE

fit - it was outputting EMPTY tables  GROUPIE

for original and fit moments, which   GROUPIE

in this case did not exist.           GROUPIE

*Only 2 band, Method#2 [sigt + <sigt>] GROUPIE

alloed for multi-band calculation.    GROUPIE

*WARNING - if input Requested MF range GROUPIE

prevents unresolved region calculationGROUPIE

*Added Target Isomer Flag              GROUPIE

*Correct MULTBAND.LST output format.   GROUPIE

VERS. 2021-1 (Jan. 2021)   *Updated for FORTRAN 2018              GROUPIE

GROUPIE

2020-1 Acknowledgment                                             GROUPIE

=====================                                             GROUPIE

I Thank Jean-Christophe Sublet (NDS, IAEA, Vienna, Austria) for   GROUPIE

reporting the ERROR in GROUPIE (2019-1) that led to the update in GROUPIE

GROUPIE (2020-1) to correctly define the PLOTTAB output, whether  GROUPIE

or not Unresolved Resonance Region (URR) fit was performed.       GROUPIE

GROUPIE

2015-2 Acknowledgment                                             GROUPIE

=====================                                             GROUPIE

I thank Chuck Whitmer (TerraPower,WA) and Andrej Trkov (NDS,IAEA) GROUPIE

for reporting the errors that led to the 2015-2 Improvements in   GROUPIE

this code.                                                        GROUPIE

GROUPIE

I thank Jean-Christophe Sublet (UKAEA) for contributing MAC       GROUPIE

executables and Bojan Zefran (IJS, Slovenia) for contributing     GROUPIE

LINUX (32 or 63 bit) executables. And most of all I must thank    GROUPIE

Andrej Trkov (NDS, IAEA) for overseeing the entire PREPRO project GROUPIE

at IAEA, Vienna. This was a truly International team who worked   GROUPIE

together to produce PREPRO 2015-2.                                GROUPIE

GROUPIE

OWNED, MAINTAINED AND DISTRIBUTED BY                              GROUPIE

------------------------------------                              GROUPIE

THE NUCLEAR DATA SECTION                                          GROUPIE

INTERNATIONAL ATOMIC ENERGY AGENCY                                GROUPIE

P.O. BOX 100                                                      GROUPIE

A-1400, VIENNA, AUSTRIA                                           GROUPIE

EUROPE                                                            GROUPIE

GROUPIE

ORIGINALLY WRITTEN BY                                             GROUPIE

------------------------------------                              GROUPIE

Dermott E. Cullen                                                 GROUPIE

GROUPIE

PRESENT CONTACT INFORMATION                                       GROUPIE

---------------------------                                       GROUPIE

Dermott E. Cullen                                                 GROUPIE

1466 Hudson Way                                                   GROUPIE

Livermore, CA 94550                                               GROUPIE

U.S.A.                                                            GROUPIE

Telephone  925-443-1911                                           GROUPIE

E. Mail    RedCullen1@Comcast.net                                 GROUPIE

Website    RedCullen1.net/HOMEPAGE.NEW                            GROUPIE

GROUPIE

AUTHORS MESSAGE                                                   GROUPIE

---------------                                                   GROUPIE

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION  GROUPIE

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDEREDGROUPIE

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASEGROUPIE

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY    GROUPIE

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                 GROUPIE

GROUPIE

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTERGROUPIE

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE    GROUPIE

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECTGROUPIE

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY      GROUPIE

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO GROUPIE

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF   GROUPIE

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR        GROUPIE

COMPUTER.                                                         GROUPIE

GROUPIE

PURPOSE                                                           GROUPIE

-------                                                           GROUPIE

THIS PROGRAM IS DESIGNED TO CALCULATE ANY COMBINATION OF          GROUPIE

THE FOLLOWING QUANTITIES FROM LINEARLY INTERPOLABLE TABULATED     GROUPIE

CROSS SECTIONS IN THE ENDF/B FORMAT                               GROUPIE

GROUPIE

(1) UNSHIELDED GROUP AVERAGED CROSS SECTIONS                      GROUPIE

(2) BONDARENKO SELF-SHIELDED GROUP AVERAGED CROSS SECTIONS        GROUPIE

(3) MULTI-BAND PARAMETERS                                         GROUPIE

GROUPIE

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B    GROUPIE

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS, GROUPIE

DISK OR ANY OTHER MEDIUM.                                         GROUPIE

GROUPIE

ENDF/B FORMAT                                                     GROUPIE

-------------                                                     GROUPIE

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS    GROUPIE

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION  GROUPIE

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV OR V FORMAT).    GROUPIE

GROUPIE

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B      GROUPIE

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS    GROUPIE

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE GROUPIE

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE         GROUPIE

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451 GROUPIE

AND ALL SECTIONS OF MF= 3 MUST BE CORRECT. THE PROGRAM COPIES ALL GROUPIE

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO  GROUPIE

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.           GROUPIE

GROUPIE

ALL FILE 3 CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE   GROUPIE

LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B         GROUPIE

INTERPOLATION LAW 2). FILE 3 BACKGROUND CROSS SECTIONS MAY BE MADEGROUPIE

LINEARLY INTERPOLABLE USING PROGRAM LINEAR (UCRL-50400, VOL. 17,  GROUPIE

PART A). THE RESONANCE CONTRIBUTION MAY BE ADDED TO THE BACKGROUNDGROUPIE

CROSS SECTIONS USING PROGRAM RECENT (UCRL-50400, VOL. 17, PART B).GROUPIE

IF THIS PROGRAM FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT      GROUPIE

LINEARLY INTERPOLABLE THIS PROGRAM WILL TERMINATE EXECUTION.      GROUPIE

GROUPIE

CONTENTS OF OUTPUT                                                GROUPIE

------------------                                                GROUPIE

IF ENDF/B FORMATTED OUTPUT IS REQUESTED ENTIRE EVALUATIONS ARE    GROUPIE

OUTPUT, NOT JUST THE MULTI-GROUPED FILE 3 CROSS SECTIONS, E.G.    GROUPIE

ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO INCLUDED.               GROUPIE

GROUPIE

DOCUMENTATION                                                     GROUPIE

-------------                                                     GROUPIE

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED GROUPIE

BY THE ADDITION OF THREE COMMENT CARDS AT THE END OF EACH         GROUPIE

HOLLERITH SECTION TO DESCRIBE THE GROUP STRUCTURE AND WEIGHTING   GROUPIE

SPECTRUM, E.G.                                                    GROUPIE

GROUPIE

********************** PROGRAM GROUPIE (2021-1) ***************   GROUPIE

UNSHIELDED GROUP AVERAGES USING   69 GROUPS (WIMS)                GROUPIE

MAXWELLIAN, 1/E, FISSION TO CONSTANT WEIGHTING SPECTRUM           GROUPIE

GROUPIE

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR, RECENT AND SIGMA1)GROUPIE

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON      GROUPIE

THE DATA.                                                         GROUPIE

GROUPIE

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS,GROUPIE

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMATGROUPIE

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF     GROUPIE

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451   GROUPIE

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF     GROUPIE

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF     GROUPIE

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO         GROUPIE

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND  GROUPIE

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT GROUPIE

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                     GROUPIE

GROUPIE

REACTION INDEX                                                    GROUPIE

--------------                                                    GROUPIE

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN    GROUPIE

SECTION MF=1, MT=451 OF EACH EVALUATION.                          GROUPIE

GROUPIE

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.  GROUPIE

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT        GROUPIE

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WASGROUPIE

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING GROUPIE

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE GROUPIE

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAMGROUPIE

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.    GROUPIE

GROUPIE

SECTION SIZE                                                      GROUPIE

------------                                                      GROUPIE

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT GROUPIE

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS     GROUPIE

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                GROUPIE

GROUPIE

SELECTION OF DATA                                                 GROUPIE

-----------------                                                 GROUPIE

THE PROGRAM SELECTS MATERIALS TO BE PROCESSED BASED EITHER ON     GROUPIE

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR   GROUPIE

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE       GROUPIE

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS   GROUPIE

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA     GROUPIE

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                 GROUPIE

GROUPIE

ENERGY ORDER AND UNITS                                            GROUPIE

----------------------                                            GROUPIE

ALL ENERGIES (FOR CROSS SECTIONS, WEIGHTING SPECTRUM OR GROUP     GROUPIE

BOUNDARIES) MUST BE IN UNITS OF EV AND MUST BE IN ASCENDING       GROUPIE

NUMERICAL ORDER.                                                  GROUPIE

GROUPIE

ENERGY GRID                                                       GROUPIE

-----------                                                       GROUPIE

ALTHOUGH ALL REACTIONS MUST TO LINEARLY INTERPOLABLE, THEY DO NOT GROUPIE

ALL HAVE TO USE THE SAME ENERGY GRID. EACH REACTION CAN BE GIVEN  GROUPIE

BY AN INDEPENDENT ENERGY GRID. THIS PROGRAM WILL PROCEED FROM     GROUPIE

THE LOWEST TO HIGHEST ENERGY SELECTING EACH ENERGY INTERVAL OVER  GROUPIE

WHICH ALL DATA, FOR ANY GIVEN CALCULATION, ARE ALL LINEARLY       GROUPIE

INTERPOLABLE.                                                     GROUPIE

GROUPIE

GROUP STRUCTURE                                                   GROUPIE

---------------                                                   GROUPIE

THIS PROGRAM IS DESIGNED TO USE AN ARBITRARY ENERGY GROUP         GROUPIE

STRUCTURE WHERE THE ENERGIES ARE IN EV AND ARE IN INCREASING      GROUPIE

ENERGY ORDER. THE MAXIMUM NUMBER OF GROUPS IS 20,000.             GROUPIE

GROUPIE

THE USER MAY INPUT AN ARBITRARY GROUP STRUCTURE OR THE USER MAY   GROUPIE

USE USE ONE OF THE BUILT-IN GROUP STRUCTURES.                     GROUPIE

(0) 175 GROUP (TART STRUCTURE)                                    GROUPIE

(1)  50 GROUP (ORNL STRUCTURE)                                    GROUPIE

(2) 126 GROUP (ORNL STRUCTURE)                                    GROUPIE

(3) 171 GROUP (ORNL STRUCTURE)                                    GROUPIE

(4) 620 GROUP (SAND-II STRUCTURE, UP TO 18 MEV)                   GROUPIE

(5) 640 GROUP (SAND-II STRUCTURE, UP TO 20 MEV)                   GROUPIE

(6)  69 GROUP (WIMS STRUCTURE)                                    GROUPIE

(7)  68 GROUP (GAM-I STRUCTURE)                                   GROUPIE

(8)  99 GROUP (GAM-II STRUCTURE)                                  GROUPIE

(9)  54 GROUP (MUFT STRUCTURE)                                    GROUPIE

(10)  28 GROUP (ABBN STRUCTURE)                                    GROUPIE

(11) 616 GROUP (TART STRUCTURE TO 20 MeV)                          GROUPIE

(12) 700 GROUP (TART STRUCTURE TO 1 GEV)                           GROUPIE

(13) 665 GROUP (SAND-II STRUCTURE, 1.0D-5 eV, UP TO 18 MEV)        GROUPIE

(14) 685 GROUP (SAND-II STRUCTURE, 1.0D-5 eV, UP TO 20 MEV)        GROUPIE

(15) 666 GROUP (TART STRUCTURE TO 200 MeV)                         GROUPIE

(16) 725 GROUP (SAND-II STRUCTURE, 1.0D-5 eV, UP TO  60 MEV)       GROUPIE

(17) 755 GROUP (SAND-II STRUCTURE, 1.0D-5 eV, UP TO 150 MEV)       GROUPIE

(18) 765 GROUP (SAND-II STRUCTURE, 1.0D-5 eV, UP TO 200 MEV)       GROUPIE

(19)1102 GROUP (UKAEA   STRUCTURE, 1.0D-5 eV, UP TO   1 GeV)       GROUPIE

GROUPIE

GROUP AVERAGES                                                    GROUPIE

--------------                                                    GROUPIE

THIS PROGRAM DEFINES GROUP AVERAGED CROSS SECTIONS AS...          GROUPIE

GROUPIE

(INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)            GROUPIE

AVERAGE = -----------------------------------------               GROUPIE

(INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)                     GROUPIE

WHERE...                                                          GROUPIE

GROUPIE

AVERAGE  = GROUP AVERAGED CROSS SECTION                           GROUPIE

E1, E2   = ENERGY LIMITS OF THE GROUP                             GROUPIE

SIGMA(E) = ENERGY DEPENDENT CROSS SECTION FOR ANY GIVEN REACTION  GROUPIE

S(E)     = ENERGY DEPENDENT WEIGHTING SPECTRUM                    GROUPIE

WT(E)    = ENERGY DEPENDENT SELF-SHIELDING FACTOR.                GROUPIE

GROUPIE

ENERGY DEPENDENT WEIGHTING SPECTRUM                               GROUPIE

-----------------------------------                               GROUPIE

THE ENERGY DEPENDENT WEIGHTING SPECTRUM IS GIVEN BY AN ARBITRARY  GROUPIE

TABULATED LINERLY INTERPOLABLE FUNCTION WHICH CAN BE DESCRIBED    GROUPIE

BY AN ARBITRARY NUMBER OF POINTS. THIS ALLOWS THE USER TO         GROUPIE

SPECIFY ANY DESIRED WEIGHTING SPECTRUM TO ANY GIVEN DEGREE OF     GROUPIE

ACCURACY. REMEMBER THAT THE PROGRAM WILL ASSUME THAT THE SPECTRUM GROUPIE

IS LINEARLY INTERPOLABLE BETWEEN TABULATED POINTS. THEREFORE THE  GROUPIE

USER SHOULD USE ENOUGH POINTS TO INSURE AN ADEQUATE REPRESENTATIONGROUPIE

OF THE SPECTRUM BETWEEN TABULATED DATA POINTS.                    GROUPIE

GROUPIE

THE PRESENT VERSION OF THE CODE HAS THREE BUILT-IN WEIGHTING      GROUPIE

SPECTRA,                                                          GROUPIE

GROUPIE

(1) CONSTANT                                                      GROUPIE

(2) 1/E                                                           GROUPIE

(3) MAXWELLIAN = E*EXP(-E/KT)/KT                (0.0 TO 4*KT)     GROUPIE

1/E        = C1/E                           (4*KT TO 67 KeV)  GROUPIE

FISSION    = C2*EXP(-E/WA)*SINH(SQRT(E*WB)) (67 KeV, 10 MeV)  GROUPIE

CONSTANT   = Equal to Fission at 10 MeV     (above 10 MeV)    GROUPIE

GROUPIE

KT     = 0.253 EV (293 KELVIN)                                GROUPIE

WA     = 9.65D+5                                              GROUPIE

WB     = 2.29D-6                                              GROUPIE

C1, C2 = DEFINED TO MAKE SPECTRUM CONTINUOUS                  GROUPIE

GROUPIE

FISSION SPECTRUM CONSTANTS FROM                               GROUPIE

A.F.HENRY, NUCLEAR REACTOR ANALYSIS, P. 11, MIT PRESS (1975)  GROUPIE

GROUPIE

UNSHIELDED GROUP AVERAGES                                         GROUPIE

-------------------------                                         GROUPIE

FOR UNSHIELDED AVERAGES THE SELF-SHIELDING FACTOR (WT(E)) IS SET  GROUPIE

TO UNITY. THIS PROGRAM ALLOWS UP TO 20,000 GROUPS.                GROUPIE

GROUPIE

SELF-SHIELDED GROUP AVERAGES                                      GROUPIE

----------------------------                                      GROUPIE

IF SELF-SHIELDED AVERAGES AND/OR MULTI-BAND PARAMETERS ARE        GROUPIE

CALCULATED THIS PROGRAM ALLOWS UP TO 20,000 GROUPS. SELF-SHIELDED GROUPIE

AVERAGES AND/OR MULTI-BAND PARAMETERS ARE CALCULATED FOR THE      GROUPIE

TOTAL, ELASTIC, CAPTURE AND FISSION.                              GROUPIE

GROUPIE

FOR THE TOTAL, ELASTIC, CAPTURE AND FISSION THE PROGRAM USES A    GROUPIE

WEIGHTING FUNCTION THAT IS A PRODUCT OF THE ENERGY DEPENDENT      GROUPIE

WEIGHTING SPECTRUM TIMES A BONDERENKO TYPE SELF-SHIELDING FACTOR. GROUPIE

GROUPIE

WT(E) = S(E)/(TOTAL(E)+SIGMA0)**N                                 GROUPIE

GROUPIE

WHERE...                                                          GROUPIE

GROUPIE

S(E)     - ENERGY DEPENDENT WEIGHTING SPECTRUM (DEFINED BY        GROUPIE

TABULATED VALUES AND LINEAR INTERPOLATION BETWEEN      GROUPIE

TABULATED VALUES).                                     GROUPIE

TOTAL(E) - ENERGY DEPENDENT TOTAL CROSS SECTION FOR ONE MATERIAL  GROUPIE

(DEFINED BY TABULATED VALUES AND LINEAR INTERPOLATION  GROUPIE

BETWEEN TABULATED VALUES).                             GROUPIE

SIGMA0   - CROSS SECTION TO REPRESENT THE EFFECT OF ALL OTHER     GROUPIE

MATERIALS AND LEAKAGE (DEFINED WITHIN EACH GROUP TO BE GROUPIE

A MULTIPLE OF THE UNSHIELDED TOTAL CROSS SECTION WITHINGROUPIE

THAT GROUP OR POWERS OF 10 - INPUT OPTION).            GROUPIE

N        - A POSITIVE INTEGER (0, 1, 2 OR 3).                     GROUPIE

GROUPIE

THE PROGRAM WILL USE ONE ENERGY DEPENDENT WEIGHTING SPECTRUM S(E) GROUPIE

AND 25 DIFFERENT BONDERENKO TYPE SELF-SHIELDING FACTORS (25 SIGMA0GROUPIE

AND N COMBINATIONS) TO DEFINE 25 DIFFERENT AVERAGE CROSS SECTIONS,GROUPIE

FOR EACH REACTION, WITHIN EACH GROUP.                             GROUPIE

GROUPIE

THE 25 WEIGHTING FUNCTIONS USED ARE....                           GROUPIE

(1)   - UNSHIELDED CROSS SECTIONS (N=0)                           GROUPIE

(2-22)- PARTIALLY SHIELDED CROSS SECTIONS (N=1 ,VARIOUS SIGMA0)   GROUPIE

THE VALUES OF SIGMA0 USED WILL BE EITHER,                 GROUPIE

(A) THE VALUES OF SIGMA0 THAT ARE USED VARY FROM 1024     GROUPIE

TIMES THE UNSHIELDED TOTAL CROSS SECTIONS IN STEPS OF 1/2 GROUPIE

DOWN TO 1/1024 TIMES THE UNSHIELDED TOTAL CROSS SECTION   GROUPIE

(A RANGE OF OVER 1 MILLION, CENTERED ON THE UNSHIELDED    GROUPIE

TOTAL CROSS SECTION WITHIN EACH GROUP).                   GROUPIE

(B) THE SAME CONSTANT VALUES OF SIGMA0 IN EACH GROUP. THE GROUPIE

VALUES OF SIGMA0 USED INCLUDE 40000, 20000, 10000, 7000,  GROUPIE

4000, 2000, 1000, 700, 400, 200, 100, 70, 40, 20, 10, 7,  GROUPIE

4, 2, 1, 0.7, 0.4 (A RANGE OF 100,000 SPANNING MORE THAN  GROUPIE

THE RANGE OF SIGMA0 VALUES THAT MAY BE ENCOUNTERED IN     GROUPIE

ACTUAL APPLICATIONS)                                      GROUPIE

(23)  - TOTALLY SHIELDED FLUX WEIGHTED CROSS SECTION              GROUPIE

(N=1, SIGMA0=0)                                           GROUPIE

(24)  - TOTALLY SHIELDED CURRENT WEIGHTED CROSS SECTION           GROUPIE

(N=2, SIGMA0=0)                                           GROUPIE

(25)  - TOTALLY SHIELDED COSINE SQUARED WEIGHTED CROSS SECTION    GROUPIE

(N=3, SIGMA0=0)                                           GROUPIE

GROUPIE

FOR ALL OTHER REACTIONS (EXCEPT TOTAL, ELASTIC, CAPTURE AND       GROUPIE

FISSION) THE PROGRAM WILL USE THE ENERGY DEPENDENT WEIGHTING      GROUPIE

SPECTRUM S(E) TO DEFINE THE UNSHIELDED (BONDERENKO N=0)           GROUPIE

AVERAGED CROSS SECTION WITHIN EACH GROUP.                         GROUPIE

GROUPIE

CALCULATION OF RESONANCE INTEGRALS                                GROUPIE

----------------------------------                                GROUPIE

IN A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH A        GROUPIE

CONSTANT CROSS SECTION THE SPECTRUM WILL BE 1/E AND THERE WILL    GROUPIE

BE NO SELF-SHIELDING.                                             GROUPIE

GROUPIE

IN THIS CASE IF THE CROSS SECTION VARIES WITH ENERGY THE          GROUPIE

SPECTRUM WILL STILL BE 1/E AND THE SELF-SHIELDING FACTOR WILL     GROUPIE

BE EXACTLY 1/SIG-TOT(E) - WHERE SIG-TOT(E) = SIG-EL(E), SINCE     GROUPIE

THERE IS ONLY SCATTERING.                                         GROUPIE

GROUPIE

IF WE HAVE AN INFINITELY DILUTE AMOUNT OF A MATERIAL UNIFORMLY    GROUPIE

MIXED WITH A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH  GROUPIE

A CONSTANT CROSS SECTION THE STANDARD DEFINITION OF THE RESONANCE GROUPIE

INTEGRAL CAN BE USED TO DEFINE REACTION RATES FOR EACH REACTION.  GROUPIE

GROUPIE

THE RESONANCE INTEGRAL IS DEFINED AS,                             GROUPIE

GROUPIE

RI      = (INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)            GROUPIE

GROUPIE

WHERE NORMALLY,                                                   GROUPIE

S(E)    = 1/E                                                     GROUPIE

WT(E)   = 1    - NO SELF-SHIELDING                                GROUPIE

GROUPIE

FROM THE ABOVE DEFINITION OF GROUP AVERAGED CROSS SECTIONS THE    GROUPIE

RESONANCE INTEGRAL IS,                                            GROUPIE

GROUPIE

RI      = AVERAGE * (INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)           GROUPIE

GROUPIE

FOR A 1/E SPECTRUM AND NO SELF-SHIELDING THIS REDUCES TO,         GROUPIE

GROUPIE

RI      = AVERAGE* LOG(E2/E1)                                     GROUPIE

GROUPIE

IN ANY OTHER SITUATION, INCLUDING ABSORPTION AND/OR ENERGY        GROUPIE

DEPENDENT CROSS SECTIONS, THE SPECTRUM WILL NOT BE 1/E -          GROUPIE

ABSORPTION WILL TEND TO DECREASE THE SPECTRUM PROGRESSIVELY       GROUPIE

MORE AT LOWER ENERGIES - ENERGY DEPENDENCE OF THE CROSS SECTION   GROUPIE

WILL LEAD TO SELF-SHIELDING.                                      GROUPIE

GROUPIE

HERE WE WILL NOT ATTEMPT TO PERFORM A DETAILED SPECTRUM           GROUPIE

CALCULATION TO ACCOUNT FOR ABSORPTION.                            GROUPIE

GROUPIE

HOWEVER, WE WILL EXTEND THE DEFINITION OF THE RESONANCE INTEGRAL  GROUPIE

TO ACCOUNT FOR SELF-SHIELDING EFFECTS BY ALLOWING FOR INCLUSION   GROUPIE

OF SELF-SHIELDING EFFECTS IN THE DEFINITION OF GROUP AVERAGES     GROUPIE

AND THEN DEFINING THE RESONANCE INTEGRAL AS,                      GROUPIE

GROUPIE

RI      = AVERAGE* LOG(E2/E1)                                     GROUPIE

GROUPIE

IN ORDER TO CALCULATE RESONANCE INTEGRALS YOU MUST FOLLOW THESE   GROUPIE

STEPS,                                                            GROUPIE

GROUPIE

1) SELECT A 1/E SPECTRUM - ON FIRST LINE OF INPUT PARAMETERS.     GROUPIE

2) SELECT THE ENERGY BOUNDARIES - NORMALLY ONLY 1 GROUP FROM      GROUPIE

0.5 EV UP TO 20 MEV - HOWEVER, YOU ARE FREE TO SELECT ANY      GROUPIE

ENERGY RANGE THAT YOU WISH - YOU MAY EVEN SELECT MORE THAN     GROUPIE

1 GROUP MERELY BY SPECIFYING MORE THAN 1 GROUP AS INPUT -      GROUPIE

THIS CAN BE USED TO DEFINE THE CONTRIBUTIONS TO THE RESONANCE  GROUPIE

INTEGRAL FROM INDIVIDUAL ENERGY RANGES.                        GROUPIE

3) SELECT THIS OPTION FOR THE UNSHIELDED AND/OR SHIELDED OUTPUT   GROUPIE

LISTING - ON THE SECOND LINE OF INPUT PARAMETERS.              GROUPIE

GROUPIE

WHEN THIS OPTION IS USED THE PROGRAM WILL CALCULATE GROUP AVERAGEDGROUPIE

CROSS SECTIONS - AS DEFINED ABOVE - PRIOR TO OUTPUT THE RESULTS   GROUPIE

WILL MERELY BE MULTIPLIED BY THE WIDTH OF THE GROUP ASSUMING YOU  GROUPIE

HAVE SELECTED A 1/E SPECTRUM - THERE IS NO CHECK ON THIS - THE    GROUPIE

PROGRAM MERELY MULTIPLIES THE GROUP AVERAGED CROSS SECTIONS BY,   GROUPIE

GROUPIE

LOG(E2/E1) - WHERE E2 AND E1 ARE THE GROUP ENERGY BOUNDARIES.     GROUPIE

GROUPIE

WARNING - IT IS UP TO YOU TO INSURE THAT YOU FOLLOW EXACTLY THE   GROUPIE

STEPS OUTLINED ABOVE IF YOU WISH TO OBTAIN MEANINGFUL   GROUPIE

RESULTS.                                                GROUPIE

GROUPIE

NOTE - OUTPUT IN THE ENDF/B FORMAT IS ALWAYS GROUP AVERAGED CROSS GROUPIE

SECTIONS, REGARDLESS OF WHETHER YOU ASK FOR AVERAGED CROSS GROUPIE

SECTIONS OR RESONANCE INTEGRALS - THIS IS BECAUSE DATA IN  GROUPIE

THE ENDF/B FORMAT IS EXPLICITLY DEFINED TO BE CROSS        GROUPIE

SECTIONS.                                                  GROUPIE

GROUPIE

RESONANCE INTEGRAL OUTPUT CAN ONLY BE OBTAINED IN THE      GROUPIE

LISTING FORMATS.                                           GROUPIE

GROUPIE

MINIMUM TOTAL CROSS SECTION TREATMENT                             GROUPIE

-------------------------------------                             GROUPIE

SINCE THE BONDARENKO SELF-SHIELDING DEPENDS ON 1/TOTAL CROSS      GROUPIE

SECTION, THE ALGORITHM WILL BECOME NUMERICALLY UNSTABLE IF THE    GROUPIE

TOTAL CROSS SECTION IS NEGATIVE (AS OCCURS IN MANY ENDF/B         GROUPIE

EVALUATIONS). IF THE TOTAL IS LESS THAN SOME MINIMUM ALLOWABLE    GROUPIE

VALUE (DEFINE BY OKMIN, PRESENTLY 1 MILLI-BARN) AN ERROR MESSAGE  GROUPIE

WILL BE PRINTED AND FOR THE SELF-SHIELDING CALCULATION ALL ENERGY GROUPIE

INTERVALS IN WHICH THE TOTAL IS LESS THAN THE MINIMUM WILL BE     GROUPIE

IGNORED.                                                          GROUPIE

GROUPIE

NOTE, FOR THE UNSHIELDED CALCULATIONS ALL CROSS SECTIONS WILL BE  GROUPIE

CONSIDERED WHETHER THEY ARE POSITIVE OR NEGATIVE. THEREFORE IF    GROUPIE

THE TOTAL CROSS SECTION IS NEGATIVE OR LESS THAN THE MINIMUM      GROUPIE

VALUE THERE MAY BE AN INCONSISTENCY BETWEEN THE UNSHIELDED AND    GROUPIE

THE SELF-SHIELDED CROSS SECTIONS. IF THE TOTAL CROSS SECTION IS   GROUPIE

NEGATIVE AND SELF-SHIELDED CROSS SECTIONS ARE CALCULATED THE      GROUPIE

PROGRAM WILL PRINT AN ERROR MESSAGE INDICATING THAT THE SELF-     GROUPIE

SHIELDED RESULTS ARE UNRELIABLE AND SHOULD NOT BE USED. THEREFORE GROUPIE

IN THIS CASE THE PROGRAM WILL NOT ATTEMPT TO MODIFY THE UNSHIELDEDGROUPIE

RESULTS TO ELIMINATE THE EFFECT OF NEGATIVE CROSS SECTIONS, SINCE GROUPIE

THE UNSHIELDED RESULTS ARE THE ONLY ONES WHICH TRULY REFLECT THE  GROUPIE

ACTUAL INPUT.                                                     GROUPIE

GROUPIE

RESOLVED RESONANCE REGION                                         GROUPIE

-------------------------                                         GROUPIE

IN THE RESOLVED RESONANCE REGION (ACTUALLY EVERYWHERE BUT IN THE  GROUPIE

UNRESOLVED RESONANCE REGION) THE CROSS SECTIONS OUTPUT BY LINEAR- GROUPIE

RECENT-SIGMA1 WILL BE ACTUAL ENERGY DEPENDENT CROSS SECTIONS AND  GROUPIE

THE CALCULATIONS BY THIS PROGRAM WILL YIELD ACTUAL SHIELDED AND   GROUPIE

UNSHIELDED CROSS SECTIONS.                                        GROUPIE

GROUPIE

UNRESOLVED RESONANCE REGION                                       GROUPIE

---------------------------                                       GROUPIE

IN THE UNRESOLVED RESONANCE REGION PROGRAM RECENT USES THE        GROUPIE

UNRESOLVED RESONANCE PARAMETERS TO CALCULATE INFINITELY DILUTE    GROUPIE

AVERAGE CROSS SECTIONS. THIS PROGRAM WILL MERELY READ THIS        GROUPIE

INFINITELY DILUTE DATA AS IF IT WERE ENERGY DEPENDENT DATA AND    GROUPIE

GROUP AVERAGE IT. AS SUCH THIS PROGRAM WILL PRODUCE THE CORRECT   GROUPIE

UNSHIELDED CROSS SECTION IN THE UNRESOLVED RESONANCE REGION, BUT  GROUPIE

IT WILL NOT PRODUCE THE CORRECT SELF-SHIELDING EFFECTS.           GROUPIE

GROUPIE

ACCURACY OF RESULTS                                               GROUPIE

-------------------                                               GROUPIE

ALL INTEGRALS ARE PERFORMED ANALYTICALLY. THEREFORE NO ERROR IS   GROUPIE

INTRODUCED DUE TO THE USE OF TRAPAZOIDAL OR OTHER INTEGRATION     GROUPIE

SCHEME. THE TOTAL ERROR THAT CAN BE ASSIGNED TO THE RESULTING     GROUPIE

AVERAGES IS JUST THAT DUE TO THE ERROR IN THE CROSS SECTIONS      GROUPIE

AND ENERGY DEPENDENT WEIGHTING SPECTRUM. GENERALLY SINCE THE      GROUPIE

THE ENERGY DEPENDENT WEIGHTING SPECTRUM APPEARS IN BOTH THE       GROUPIE

NUMERATOR AND THE DENOMINATOR THE AVERAGES RAPIDLY BECOME         GROUPIE

INSENSITIVE TO THE WEIGHTING SPECTRUM AS MORE GROUPS ARE USED.    GROUPIE

SINCE THE WEIGHTING SPECTRUM IS LOADED IN THE PAGING SYSTEM THE   GROUPIE

USER CAN DESCRIBE THE SPECTRUM TO ANY REQUIRED ACCURACY USING     GROUPIE

ANY NUMBER OF ENERGY VS. SPECTRUM PAIRS.                          GROUPIE

GROUPIE

MULTI-BAND PARAMETERS                                             GROUPIE

---------------------                                             GROUPIE

MULTI-BAND PARAMETERS ARE CALCULATED FOR THE TOTAL, ELASTIC,      GROUPIE

CAPTURE AND FISSION REACTIONS. WITH THE NUMBER OF GROUPS THAT     GROUPIE

ARE NORMALLY USED (SEE BUILT IN GROUP STRUCTURES) ALL OTHER       GROUPIE

REACTIONS RESULT IN A NEGLIGABLE AMOUNT OF SELF-SHIELDING. AS     GROUPIE

SUCH THEIR EQUIVALENT BAND CROSS SECTION WILL MERELY BE THEIR     GROUPIE

UNSHIELDED VALUE WITHIN EACH BAND.                                GROUPIE

GROUPIE

FOR ANY GIVEN EVALUATION, WITHIN ANY GIVEN GROUP THIS PROGRAM     GROUPIE

WILL GENERATE THE MINIMUM NUMBER OF BANDS REQUIRED WITHIN THAT    GROUPIE

GROUP. AS OUTPUT TO THE COMPUTER READABLE DISK FILE THE BAND      GROUPIE

PARAMETERS FOR EACH EVALUATION WILL BE FORMATTED TO HAVE THE      GROUPIE

SAME NUMBER OF BANDS IN ALL GROUPS (WITH ZERO WEIGHT FOR SOME     GROUPIE

BANDS WITHIN ANY GROUP). THE USER MAY DECIDE TO HAVE OUTPUT       GROUPIE

EITHER WITH THE MINIMUM NUMBER OF BANDS REQUIRED FOR EACH         GROUPIE

EVALUATION (E.G. 2 BANDS FOR HYDROGEN AND 4 BANDS FOR U-233) OR   GROUPIE

THE SAME NUMBER OF BANDS FOR ALL EVALUATIONS (E.G. 4 BANDS FOR    GROUPIE

BOTH HYDROGEN AND U-233).                                         GROUPIE

GROUPIE

FOR 2 OR FEWER BANDS THE PROGRAM USES AN ANALYTIC EXPRESSION      GROUPIE

TO DEFINE ALL MULTI-BAND PARAMETERS. FOR MORE THAN 2 BANDS THE    GROUPIE

PROGRAM PERFORMS A NON-LINEAR FIT TO SELECT THE MULTI-BAND        GROUPIE

PARAMETERS THAT MINIMIZE THE MAXIMUM FRACTIONAL ERROR AT ANY      GROUPIE

POINT ALONG THE ENTIRE SELF-SHIELDING CURVE. THE NUMBER OF BANDS  GROUPIE

REQUIRED WITHIN ANY GIVEN GROUP IS DEFINED BY INSURING THAT THE   GROUPIE

MULTI-BAND PARAMETERS CAN BE USED TO ACCURATELY DEFINE SELF-      GROUPIE

SHIELDED CROSS SECTIONS ALONG THE ENTIRE SELF-SHIELDING CURVE     GROUPIE

FROM SIGMA0 = 0 TO INFINITY. THE USER MAY DEFINE THE ACCURACY     GROUPIE

REQUIRED.                                                         GROUPIE

GROUPIE

ENDF/B FORMATTED UNSHIELDED AVERAGES                              GROUPIE

------------------------------------                              GROUPIE

UNSHIELDED MULTI-GROUP AVERAGED CROSS SECTIONS FOR ALL REACTIONS  GROUPIE

MAY BE OBTAINED IN THE ENDF/B FORTRAN IN EITHER HISTOGRAM         GROUPIE

(INTERPOLATION LAW 1) OR LINEARLY INTERPOLABLE (INTERPOLATION     GROUPIE

LAW 2) FORM. SEE INPUT BELOW FOR DETAILS.                         GROUPIE

GROUPIE

MIXTURES OF MATERIALS AND RESONANCE OVERLAP                       GROUPIE

-------------------------------------------                       GROUPIE

THE SELF-SHIELDED CROSS SECTIONS FOR THE INDIVIDUAL CONSTITUENTS  GROUPIE

OF ANY MIXTURE CAN BE CALCULATED BY THIS PROGRAM BY REALIZING THATGROUPIE

THIS PROGRAM ESSENTIALLY ONLY USES THE TOTAL CROSS SECTION AS A   GROUPIE

WEIGHTING FUNCTION TO ACCOUNT FOR SELF-SHIELDING EFFECTS. FOR A   GROUPIE

MIXTURE IT IS THEREFORE ONLY NECESSARY TO USE THE TOTAL CROSS     GROUPIE

SECTION FOR THE MIXTURE IN PLACE OF THE ACTUAL TOTAL CROSS SECTIONGROUPIE

FOR EACH CONSTITUENT AND TO RUN THIS PROGRAM. THIS CAN BE DONE BY GROUPIE

FIRST RUNNING PROGRAM MIXER TO CALCULATE THE ENERGY DEPENDENT     GROUPIE

TOTAL CROSS SECTION FOR ANY COMPOSITE MIXTURE. NEXT, SUBSTITUTE   GROUPIE

THIS COMPOSITE TOTAL CROSS SECTION FOR THE ACTUAL TOTAL CROSS     GROUPIE

SECTION OF EACH CONSTITUENT (IN EACH ENDF/B FORMATTED EVALUATION).GROUPIE

FINALLY, RUN THIS PROGRAM TO CALCULATE THE SELF-SHIELDED CROSS    GROUPIE

SECTION FOR EACH CONSTITUENT, PROPERLY ACCOUNTING FOR RESONANCE   GROUPIE

OVERLAP BETWEEN THE RESONANCES OF ALL OF THE CONSTITUENTS OF THE  GROUPIE

MIXTURE. DURING THE SAME RUN THESE SELF-SHIELDED CROSS SECTIONS   GROUPIE

CAN IN TURN BE USED TO CALCULATE FULLY CORRELATED MULT-BAND       GROUPIE

GROUPIE

MULTI-BAND PARAMETER OUTPUT FORMAT                                GROUPIE

----------------------------------                                GROUPIE

FOR VERSIONS 92-2 AND LATER VERSIONS THE MULTI-BAND PARAMETERS    GROUPIE

ARE OUTPUT IN A SIMPLE CHARACTER FORMAT, THAT CAN BE TRANSFERRED  GROUPIE

AND USED ON VIRTUALLY ANY COMPUTER.                               GROUPIE

GROUPIE

THE BINARY FORMAT USED IN EARLIER VERSIONS OF THIS CODE IS NO     GROUPIE

LONGER USED.                                                      GROUPIE

GROUPIE

CONTACT THE AUTHOR IF YOU WOULD LIKE TO RECEIVE A SIMPLE PROGRAM  GROUPIE

TO READ THE CHARACTER FORMATTED MULTI-BAND PARAMETER FILE AND     GROUPIE

CREATE A BINARY, RANDOM ACCESS FILE FOR USE ON VIRTUALLY ANY      GROUPIE

COMPUTER.                                                         GROUPIE

GROUPIE

THE FORMAT OF THE CHARACTER FILE IS,                              GROUPIE

GROUPIE

RECORD   COLUMNS   FORMAT   DESCRIPTION                           GROUPIE

1       1-72     18A4    LIBRARY DESCRIPTION (AS READ)         GROUPIE

2       1-11      I11    MATERIAL ZA                           GROUPIE

12-22      I11    NUMBER GROUPS                         GROUPIE

23-33      I11    NUMBER OF BANDS                       GROUPIE

34-44     E11.4   TEMPERATURE (KELVIN)                  GROUPIE

45-57    1X,12A1  HOLLERITH DESCRIPTION OF ZA           GROUPIE

3       1-11     E11.4   ENERGY (EV) - GROUP BOUNDARY.         GROUPIE

12-22     E11.4   TOTAL      (FIRST BAND)               GROUPIE

23-33     E11.4   ELASTIC                               GROUPIE

34-44     E11.4   CAPTURE                               GROUPIE

35-55     E11.4   FISSION                               GROUPIE

4       1-11     -----   BLANK                                 GROUPIE

12-22     E11.4   TOTAL      (SECOND BAND)              GROUPIE

23-33     E11.4   ELASTIC                               GROUPIE

34-44     E11.4   CAPTURE                               GROUPIE

35-55     E11.4   FISSION                               GROUPIE

GROUPIE

LINES 3 AND 4 ARE REPEATED FOR EACH GROUP. THE LAST LINE FOR EACH GROUPIE

MATERIAL (ZA) IS,                                                 GROUPIE

GROUPIE

N       1-11     E11.4   ENERGY (EV) - UPPER ENERGY LIMIT OF   GROUPIE

LAST GROUP.             GROUPIE

GROUPIE

FOR EXAMPLE, A 175 GROUP, 2 BAND FILE, FOR EACH MATERIAL WILL     GROUPIE

CONTAIN 352 LINES = 1 HEADER LINE, 175 * 2 LINES OF PARAMETERS,   GROUPIE

AND 1 FINAL LINE WITH THE UPPER ENERGY LIMIT  GROUPIE

OF THE LAST GROUP.                            GROUPIE

GROUPIE

INPUT FILES                                                       GROUPIE

-----------                                                       GROUPIE

UNIT  DESCRIPTION                                                 GROUPIE

----  -----------                                                 GROUPIE

2   INPUT DATA (BCD - 80 CHARACTERS/RECORD)                     GROUPIE

10   ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)           GROUPIE

GROUPIE

OUTPUT FILES                                                      GROUPIE

------------                                                      GROUPIE

UNIT  DESCRIPTION                                                 GROUPIE

----  -----------                                                 GROUPIE

3   OUTPUT REPORT (BCD - 80 CHARACTERS/RECORD)                  GROUPIE

11   MULTI-GROUP ENDF/B DATA - OPTIONAL                          GROUPIE

(BCD - 80 CHARACTERS/RECORD)                                GROUPIE

16   PLOTTAB FORMATTED SELF-SHIELDING RESULTS                    GROUPIE

(BCD - 80 CHARACTERS/RECORD)                                GROUPIE

31   MULTI-BAND PARAMETERS CHARACTER FILE - OPTIONAL             GROUPIE

(BCD - 80 CHARACTERS/RECORD)                                GROUPIE

32   UNRESOLVED FSELF-SHIELDED PSEUDO ENDF FORMAT - OPTIONAL     GROUPIE

(BCD - 120 CHARACTERS/RECORD)                               GROUPIE

33   SELF-SHIELDED CROSS SECTION LISTING - OPTIONAL              GROUPIE

(BCD - 120 CHARACTERS/RECORD)                               GROUPIE

34   MULTI-BAND PARAMETER LISTING - OPTIONAL                     GROUPIE

(BCD - 120 CHARACTERS/RECORD)                               GROUPIE

35   UNSHIELDED CROSS SECTION LISTING - OPTION                   GROUPIE

(BCD - 120 CHARACTERS/RECORD)                               GROUPIE

GROUPIE

SCRATCH FILES                                                     GROUPIE

-------------                                                     GROUPIE

UNIT  FILENAME  DESCRIPTION                                       GROUPIE

----  --------  -----------                                       GROUPIE

8   ENERGY DEPENDENT WEIGHTING SPECTRUM                         GROUPIE

(BINARY - 40080 WORDS/BLOCK)                                GROUPIE

9   TOTAL CROSS SECTION                                         GROUPIE

(BINARY - 40080 WORDS/BLOCK)                                GROUPIE

12   ELASTIC CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION GROUPIE

(BINARY - 40080 WORDS/BLOCK)                                GROUPIE

13   CAPTURE CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION GROUPIE

(BINARY - 40080 WORDS/BLOCK)                                GROUPIE

14   FISSION CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION GROUPIE

(BINARY - 40080 WORDS/BLOCK)                                GROUPIE

GROUPIE

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)  GROUPIE

----------------------------------------------------------------  GROUPIE

UNIT  FILE NAME                                                   GROUPIE

----  ----------                                                  GROUPIE

2   GROUPIE.INP                                                 GROUPIE

3   GROUPIE.LST                                                 GROUPIE

----------------------                                                 GROUPIE

8   (SCRATCH)                                                   GROUPIE

9   (SCRATCH)                                                   GROUPIE

10   ENDFB.IN                                                    GROUPIE

11   ENDFB.OUT                                                   GROUPIE

12   (SCRATCH)                                                   GROUPIE

13   (SCRATCH)                                                   GROUPIE

14   (SCRATCH)                                                   GROUPIE

-----2019/6/23 - New Filenames (added ZAzzzaaa at Beginning)-----------GROUPIE

-----------(OLD)-------------(NEW)-------------------------------------GROUPIE

16   PLOTTAB.CUR     ZAzzzaaa.PLOT.CUR                           GROUPIE

31   MULTBAND.TAB    ZAzzzaaa.MULTBAND.TAB                       GROUPIE

32                   ZAzzzaaa.URR.ENDF                           GROUPIE

33   SHIELD.LST      ZAzzzaaa.SHIELD.LST                         GROUPIE

34   MULTBAND.LST    ZAzzzaaa.MULTBAND.LST                       GROUPIE

35   UNSHIELD.LST    ZAzzzaaa.UNSHIELD.LST                       GROUPIE

GROUPIE

I/O UNITS USED                                                   GROUPIE

--------------                                                   GROUPIE

UNITS 2, 3 8, 9 AND 10 WILL ALWAYS BE USED.                      GROUPIE

UNITS 31 THROUGH 35, 11 AND 16 ARE OPTIONALLY USED DEPENDING     GROUPIE

ON THE OUTPUT REQUESTED.                                         GROUPIE

UNITS 12, 13 AND 14 WILL ONLY BE USED IF SELF-SHIELDED OR        GROUPIE

MULTIBAND OUTPUT IS REQUESTED.                                   GROUPIE

GROUPIE

INPUT CARDS                                                       GROUPIE

-----------                                                       GROUPIE

CARD  COLS.  FORMAT  DESCRIPTION                                  GROUPIE

----  -----  ------  -----------                                  GROUPIE

1    1-11    I11   SELECTION CRITERIA (0=MAT, 1=ZA)             GROUPIE

1   12-22    I11   NUMBER OF GROUPS.                            GROUPIE

--------------------------2019/6/23 -11 (TART 616 groups) required for GROUPIE

Unresolved Resonance Region Self-Shielding   GROUPIE

calculation.                                 GROUPIE

=.GT.0 - ARBITRARY GROUP BOUNDARIES ARE READ GROUPIE

FROM INPUT FILE (N GROUPS REQUIRE   GROUPIE

N+1 GROUP BOUNDARIES). CURRENT      GROUPIE

PROGRAM MAXIMUM IS 20,000 GROUPS.   GROUPIE

BUILT-IN OPTIONS INCLUDE....        GROUPIE

=  0   - TART    175 GROUPS                  GROUPIE

= -1   - ORNL     50 GROUPS                  GROUPIE

= -2   - ORNL    126 GROUPS                  GROUPIE

= -3   - ORNL    171 GROUPS                  GROUPIE

= -4   - SAND-II 620 (665) GROUPS TO 18 MEV  GROUPIE

= -5   - SAND-II 640 (685) GROUPS TO 20 MEV  GROUPIE

= -6   - WIMS     69 GROUPS                  GROUPIE

= -7   - GAM-I    68 GROUPS                  GROUPIE

= -8   - GAM-II   99 GROUPS                  GROUPIE

= -9   - MUFT     54 GROUPS                  GROUPIE

=-10   - ABBN     28 GROUPS                  GROUPIE

Current TART Standard =-11   - TART    616 GROUPS TO 20 MEV        GROUPIE

(-11 is required for  =-12   - TART    700 GROUPS TO 1 GEV         GROUPIE

unresolved resonance =-13   - SAND-II 665 GROUPS TO 18 MEV        GROUPIE

region self-shielding=-14   - SAND-II 685 GROUPS TO 20 MEV        GROUPIE

calculations)        =-15   - TART    666 GROUPS TO 200 MEV       GROUPIE

=-16   - SAND-II 725 GROUPS TO 60 MEV        GROUPIE

=-17   - SAND-II 755 GROUPS TO 150 MEV       GROUPIE

=-18   - SAND-II 765 GROUPS TO 200 MEV       GROUPIE

=-19   - UKAEA  1102 GROUPS TO   1 GeV       GROUPIE

1   23-33    I11   MULTI-BAND SELECTOR                          GROUPIE

--------------------------2019/6/23 - ONLY 0 or 2 allowed = the = 1    GROUPIE

option has proven to give very poor results, GROUPIE

and therefore is no longer allowed.          GROUPIE

=  0 - NO MULTI-BAND CALCULATIONS            GROUPIE

No longer allowed   =  1 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)   GROUPIE

AND AV(1/TOT**2)                      GROUPIE

=  2 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)   GROUPIE

AND AV(1/(TOT+SIGMA0)) WHERE          GROUPIE

SIGMA0 = AV(TOT) IN EACH GROUP        GROUPIE

No longer allowed   = 3-5- MULTI-BAND FIT. CONSERVE AV(TOT) AND  GROUPIE

MINIMIZE FRACTIONAL ERROR FOR ENTIRE  GROUPIE

SELF-SHIELDING CURVE (SIGMA0 = 0 TO   GROUPIE

INFINITY)                             GROUPIE

IF THE SELECTOR IS POSITIVE (1 TO 5) THE     GROUPIE

MINIMUM NUMBER OF BANDS WILL BE OUTPUT FOR   GROUPIE

EACH ISOTOPE INDEPENDENTLY. IF THE SELECTOR  GROUPIE

IS NEGATIVE (-1 TO -5) THE SAME NUMBER OF    GROUPIE

BANDS (ABS(SELECTOR)) WILL BE OUTPUT FOR     GROUPIE

ALL ISOTOPES.                                GROUPIE

1   34-44    I11   NUMBER OF POINTS USED TO DESCRIBE ENERGY     GROUPIE

DEPENDENT WEIGHTING SPECTRUM S(E).           GROUPIE

= 0 or 1 - Flat (Constant)                   GROUPIE

= -1     - 1/E at ALL energies               GROUPIE

= -2     - MAXWELLIAN - UP TO 0.1 eV         GROUPIE

1/E        - 0.1 eV TO 67 KeV     GROUPIE

FISSION    - 67 KeV to 10 MeV     GROUPIE

CONSTANT   - Above 10 MeV         GROUPIE

= > 1    - Read input table                  GROUPIE

2005/01/20---------------ADDED OPTION TO ALLOW TEMPERATURE OF THE     GROUPIE

MAXWELLIAN TO BE CHANGED - SEE INPUT LINE 4, GROUPIE

COLUMNS 55 - 66.                             GROUPIE

= -1    - 1/E                                GROUPIE

= 0 OR 1- ENERGY INDEPENDENT (SO CALLED FLAT GROUPIE

WEIGHTING SPECTRUM).               GROUPIE

= .GT.1 - READ THIS MANY POINTS FROM INPUT   GROUPIE

TO DESCRIBE WEIGHTING SPECTRUM.    GROUPIE

NO LIMIT TO THE NUMBER OF POINTS   GROUPIE

USED TO DESCRIBE WEIGHTING.        GROUPIE

1   45-55   E11.4  MULTI-BAND CONVERGENCE CRITERIA.             GROUPIE

--------------------------2019/6/23 - No longer used now that code     GROUPIE

is restricted to no more than 2 bands.       GROUPIE

ONLY USED FOR 3 OR MORE BANDS. THE NUMBER OF GROUPIE

BANDS IN EACH GROUPS IS SELECTED TO INSURE   GROUPIE

THAT THE ENTIRE SELF-SHIELDING CURVE CAN BE  GROUPIE

REPRODUCED TO WITHIN THIS FRACTIONAL ERROR.  GROUPIE

= .LT. 0.0001 - USE STANDARD 0.001           GROUPIE

(0.1 PER-CENT)               GROUPIE

= .GE. 0.0001 - USE AS CONVERGENCE CRITERIA  GROUPIE

1   56-66    I11   SIGMA-0 DEFINITION SELECTOR.                 GROUPIE

--------------------------2019/6/23 - For multi-band calculations      GROUPIE

only 0 is alllowed = mulriples of unshielded GROUPIE

total in each group = This is required for   GROUPIE

the BEST self-shielding results.             GROUPIE

< 0 - 21 VALUES OF SIGMA0 ARE READ INPUT AND GROUPIE

INTERPRETED AS FIXED VALUES = SAME AS  GROUPIE

= 1 DESCRIPTION BELOW                  GROUPIE

INPUT VALUES MUST ALL BE,              GROUPIE

1) GREATER THAN 0                      GROUPIE

2) IN DESCENDING VALUE ORDER           GROUPIE

= 0 - SIGMA-0 WILL BE DEFINED AS A MULTIPLE  GROUPIE

OF THE UNSHIELDED TOTAL CROSS SECTION  GROUPIE

IN EACH GROUP (VALUES OF 1/1024 TO     GROUPIE

1024 IN STEPS OF A FACTOR OF 2 WILL    GROUPIE

BE USED AS THE MULTIPLIER).            GROUPIE

= 1 - SIGMA-0 WILL BE DEFINED AS THE SAME    GROUPIE

NUMBER OF BARNS IN EACH GROUP (VALUES  GROUPIE

40000 TO 0.4 BARNS WILL BE USED. WITHINGROUPIE

EACH DECADE VALUES OF 10, 7, 4, 2, 1   GROUPIE

BARNS WILL BE USED).                   GROUPIE

1   67-70    I4    High energy extension = definition of cross  GROUPIE

section above highest tabulated energy.      GROUPIE

--------------------------2019/6/23 - Ignored - will always use ENDF   GROUPIE

Standard Definition = 0.                     GROUPIE

= 0 = cross section = 0 (standard ENDF/B)    GROUPIE

= 1 = cross section = constant (equal to     GROUPIE

value at highest tabulated energy).    GROUPIE

2-4    1-66 6E11.4   SIGMA-0 Definition                           GROUPIE

--------------------------2019/6/23 - Only the GROUPIE standard = 0    GROUPIE

in allowed for Unresolved Resonance Region   GROUPIE

Self-Shielding calculation                   GROUPIE

IF SIGMA-0 DEFINITION SELECTOR < 0, THE NEXT GROUPIE

4 LINES OF INPUT ARE THE 22 VALUES OF SIGMA0,GROUPIE

6 PER LINE.                                  GROUPIE

2    1-72    A72   ENDF/B INPUT DATA FILENAME                   GROUPIE

(STANDARD OPTION = ENDFB.IN)                 GROUPIE

3    1-72    A72   ENDF/B OUTPUT DATA FILENAME                  GROUPIE

(STANDARD OPTION = ENDFB.OUT)                GROUPIE

GROUPIE

THE FOURTH INPUT CARD IS USED TO SELECT ALL DESIRED OUTPUT MODES. GROUPIE

EACH OUTPUT DEVICE MAY BE TURNED OFF (0) OR ON (1). THEREFORE     GROUPIE

THEREFORE EACH OF THE FOLLOWING INPUT PARAMETERS MAY BE EITHER    GROUPIE

ZERO TO INDICATE NO OUTPUT OR NON-ZERO TO INDICATE OUTPUT.        GROUPIE

GROUPIE

4     1-11   I11   SELF-SHIELDED CROSS SECTION LISTING          GROUPIE

= 1 - CROSS SECTIONS                         GROUPIE

= 2 - RESONANCE INTEGRALS                    GROUPIE

4    12-22   I11   MULTI-BAND PARAMETER LISTING                 GROUPIE

4    23-33   I11   MULTI-BAND PARAMETERS COMPUTER READABLE      GROUPIE

4    34-44   I11   UNSHIELDED CROSS SECTIONS IN ENDF/B FORMAT   GROUPIE

= 1 - HISTOGRAM FORMAT (INTERPOLATION LAW 1) GROUPIE

= 2 - LINEAR-LINEAR (INTERPOLATION LAW 2)    GROUPIE

4    45-55   I11   UNSHIELDED CROSS SECTIONS LISTING            GROUPIE

= 1 - CROSS SECTIONS                         GROUPIE

= 2 - RESONANCE INTEGRALS                    GROUPIE

05/01/20 - ADDED THE BELOW OPTION                                     GROUPIE

4    56-66   E11.4 IF THE STANDARD BUILT-IN SPECTRA IS USED,    GROUPIE

INPUT LINE 1, COLUMNS 34-44 = 2, THIS FIELD  GROUPIE

CAN BE USED TO OPTIONALLY CHANGE TEMPERATURE GROUPIE

OF THE MAXWELLIAN.                           GROUPIE

INPUT IS IN EV (0.0253 EV = ROOM TEMPERATURE)GROUPIE

= 0 - USE DEFAULT 0.0253 EV, ROOM TEMPERATUREGROUPIE

> 0 - USE THIS AS THE TEMPERATURE            GROUPIE

RESTRICTION - TEMPERATURE CANNOT EXCEED      GROUPIE

1000 EV.                                     GROUPIE

GROUPIE

5     1-80   18A4  LIBRARY IDENTIFICATION. ANY TEXT THAT THE    GROUPIE

USER WISHES TO IDENTIFY THE MULTI-BAND       GROUPIE

PARAMETERS. THIS LIBRARY IDENTIFICATION IS   GROUPIE

WRITTEN INTO THE COMPUTER READABLE MULTI-BANDGROUPIE

DATA FILE.                                   GROUPIE

GROUPIE

6-N    1- 6    I6   LOWER MAT OR ZA LIMIT                        GROUPIE

7- 8    I2   LOWER MF LIMIT                               GROUPIE

9-11    I3   LOWER MT LIMIT                               GROUPIE

12-17   I11   UPPER MAT OR ZA LIMIT                        GROUPIE

18-19    I2   UPPER MF LIMIT                               GROUPIE

20-22    I3   UPPER MT LIMIT                               GROUPIE

UP TO 100 RANGES MAY BE SPECIFIED, ONE RANGE GROUPIE

PER LINE. THE LIST OF RANGES IS TERMINATED   GROUPIE

BY A BLANK CARD. IF THE UPPER MAT OR ZA      GROUPIE

LIMIT IS LESS THAN THE LOWER LIMIT THE UPPER GROUPIE

IS SET EQUAL TO THE LOWER LIMIT. IF THE UPPERGROUPIE

MF OR MT LIMIT IS ZERO IT WILL BE SET EQUAL  GROUPIE

TO ITS MAXIMUM VALUE, 99 OR 999, RESPECTIVELYGROUPIE

IF THE FIRST REQUEST LINE IS BLANK IT WILL   GROUPIE

TERMINATE THE LIST OF REQUESTS AND CAUSE ALL GROUPIE

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).    GROUPIE

GROUPIE

VARY   1-66  6E11.4 ENERGY GROUP BOUNDARIES. ONLY REQUIRED IF    GROUPIE

THE NUMBER OF GROUPS INDICATED ON THE FIRST  GROUPIE

INPUT CARD IS POSITIVE. ALL ENERGIES MUST    GROUPIE

BE IN ASCENDING ENERGY IN EV. THE PRESENT    GROUPIE

LIMITS ARE 1 TO 20,000 GROUPS. FOR N GROUPS  GROUPIE

N+1 BOUNDARIES WILL BE READ FROM THE         GROUPIE

INPUT FILE, E.G. IF THE FIRST INPUT CARD     GROUPIE

INDICATES 20 GROUPS, 21 ENERGY BOUNDARIES    GROUPIE

WILL BE READ FROM THE INPUT FILE.            GROUPIE

GROUPIE

VARY   1-66  6E11.4 ENERGY DEPENDENT WEIGHTING SPECTRUM. ONLY    GROUPIE

REQUIRED IF THE NUMBER OF POINTS INDICATED   GROUPIE

ON FIRST CARD IS MORE THAN ONE. DATA IS      GROUPIE

GIVEN IN (ENERGY, WEIGHT) PAIRS, UP TO 3     GROUPIE

PAIRS PER CARD, USING ANY NUMBER OF CARDS    GROUPIE

REQUIRED. ENERGIES MUST BE IN ASCENDING      GROUPIE

ORDER IN EV. THE SPECTRUM VALUES MUST BE     GROUPIE

NON-NEGATIVE. THE ENERGY RANGE OF SPECTRUM   GROUPIE

MUST AT LEAST SPAN THE ENERGY RANGE OF THE   GROUPIE

ENERGY GROUPS. SINCE SPECTRUM IS STORED IN   GROUPIE

PAGING SYSTEM THERE IS NO LIMIT TO NUMBER    GROUPIE

OF POINTS THAT CAN BE USED TO DESCRIBE THE   GROUPIE

WEIGHTING SPECTRUM.                          GROUPIE

GROUPIE

EXAMPLE INPUT NO. 1                                               GROUPIE

-------------------                                               GROUPIE

REQUEST DATA BY MAT AND PROCESS ALL DATA (ALL MAT BETWEEN 1 AND   GROUPIE

9999). USE THE TART 175 GROUP STRUCTURE, GENERATE 2 BAND          GROUPIE

PARAMETERS (THE FOR ALL ISOTOPES) TO 0.1 PER-CENT ACCURACY        GROUPIE

IN THE SELF-SHIELDING CURVE. OUTPUT ALL  LISTING, COMPUTER        GROUPIE

READABLE AND ENDF/B FORMAT GROUP AVERAGES.                        GROUPIE

GROUPIE

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                        GROUPIE

GROUPIE

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                         GROUPIE

GROUPIE

0          0         -2          0 1.00000-03          0     GROUPIE

ENDFB.IN                                                              GROUPIE

ENDFB.OUT                                                             GROUPIE

1          1          1          1          1                GROUPIE

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY               GROUPIE

1 1  1  9999 0  0                                                 GROUPIE

(BLANK CARD TERMINATES REQUEST LIST)            GROUPIE

GROUPIE

EXAMPLE INPUT NO. 2                                               GROUPIE

-------------------                                               GROUPIE

THE SAME EXAMPLE 1, AS ABOVE, ONLY THE ENDF/B DATA WILL BE READ   GROUPIE

FROM \ENDFB6\SIGMA1\K300\ZA092238 (U-238 AT 300 KELVIN) AND       GROUPIE

WRITTEN TO \ENDFB6\GROUPIE\K300\ZA092238                          GROUPIE

GROUPIE

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                         GROUPIE

GROUPIE

0          0         -2          0 1.00000-03          0     GROUPIE

\ENDFB6\SIGMA1\K300\ZA092238                                          GROUPIE

\ENDFB6\GROUPIE\K300\ZA092238                                         GROUPIE

1          1          1          1          1                GROUPIE

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY               GROUPIE

1 1  1  9999 0  0                                                 GROUPIE

(BLANK CARD TERMINATES REQUEST LIST)            GROUPIE

GROUPIE

EXAMPLE INPUT NO. 3                                               GROUPIE

-------------------                                               GROUPIE

PROCESS ALL DATA. USE 1/E WEIGHTING IN ORDER TO CALCULATE         GROUPIE

UNSHIELDED ONE GROUP CROSS SECTIONS OVER THE ENERGY RANGE 0.5 EV  GROUPIE

TO 1 MEV (NOTE THAT THE RESULTS ARE SIMPLY PROPORTIONAL TO THE    GROUPIE

RESONANCE INTEGRAL FOR EACH REACTION). OUTPUT UNSHIELDED LISTING. GROUPIE

GROUPIE

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL    GROUPIE

THEN USE STANDARD FILENAMES.                                      GROUPIE

GROUPIE

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                         GROUPIE

GROUPIE

0          0          1         -1                     0     GROUPIE

(USE STANDARD FILENAME = ENDFB.IN)              GROUPIE

(USE STANDARD FILENAME = ENDFB.OUT)             GROUPIE

0          0          0          0          1                GROUPIE

RESONANCE INTEGRAL CALCULATION (FROM 0.5 EV TO 1 MEV)                 GROUPIE

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)     GROUPIE

5.00000-01 1.00000+06                                                 GROUPIE

GROUPIE

EXAMPLE INPUT NO. 4                                               GROUPIE

-------------------                                               GROUPIE

THIS EXAMPLE USES A USER DEFINED GROUP STRUCTURE AND WEIGHTING    GROUPIE

FUNCTION - THESE ARE NOT REALISTIC IN TERMS OF ACTUAL ENERGIES    GROUPIE

AND WEIGHTS - THEY ARE ONLY INTENDED TO ILLUSTRATE THE ORDER OF   GROUPIE

THE INPUT PARAMETERS.                                             GROUPIE

GROUPIE

0         11          0          6                     0     GROUPIE

RECENT.OUT                                                            GROUPIE

GROUPIE.OUT                                                           GROUPIE

1          1          1          1          1                GROUPIE

Example with users defined groupus and spectrum weighting             GROUPIE

1 1  1  999999999                                                 GROUPIE

(blabk line terminates request list)            GROUPIE

1.00000-05 1.00000-04 1.00000-03 1.00000-02 1.00000-01 1.00000+00 grouGROUPIE

1.00000+01 1.00000+02 1.00000+03 1.00000+04 1.00000+05 1.00000+06 grouGROUPIE

1.00000-05 1.0        1.00000-02 0.1        1.00000+00 0.01       weigGROUPIE

1.00000+02 0.001      1.00000+04 0.0001     1.00000+06 0.000001   weigGROUPIE

GROUPIE

=======================================================================GROUPIE