=======================================================================LINEAR

LINEAR

PROGRAM LINEAR                                                    LINEAR

==============                                                    LINEAR

VERSION 74-1 (MAY 1974)                                           LINEAR

VERSION 75-1 (APRIL 1975)                                         LINEAR

VERSION 76-2 (OCTOBER 1976)                                       LINEAR

VERSION 77-1 (JANUARY 1977)                                       LINEAR

VERSION 78-1 (JULY 1978)                                          LINEAR

VERSION 79-1 (JULY 1979) CDC-7600 AND CRAY-1 VERSION.             LINEAR

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION.                LINEAR

VERSION 80-2 (DECEMBER 1980)                                      LINEAR

VERSION 81-1 (MARCH 1981)                                         LINEAR

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY.      LINEAR

VERSION 83-1 (JANUARY 1983) *MAJOR RE-DESIGN.                     LINEAR

*PAGE SIZE INCREASED - 1002 TO 3006.  LINEAR

*ELIMINATED COMPUTER DEPENDENT CODING.LINEAR

*NEW, MORE COMPATIBLE I/O UNIT NUMBER.LINEAR

*ADDED OPTION TO KEEP ALL ORIGINAL    LINEAR

ENERGY POINTS FROM EVALUATION.       LINEAR

*ADDED STANDARD ALLOWABLE ERROR OPTIONLINEAR

(CURRENTLY 0.1 PER-CENT).            LINEAR

VERSION 83-2 (OCTOBER 1983) IMPROVED BASED ON USER COMMENTS.      LINEAR

VERSION 84-1 (APRIL 1984)   IMPROVED BASED ON USER COMMENTS.      LINEAR

VERSION 84-2 (JUNE 1984)   *UPDATED FOR ENDF/B-6 FORMATS.         LINEAR

*SPECIAL I/O ROUTINES TO GUARANTEE     LINEAR

ACCURACY OF ENERGY.                   LINEAR

*DOUBLE PRECISION TREATMENT OF ENERGY  LINEAR

(REQUIRED FOR NARROW RESONANCES).     LINEAR

VERSION 85-1 (AUGUST 1985) *FORTRAN-77/H VERSION                  LINEAR

VERSION 86-1 (JANUARY 1986)*ENDF/B-6 FORMAT                       LINEAR

VERSION 87-1 (JANUARY 1987)*DOUBLE PRECISION TREATMENT OF CROSS   LINEAR

SECTION                               LINEAR

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O    LINEAR

FILE NAMES (SEE, SUBROUTINE FILEIO    LINEAR

FOR DETAILS).                         LINEAR

*IMPROVED BASED ON USER COMMENTS.      LINEAR

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO    LINEAR

INSURE PROGRAM WILL NOT DO ANYTHING   LINEAR

CRAZY.                                LINEAR

*UPDATED TO USE NEW PROGRAM CONVERT    LINEAR

KEYWORDS.                             LINEAR

*ADDED LIVERMORE CIVIC COMPILER        LINEAR

CONVENTIONS.                          LINEAR

VERSION 90-1 (JUNE 1990)   *EXTENDED TO LINEARIZE PHOTON          LINEAR

INTERACTION DATA, MF=23 AND 27        LINEAR

*ADDED FORTRAN SAVE OPTION             LINEAR

*UPDATED BASED ON USER COMMENTS.       LINEAR

*NEW MORE CONSISTENT ENERGY OUTPUT     LINEAR

ROUTINE.                              LINEAR

*WARNING...INPUT PARAMETER FORMAT      LINEAR

HAS BEEN CHANGED...SEE DESCRIPTION    LINEAR

BELOW.                                LINEAR

VERSION 91-1 (JULY 1991)   *ADDED INTERPOLATION LAW 6 - ONLY USED LINEAR

FOR CHARGED PARTICLE CROSS SECTIONS   LINEAR

FOR COULOMB PENETRABILITIES.          LINEAR

VERSION 92-1 (JANUARY 1992)*ADDED NU-BAR (TOTAL, DELAYED, PROMPT) LINEAR

POLYNOMIAL OR TABULATED ALL CONVERTED LINEAR

TO LINEARLY INTERPOLABLE              LINEAR

*INCREASED PAGE SIZE FROM 3006 TO 5010 LINEAR

POINTS.                               LINEAR

*ALL ENERGIES INTERNALLY ROUNDED PRIOR LINEAR

TO CALCULATIONS.                      LINEAR

*COMPLETELY CONSISTENT I/O AND ROUNDINGLINEAR

ROUTINES - TO MINIMIZE COMPUTER       LINEAR

DEPENDENCE.                           LINEAR

VERSION 92-2 (JULY 1992)   *CORRECTED CONVERSION OF NU-BAR FROM   LINEAR

POLYNOMIAL TO TABULATED - COPY        LINEAR

SPONTANEOUS NU-BAR (BY DEFINITION     LINEAR

THE SPONTANEOUS NU-BAR IS NOT AN      LINEAR

ENERGY DEPENDENT QUANTITY).           LINEAR

VERSION 93-1 (MARCH 1993)  *UPDATED FOR USE WITH LAHEY COMPILER   LINEAR

ON IBM-PCS.                           LINEAR

*INCREASED PAGE SIZE FROM 5010 TO      LINEAR

30000 POINTS                          LINEAR

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES        LINEAR

TO ALLOW ACCESS TO FILE STRUCTURES    LINEAR

(WARNING - INPUT PARAMETER FORMAT     LINEAR

HAS BEEN CHANGED)                     LINEAR

*CLOSE ALL FILES BEFORE TERMINATING    LINEAR

(SEE, SUBROUTINE ENDIT)               LINEAR

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    LINEAR

*IMPROVED COMPUTER INDEPENDENCE       LINEAR

*ALL DOUBLE PRECISION                 LINEAR

*ON SCREEN OUTPUT                     LINEAR

*UNIFORM TREATMENT OF ENDF/B I/O      LINEAR

*IMPROVED OUTPUT PRECISION            LINEAR

*DEFINED SCRATCH FILE NAMES           LINEAR

*ALWAYS INCLUDE THERMAL VALUE         LINEAR

*INCREASED PAGE SIZE FROM 30000 TO    LINEAR

60000 POINTS                         LINEAR

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      LINEAR

POINT READ FOR MORE DIGITS           LINEAR

*UPDATED TEST FOR ENDF/B FORMAT       LINEAR

VERSION BASED ON RECENT FORMAT CHANGELINEAR

*GENERAL IMPROVEMENTS BASED ON        LINEAR

USER FEEDBACK                        LINEAR

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING  LINEAR

MF=1, MT-451.                        LINEAR

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF = 9 AND 10 LINEARIZATION    LINEAR

*GENERAL IMPROVEMENTS BASED ON        LINEAR

USER FEEDBACK                        LINEAR

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS            LINEAR

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACKLINEAR

VERS. 2005-1 (JAN. 2005)    *ALWAYS KEEP ORIGINAL TABULATED       LINEAR

NU-BAR POINTS.                       LINEAR

VERS. 2006-1 (FEB. 2006)    *CORRECTED INT=6 NEAR THRESHOLD       LINEAR

*NO SUBDIVIDE BELOW MINIMUM XCLOW     LINEAR

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.      LINEAR

*INCREASED PAGE SIZE FROM 60,000 TO   LINEAR

600,000 POINTS                       LINEAR

VERS. 2007-2 (DEC. 2007)    *72 CHARACTER FILE NAMES.             LINEAR

VERS. 2010-1 (Apr. 2010)    *Skipped leading cross section = 0    LINEAR

up to effective start, unless keepingLINEAR

ALL original energy points.          LINEAR

*Replaced ETHRES by ESTART - it is    LINEAR

not a threshold - just a minimum     LINEAR

energy - if a section starts above   LINEAR

this energy with a positive cross    LINEAR

section, an additional point will    LINEAR

inserted with cross section = 0.     LINEAR

VERS. 2012-1 (Aug. 2012)    *Minor Updates based on User Feedback.LINEAR

*Added CODENAME                       LINEAR

*32 and 64 bit Compatible             LINEAR

*Added ERROR stops.                   LINEAR

VERS. 2012-2 (Nov. 2012)    *Never thin nu-bar.                   LINEAR

VERS. 2013-1 (Nov. 2013)    *Extended OUT9.                       LINEAR

VERS. 2015-1 (Jan. 2015)    *Allow Imaginary Anomolous Scattering LINEAR

Factor to be Negative (MF/MT=27/506).LINEAR

*Replaced ALL 3 way IF Statements.    LINEAR

VERS. 2016-1 (June 2016)    *Cosmetic changes based on FREUD      LINEAR

psychoanalysis.                      LINEAR

VERS. 2017-1 (May  2017)    *Updated based on user feedback.      LINEAR

*Inceased page size to 3,000,000.     LINEAR

*All floating input parameters changedLINEAR

to character input + IN9 conversion. LINEAR

VERS. 2018-1 (Dec. 2018)    *Updated based on user feedback.      LINEAR

*Added on-line output for ALL ENDERRORLINEAR

VERS. 2019-1 (June 2019)    *Additional Interpolation Law Tests   LINEAR

*Checked Maximum Tabulated Energy to  LINEAR

insure it is the same for all MTs -  LINEAR

if not, print WARNING messages.      LINEAR

*Corrected END Histogram linearized - LINEAR

Previously assumed Y = 0 and deleted LINEAR

now whatever the value it is includedLINEAR

VERS. 2020-1 (Dec. 2020)    *Major Re-write of Convergence        LINEAR

*Replaced INCORE9 by INCORE10.        LINEAR

*Added Target Isomer Flag             LINEAR

*Keep iterating toward MAX & MIN      LINEAR

VERS. 2021-1 (Mar. 2021)    *Complete re-write of convergence.    LINEAR

*Optionlly add MF/MT=1/451 comments   LINEAR

*Updated from FORTRAN 2018            LINEAR

*Minimum Cross Section is no longer   LINEAR

an input option = set to 1.0d-30.    LINEAR

LINEAR

OWNED, MAINTAINED AND DISTRIBUTED BY                              LINEAR

------------------------------------                              LINEAR

THE NUCLEAR DATA SECTION                                          LINEAR

INTERNATIONAL ATOMIC ENERGY AGENCY                                LINEAR

P.O. BOX 100                                                      LINEAR

A-1400, VIENNA, AUSTRIA                                           LINEAR

EUROPE                                                            LINEAR

LINEAR

ORIGINALLY WRITTEN BY                                             LINEAR

------------------------------------                              LINEAR

Dermott E. Cullen                                                 LINEAR

LINEAR

PRESENT CONTACT INFORMATION                                       LINEAR

---------------------------                                       LINEAR

Dermott E. Cullen                                                 LINEAR

1466 Hudson Way                                                   LINEAR

Livermore, CA 94550                                               LINEAR

U.S.A.                                                            LINEAR

Telephone  925-443-1911                                           LINEAR

E. Mail    RedCullen1@Comcast.net                                 LINEAR

Website    RedCullen1.net/HOMEPAGE.NEW                            LINEAR

LINEAR

AUTHORS MESSAGE                                                   LINEAR

---------------                                                   LINEAR

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION  LINEAR

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDEREDLINEAR

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASELINEAR

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                 LINEAR

LINEAR

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTERLINEAR

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE    LINEAR

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECTLINEAR

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY      LINEAR

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO LINEAR

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF   LINEAR

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR        LINEAR

COMPUTER.                                                         LINEAR

LINEAR

PURPOSE                                                           LINEAR

-------                                                           LINEAR

THIS PROGRAM IS DESIGNED TO CONVERT ENDF/B FILE 3, 23 AND 27 DATA LINEAR

TO LINEAR-LINEAR INTERPOLABLE FORM. ANY SECTION THAT IS ALREADY   LINEAR

LINEAR-LINEAR INTERPOLABLE WILL BE THINNED.                       LINEAR

LINEAR

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY LINEAR

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE   LINEAR

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                            LINEAR

LINEAR

ENDF/B FORMAT                                                     LINEAR

-------------                                                     LINEAR

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS    LINEAR

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION  LINEAR

OF THE ENDF/B FORMAT (I.E., ENDF/B-1, 2, 3, 4, 5, 6 FORMAT).      LINEAR

LINEAR

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B      LINEAR

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS    LINEAR

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE LINEAR

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE         LINEAR

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451 LINEAR

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL  LINEAR

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO  LINEAR

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.           LINEAR

LINEAR

OUTPUT FORMAT                                                     LINEAR

-------------                                                     LINEAR

IN THIS VERSION OF LINEAR ALL ENERGIES WILL BE OUTPUT IN          LINEAR

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN  LINEAR

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN LINEAR

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS  LINEAR

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS   LINEAR

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE LINEAR

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA    LINEAR

DUE TO TRUNCATION OF ENERGIES TO 6 DIGITS DURING OUTPUT.          LINEAR

LINEAR

CONTENTS OF OUTPUT                                                LINEAR

------------------                                                LINEAR

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE LINEARIZED DATA       LINEAR

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO    LINEAR

INCLUDED.                                                         LINEAR

LINEAR

DOCUMENTATION                                                     LINEAR

-------------                                                     LINEAR

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED LINEAR

BY THE ADDITION OF 3 COMMENT LINES AT THE END OF EACH HOLLERITH   LINEAR

SECTION IN THE FORM                                               LINEAR

LINEAR

***************** PROGRAM LINEAR (2021-1) ****************        LINEAR

FOR ALL DATA GREATER THAN 1.00000-30 IN ABSOLUTE VALUE            LINEAR

DATA LINEARIZED TO WITHIN AN ACCURACY OF  0.1  PER-CENT           LINEAR

LINEAR

THE ORDER OF SIMILAR COMMENTS (FROM RECENT, SIGMA1 AND GROUPIE)   LINEAR

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON      LINEAR

THE DATA BY THESE PROGRAMS.                                       LINEAR

LINEAR

THESE COMMENT LINES ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS,LINEAR

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMATLINEAR

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF     LINEAR

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451   LINEAR

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF     LINEAR

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF     LINEAR

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO         LINEAR

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND  LINEAR

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT LINEAR

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                     LINEAR

LINEAR

REACTION INDEX                                                    LINEAR

--------------                                                    LINEAR

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN    LINEAR

SECTION MF=1, MT=451 OF EACH EVALUATION.                          LINEAR

LINEAR

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.  LINEAR

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT        LINEAR

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WASLINEAR

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING LINEAR

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE LINEAR

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAMLINEAR

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.    LINEAR

LINEAR

SECTION SIZE                                                      LINEAR

------------                                                      LINEAR

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT LINEAR

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS     LINEAR

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                LINEAR

LINEAR

FOR ANY LINEARIZED SECTION THAT CONTAINS 60000 OR FEWER POINTS    LINEAR

THE ENTIRE OPERATION WILL BE PERFORMED IN CORE AND THE LINEARIZED LINEAR

DATA WILL BE OUTPUT DIRECTLY TO THE ENDF/B FORMAT. FOR ANY SECTIONLINEAR

THAT CONTAINS MORE POINTS THE DATA WILL BE LINEARIZED A PAGE AT A LINEAR

TIME (1 PAGE = 60000 POINTS) AND OUTPUT TO SCRATCH. AFTER THE     LINEAR

ENTIRE SECTION HAS BEEN LINEARIZED THE DATA WILL BE READ BACK FROMLINEAR

SCRATCH AND OUTPUT TO THE ENDF/B FORMAT.                          LINEAR

LINEAR

SELECTION OF DATA                                                 LINEAR

-----------------                                                 LINEAR

THE PROGRAM SELECTS DATA TO BE LINEARIZED BASED EITHER ON EITHER  LINEAR

MAT (ENDF/B MAT NO.) OR ZA AS WELL AS MF AND MT NUMBERS. THIS     LINEAR

PROGRAM ALLOWS UP TO 100 MAT/MF/MT OR ZA/MF/MT RANGES TO BE       LINEAR

SPECIFIED BY INPUT PARAMETERS. THE PROGRAM WILL ASSUME THAT THE   LINEAR

ENDF/B TAPE IS IN MAT ORDER, REGARDLESS OF THE CRITERIA USED      LINEAR

TO RETRIEVE MATERIALS. IF RETRIEVAL IS BY MAT RANGE THE PROGRAM   LINEAR

WILL TERMINATE WHEN A MAT IS FOUND THAT IS ABOVE ALL REQUESTED    LINEAR

MAT RANGES. IF RETRIEVAL IS BY ZA RANGE THE PROGRAM WILL SEARCH   LINEAR

THE ENTIRE ENDF/B TAPE.                                           LINEAR

LINEAR

PROGRAM OPERATION                                                 LINEAR

-----------------                                                 LINEAR

EACH SECTION OF DATA IS CONSIDERED SEPARATELY. EACH SECTION OF    LINEAR

ENDF/B DATA TO LINEARIZE IS REPRESENTED BY A TABLE OF ENERGY      LINEAR

VS. CROSS SECTION AND ANY ONE OF FIVE ALLOWABLE INTERPOLATION LAWSLINEAR

BETWEEN ANY TWO TABULATED POINTS. THIS PROGRAM WILL REPLACE EACH  LINEAR

SECTION OF DATA CROSS SECTIONS BY A NEW TABLE OF ENERGY VS.       LINEAR

CROSS SECTION IN WHICH THE INTERPOLATION LAW IS ALWAYS LINEAR IN  LINEAR

ENERGY AND CROSS SECTION BETWEEN ANY TWO TABULATED POINTS.        LINEAR

LINEAR

DATA IS READ AND LINEARIZED A PAGE AT A TIME (ONE PAGE CONTAINS   LINEAR

60000 DATA POINTS). IF THE FINAL LINEARIZED SECTION CONTAINS TWO  LINEAR

PAGES OR LESS, DATA POINTS IT WILL BE ENTIRELY CORE RESIDENT      LINEAR

AFTER IT HAS BEEN LINEARIZED AND WILL BE WRITTEN DIRECTLY FROM    LINEAR

CORE TO THE OUTPUT TAPE. IF THE LINEARIZED SECTION IS LARGER THAN LINEAR

TWO PAGES, AFTER EACH PAGE IS LINEARIZED IT WILL BE WRITTEN TO    LINEAR

SCRATCH. AFTER THE ENTIRE SECTION HAS BEEN LINEARIZED IT WILL     LINEAR

BE READ BACK FROM SCRATCH, TWO PAGES AT A TIME, AND WRITTEN TO    LINEAR

THE OUTPUT TAPE.                                                  LINEAR

LINEAR

KEEP EVALUATED DATA POINTS                                        LINEAR

--------------------------                                        LINEAR

SOMETIMES IT IS CONVENIENT TO KEEP ALL ENERGY POINTS WHICH WERE   LINEAR

PRESENT IN THE ORIGINAL EVALUATION AND TO MERELY SUPPLEMENT THESE LINEAR

POINTS WITH ADDITIONAL ENERGY POINTS IN ORDER TO LINEARIZE THE    LINEAR

CROSS SECTIONS. FOR EXAMPLE, IT IS OFTEN CONVENIENT TO KEEP THE   LINEAR

THERMAL VALUE (AT 0.0253 EV) OR THE VALUE AT 14.1 MEV.            LINEAR

LINEAR

THE CURRENT VERSION OF THIS PROGRAM WILL ALLOW THE USER TO KEEP   LINEAR

ALL ORIGINAL EVALUATED DATA POINTS BY SPECIFYING 1 IN COLUMNS     LINEAR

34-44 OF THE FIRST INPUT LINE. THIS WILL TURN OFF THE BACKWARD    LINEAR

THINNING (SEE UCRL-50400, VOL. 17, PART A FOR EXPLANATION) AND    LINEAR

RESULT IN ALL ORIGINAL ENERGY POINTS BEING KEPT. CAUTION SHOULD   LINEAR

BE EXERCISED IN USING THIS OPTION SINCE IT CAN RESULT IN A        LINEAR

CONSIDERABLE INCREASE IN THE NUMBER OF DATA POINTS OUTPUT BY      LINEAR

THIS CODE.                                                        LINEAR

LINEAR

FOR ALL USERS WHO ARE NOT INTERESTED IN THIS OPTIONS NO CHANGES   LINEAR

ARE REQUIRED IN THE INPUT TO THIS PROGRAM, I. E. IF COLUMNS       LINEAR

34-44 ARE BLANK (AS FOR ALL PREVIOUS VERSIONS OF THIS CODE) THE   LINEAR

PROGRAM WILL OPERATE EXACTLY AS IT DID BEFORE.                    LINEAR

LINEAR

ALLOWABLE ERROR                                                   LINEAR

---------------                                                   LINEAR

ALLOWABLE ERROR MUST ALWAYS BE SPECIFIED IN THE INPUT TO THIS     LINEAR

PROGRAM AS A FRACTION, NOT A PER-CENT. FOR EXAMPLE, INPUT THE     LINEAR

ALLOWABLE FRACTIONAL ERROR 0.001 IN ORDER TO OBTAIN DATA THAT IS  LINEAR

ACCURATE TO WITHIN 0.1 PER-CENT.                                  LINEAR

LINEAR

THE CONVERSION OF THE DATA FROM THE GENERAL INTERPOLATION FORM TO LINEAR

LINARLY INTERPOLABLE FORM CANNOT BE PERFORMED EXACTLY. HOWEVER, ITLINEAR

CAN BE PERFORMED TO VIRTUALLY ANY REQUIRED ACCURACY AND MOST      LINEAR

IMPORTANTLY CAN BE PERFORMED TO A TOLERANCE THAT IS SMALL COMPAREDLINEAR

TO THE UNCERTAINTY IN THE CROSS SECTIONS THEMSELVES. AS SUCH THE  LINEAR

CONVERSION OF CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM CAN BE LINEAR

PERFORMED WITH ESSENTIALLY NO LOSE OF INFORMATION.                LINEAR

LINEAR

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGYLINEAR

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED        LINEAR

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATIONLINEAR

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THELINEAR

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.   LINEAR

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR  LINEAR

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE  LINEAR

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.    LINEAR

LINEAR

DEFAULT ALLOWABLE ERROR                                           LINEAR

-----------------------                                           LINEAR

IN ORDER TO INSURE CONVERGENCE OF THE LINEARIZING ALGORITHM THE   LINEAR

ALLOWABLE ERROR MUST BE POSITIVE. IF THE USER INPUTS AN ERROR     LINEAR

THAT IS NOT POSITIVE IT WILL AUTOMATICALLY BE SET TO THE DEFAULT  LINEAR

VALUE (CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT) AND        LINEAR

INDICATED AS SUCH IN THE OUTPUT LISTING.                          LINEAR

LINEAR

COULOMB PENETRABILITY (INTERPOLATION LAW = 6)                     LINEAR

--------------------------------------------                      LINEAR

INTRODUCED FOR ENDF/B-VI. THIS IS DEFINED AS,                     LINEAR

LINEAR

SIG(E) = C1*EXP(-C2/SQRT(E - T))                                  LINEAR

LINEAR

THIS PROGRAM ONLY CONSIDERS EXOTHERMIC REACTIONS - T = 0          LINEAR

LINEAR

SIG(E) = C1*EXP(-C2/SQRT(E))                                      LINEAR

LINEAR

WARNING...THIS INTERPOLATION LAW SHOULD ONLY BE USED FOR REACTIONSLINEAR

WHICH HAVE A POSITIVE Q-VALUE (EXOTHERMIC REACTIONS),   LINEAR

SINCE HERE WE ONLY CONSIDER T = 0.0 IN THE FORMALISM.   LINEAR

IN ALL OTHER CASES A WARNING MESSAGE WILL BE PRINTED.   LINEAR

LINEAR

INPUT FILES                                                       LINEAR

-----------                                                       LINEAR

UNIT  DESCRIPTION                                                 LINEAR

----  -----------                                                 LINEAR

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                    LINEAR

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)           LINEAR

LINEAR

OUTPUT FILES                                                      LINEAR

------------                                                      LINEAR

UNIT  DESCRIPTION                                                 LINEAR

----  -----------                                                 LINEAR

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                 LINEAR

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)              LINEAR

LINEAR

SCRATCH FILES                                                     LINEAR

-------------                                                     LINEAR

UNIT  DESCRIPTION                                                 LINEAR

----  -----------                                                 LINEAR

12  SCRATCH FILE (BINARY - 180000 WORDS/RECORD                  LINEAR

LINEAR

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)              LINEAR

----------------------------------------------------              LINEAR

UNIT  FILE NAME                                                   LINEAR

----  ----------                                                  LINEAR

2   LINEAR.INP                                                  LINEAR

3   LINEAR.LST                                                  LINEAR

10   ENDFB.IN                                                    LINEAR

11   ENDFB.OUT                                                   LINEAR

12   (SCRATCH)                                                   LINEAR

LINEAR

LINEAR

INPUT PARAMETERS                                                  LINEAR

----------------                                                  LINEAR

FOR VERSIONS EARLIER THAN 90-1 THIS PROGRAM ONLY ALLOWED THE USER LINEAR

TO SPECIFY BY INPUT PARAMETERS WHICH MATERIALS (MAT) TO PROCESS.  LINEAR

FOR EACH REQUESTED MATERIAL NEUTRON INTERACTION CROSS SECTIONS    LINEAR

(MF=3) WOULD BE LINEARIZED AND THE REMAINDER OF THE MATERIAL      LINEAR

WOULD BE COPIED.                                                  LINEAR

LINEAR

FOR VERSIONS 90-1 AND LATER THIS PROGRAM WILL ALLOW THE USER TO   LINEAR

TO SPECIFY BY INPUT PARAMETERS EXACTLY WHAT SECTIONS OF DATA      LINEAR

TO PROCESS. FOR EACH SECTION OF DATA, SPECIFIED BY MAT, MF, MT    LINEAR

RANGES, SECTIONS OF MF=3, 23 AND 27 WILL BE LINEARIZED AND ALL    LINEAR

OTHER REQUESTED SECTIONS WILL BE COPIED. ALL SECTIONS WHICH ARE   LINEAR

NOT EXPLICITLY REQUESTED WILL BE SKIPPED AND WILL NOT APPEAR ON   LINEAR

ENDF/B FILE OUTPUT BY THIS PROGRAM.                               LINEAR

LINEAR

WITH THIS NEW PROCEDURE YOU CAN MINIMIZE THE SIZE OF THE ENDF/B   LINEAR

FILE OUTPUT BY THIS PROGRAM, E.G., IF YOU ONLY WANT NEUTRON       LINEAR

CROSS SECTIONS FOR SUBSEQUENT PROCESSING YOU NEED ONLY REQUEST    LINEAR

ONLY MF=3 DATA.                                                   LINEAR

LINEAR

HOWEVER, YOU MUST UNDERSTAND THAT ONLY THOSE SECTIONS WHICH YOU   LINEAR

EXPLICITLY REQUEST WILL APPEAR ON THE ENDF/B FILE OUTPUT BY       LINEAR

THIS PROGRAM. FOR EXAMPLE, IF YOU WISH TO DOCUMENT EXACTLY        LINEAR

HOW YOU LINEARIZED THE DATA BY INCLUDING COMMENTS IN MF=1, MT=451 LINEAR

THEN YOU MUST EXPLICITLY REQUEST THAT MF=1, MT=451 BE PROCESSED   LINEAR

FOR EACH MATERIAL THAT YOU REQUEST. SIMILAR IF YOU WANT THE       LINEAR

ENTIRE EVALUATION YOU MUST REQUEST ALL MF AND MT TO BE OUTPUT.    LINEAR

LINEAR

LINE  COLS.  DESCRIPTION                                          LINEAR

----  -----  -----------                                          LINEAR

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                     LINEAR

12-22  MONITOR MODE SELECTOR                                LINEAR

= 0 - NORMAL OPERATION                               LINEAR

= 1 - MONITOR PROGRESS OF LINEARIZING OF THE DATA.   LINEAR

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO  LINEAR

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF LINEAR

POINTS ON SCRATCH AND THE LOWER AND UPPER      LINEAR

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE  LINEAR

USED IN ORDER TO MONITOR THE EXECUTION SPEED   LINEAR

OF LONG RUNNING JOBS).                         LINEAR

23-33  MINIMUM CROSS SECTION OF INTEREST (BARNS).           LINEAR

(IF 0.0 OR LESS IS INPUT THE PROGRAM WILL            LINEAR

USE 1.0E-10). ENERGY INTERVALS WILL NOT BE           LINEAR

SUB-DIVIDED IF THE ABSOLUTE VALUE OF THE CROSS       LINEAR

SECTION WITHIN THE INTERVAL IS LESS THAN THIS VALUE. LINEAR

AN EXCEPTION TO THIS RULE IS NEAR THRESHOLDS ENERGY  LINEAR

INTERVALS WILL BE SUB-DIVIDED UNTIL CONVERGENCE      LINEAR

REGARDLESS OF THE MAGNITUDE OF THE CROSS SECTION.    LINEAR

34-44  KEEP ORIGINAL EVALUATED DATA POINTS.                 LINEAR

= 0 - NO.                                            LINEAR

= 1 - YES - ADDITIONAL POINTS MAY BE ADDED IN ORDER  LINEAR

TO LINEARIZE DATA, BUT ALL ORIGINAL      LINEAR

DATA POINTS WILL BE INCLUDED IN THE      LINEAR

RESULTS.                                 LINEAR

2   1-72  ENDF/B INPUT DATA FILENAME                           LINEAR

(STANDARD OPTION = ENDFB.IN)                         LINEAR

3   1-72  ENDF/B OUTPUT DATA FILENAME                          LINEAR

(STANDARD OPTION = ENDFB.OUT)                        LINEAR

4-N   1- 6  LOWER MAT OR ZA LIMIT                                LINEAR

7- 8  LOWER MF LIMIT                                       LINEAR

9-11  LOWER MT LIMIT                                       LINEAR

12-17  UPPER MAT OR ZA LIMIT                                LINEAR

18-19  UPPER MF LIMIT                                       LINEAR

20-22  UPPER MT LIMIT                                       LINEAR

UP TO 100 RANGES MAY BE SPECIFIED, ONLY ONE RANGE    LINEAR

PER LINE. THE LIST OF RANGES IS TERMINATED BY A      LINEAR

BLANK LINE. IF THE UPPER MAT LIMIT OF ANY REQUEST    LINEAR

IS LESS THAN THE LOW LIMIT IT WILL BE SET EQUAL TO   LINEAR

THE LOWER LIMIT. IF THE UPPER LIMIT IS STILL ZERO    LINEAR

IT WILL BE SET EQUAL TO 999999. IF THE UPPER MF OR   LINEAR

MT LIMIT IS ZERO IT WILL BE SET TO 99 OR 999         LINEAR

RESPECTIVELY.                                        LINEAR

VARY  1-11  ENERGY FOR ERROR LAW                                 LINEAR

12-22  ALLOWABLE FRACTIONAL ERROR FOR ERROR LAW.            LINEAR

THE ACCEPTABLE LINEARIZING ERROR MAY BE SPECIFIED TO LINEAR

BE EITHER ENERGY INDEPENDENT (DEFINED BY A SINGLE    LINEAR

ERROR), OR ENERGY DEPENDENT (DEFINED BY UP TO 20     LINEAR

ENERGY, ERROR PAIRS). FOR THE ENERGY DEPENDENT CASE  LINEAR

LINEAR INTERPOLATION WILL BE USED TO DEFINE THE ERRORLINEAR

AT ENERGIES BETWEEN THOSE AT WHICH IT IS TABULATED.  LINEAR

IN ALL CASES THE ERROR LAW IS TERMINATED BY A BLANK  LINEAR

LINE. IF ONLY ONE ENERGY, ERROR PAIR IS GIVEN THE    LINEAR

THE LAW WILL BE CONSIDERED TO BE ENERGY INDEPENDENT. LINEAR

IF MORE THAN ONE PAIR IS GIVEN IT WILL BE CONSIDERED LINEAR

TO BE ENERGY DEPENDENT (NOTE, ENERGY INDEPENDENT     LINEAR

FORM WILL RUN FASTER THAN THE EQUIVALENT ENERGY      LINEAR

DEPENDENT FORM). FOR AN ENERGY DEPENDENT ERROR LAW   LINEAR

ALL ENERGIES MUST BE ASCENDING ENERGY ORDER. FOR     LINEAR

CONVERGENCE OF THE LINEARIZING ALGORITHM ALL ERRORS  LINEAR

MUST BE POSITIVE. IF AN ALLOWABLE ERROR IS NOT       LINEAR

POSITIVE IT WILL BE SET EQUAL TO THE STANDARD OPTION LINEAR

(CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT).    LINEAR

IF THE FIRST ERROR LINE IS BLANK IT WILL TERMINATE   LINEAR

THE ERROR LAW AND THE ERROR WILL BE TREATED AS       LINEAR

ENERGY INDEPENDENT, EQUAL TO THE STANDARD OPTION     LINEAR

(CURRENTLY 0.1 PER-CENT). (SEE EXAMPLE INPUT 4).     LINEAR

LINEAR

EXAMPLE INPUT NO. 1                                               LINEAR

-------------------                                               LINEAR

RETRIEVE DATA BY ZA IN ORDER TO FIND ALL URANIUM ISOTOPES AND     LINEAR

THORIUM 232. RETRIEVE ALL NEUTRON INTERACTION CROSS SECTIONS      LINEAR

(MF=3). ALL ENERGY INTERVALS IN WHICH THE CROSS SECTION IS        LINEAR

AT LEAST 1 MICRO-BARN (1.0E-06 BARNS) WILL BE SUBDIVIDED.         LINEAR

BACKWARD THINNING WILL BE PERFORMED. FROM 0 TO 100 EV LINEARIZE   LINEAR

TO WITHIN 0.1 PER-CENT ACCURACY. FROM 100 EV TO 1 KEV VARY        LINEAR

ACCURACY BETWEEN 0.1 AND 1.0 PER-CENT. ABOVE 1 KEV USE 1          LINEAR

PER-CENT ACCURACY.                                                LINEAR

LINEAR

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                        LINEAR

LINEAR

IN THIS CASE THE FOLLOWING 11 INPUT LINES ARE REQUIRED            LINEAR

LINEAR

1          0 1.00000- 6          0                           LINEAR

ENDFB.IN                                                              LINEAR

ENDFB.OUT                                                             LINEAR

92000 3  0 92999 3999                                                 LINEAR

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999)LINEAR

(END OF REQUEST LIST)                        LINEAR

0.00000+ 0 1.00000-03                                                 LINEAR

1.00000+ 2 1.00000-03                                                 LINEAR

1.00000+ 3 1.00000-02                                                 LINEAR

1.00000+ 9 1.00000-02                                                 LINEAR

(END OF ERROR LAW)                           LINEAR

LINEAR

EXAMPLE INPUT NO. 2                                               LINEAR

-------------------                                               LINEAR

SAME AS THE ABOVE CASE, EXCEPT LINEARIZE ALL DATA TO WITHIN THE   LINEAR

STANDARD ACCURACY (CURRENTLY 0.1 PER-CENT). IN ORDER TO USE THE   LINEAR

STANDARD ACCURACY YOU NEED NOT SPECIFY ANY ERROR LAW AT ALL. IN   LINEAR

THIS CASE INCLUDE THE HOLLERITH SECTION, MF=1, MT=451, FOR EACH   LINEAR

MATERIAL.                                                         LINEAR

LINEAR

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL    LINEAR

THEN USE STANDARD FILENAMES.                                      LINEAR

LINEAR

IN THIS CASE THE FOLLOWING 9 INPUT LINES ARE REQUIRED             LINEAR

LINEAR

1          0 1.00000- 6          0                           LINEAR

(USE DEFAULT FILENAME = ENDFB.IN)            LINEAR

(USE DEFAULT FILENAME = ENDFB.OUT)           LINEAR

92000 1451 92999 1451                                                 LINEAR

92000 3  0 92999 3999                                                 LINEAR

90232 1451     0 1451                                                 LINEAR

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999)LINEAR

(END OF REQUEST LIST)                        LINEAR

(0.1 PER-CENT ERROR, END OF ERROR LAW)       LINEAR

LINEAR

EXAMPLE INPUT NO. 3                                               LINEAR

-------------------                                               LINEAR

LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO WITHIN AN ACCURACY   LINEAR

OF 0.5 PER-CENT (0.005 AS A FRACTION). IN THIS CASE YOU NEED NOT  LINEAR

SPECIFY THE MAT, MF, MT RANGES.                                   LINEAR

LINEAR

READ THE ENDF/B DATA FROM \ENDFB6\ZA092238 AND WRITE THE ENDF/B   LINEAR

DATA TO \ENDFB6\LINEAR\ZA092238.                                  LINEAR

LINEAR

IN THIS CASE THE FOLLOWING 6 INPUT LINES ARE REQUIRED             LINEAR

LINEAR

(MAT, 1.0E-10 BARNS, THIN)LINEAR

\ENDFB6\ZA092238                                                      LINEAR

\ENDFB6\LINEAR\ZA092238                                               LINEAR

(RETRIEVE ALL DATA, END REQUEST LIST)        LINEAR

5.00000-03                                                 LINEAR

(END OF ERROR LAW)                           LINEAR

LINEAR

NOTE THAT IN THIS CASE IF THE INPUT HAD SPECIFIED AN EQUIVALENT   LINEAR

ENERGY DEPENDENT ERROR LAW BY GIVING A NUMBER OF ENERGY POINTS    LINEAR

AT EACH OF WHICH THE ERROR IS 0.5 PER-CENT THE PROGRAM WOULD TAKE LINEAR

LONGER TO RUN (I.E., ONLY USE AN ENERGY DEPENDENT ERROR LAW WHEN  LINEAR

IT IS NECESSARY).                                                 LINEAR

LINEAR

EXAMPLE INPUT NO. 4                                               LINEAR

-------------------                                               LINEAR

IN ORDER TO LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO THE      LINEAR

STANDARD OPTION OF 0.1 PER-CENT IT IS ADEQUATE TO INPUT A SET     LINEAR

OF COMPLETELY BLANK LINES WHICH WILL AUTOMATICALLY INVOKE ALL     LINEAR

OF THE STANDARD OPTIONS.                                          LINEAR

LINEAR

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL    LINEAR

THEN USE STANDARD FILENAMES.                                      LINEAR

LINEAR

IN THIS CASE THE FOLLOWING THREE INPUT LINES ARE REQUIRED         LINEAR

LINEAR

(MAT, 1.0E-10 BARNS, THIN)LINEAR

(USE DEFAULT FILENAME = ENDFB.IN)            LINEAR

(USE DEFAULT FILENAME = ENDFB.OUT)           LINEAR

(RETRIEVE ALL DATA, END REQUEST LIST)        LINEAR

(0.1 PER-CENT ERROR, END OF ERROR LAW)       LINEAR

LINEAR

=======================================================================LINEAR