=======================================================================VIRGIN

VIRGIN

PROGRAM VIRGIN                                                    VIRGIN

VERSION 76-1 (NOVEMBER 1976)                                      VIRGIN

VERSION 84-1 (JUNE 1984)   *DOUBLE PRECISION ENERGY               VIRGIN

VERSION 86-1 (JANUARY 1986)*FORTRAN-77/H VERSION                  VIRGIN

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O    VIRGIN

FILE NAMES (SEE, SUBROUTINE FILEIO    VIRGIN

FOR DETAILS).                         VIRGIN

*IMPROVED BASED ON USER COMMENTS.      VIRGIN

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO    VIRGIN

INSURE PROGRAM WILL NOT DO ANYTHING   VIRGIN

CRAZY.                                VIRGIN

*UPDATED TO USE NEW PROGRAM CONVERT    VIRGIN

KEYWORDS.                             VIRGIN

*ADDED LIVERMORE CIVIC COMPILER        VIRGIN

CONVENTIONS.                          VIRGIN

VERSION 92-1 (JANUARY 1992)*COMPLETE RE-WRITE                     VIRGIN

*OUTPUT IN PLOTTAB FORMAT              VIRGIN

*UP TO 2000 THICKNESSES                VIRGIN

*INCREASED INCORE PAGE SIZE TO 6000    VIRGIN

CROSS SECTION POINTS                  VIRGIN

*ADDED PHOTON CALCULATIONS             VIRGIN

*ADDED BLACKBODY SPECTRUM              VIRGIN

*ADDED MULTIPLE LAYERS                 VIRGIN

*ADDED SPATIALLY DEPENDENT DENSITY     VIRGIN

*ADDED FORTRAN SAVE OPTION             VIRGIN

*COMPLETELY CONSISTENT I/O ROUTINES -  VIRGIN

TO MINIMIZE COMPUTER DEPENDENCE.      VIRGIN

VERSION 92-2 (MAY 1992)    *CORRECTED TO HANDLE MULTIGROUP CROSS  VIRGIN

SECTIONS AS INPUT IN ENDF/B FORMAT.   VIRGIN

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    VIRGIN

*IMPROVED COMPUTER INDEPENDENCE       VIRGIN

*ALL DOUBLE PRECISION                 VIRGIN

*ON SCREEN OUTPUT                     VIRGIN

*UNIFORM TREATMENT OF ENDF/B I/O      VIRGIN

*IMPROVED OUTPUT PRECISION            VIRGIN

*DEFINED SCRATCH FILE NAMES           VIRGIN

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      VIRGIN

POINT READ FOR MORE DIGITS           VIRGIN

*UPDATED TEST FOR ENDF/B FORMAT       VIRGIN

VERSION BASED ON RECENT FORMAT CHANGEVIRGIN

*GENERAL IMPROVEMENTS BASED ON        VIRGIN

USER FEEDBACK                        VIRGIN

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON        VIRGIN

USER FEEDBACK                        VIRGIN

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS            VIRGIN

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON             VIRGIN

*UP TO 2000 THICKNESSES               VIRGIN

*INCREASED INCORE PAGE SIZE TO 60,000 VIRGIN

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.      VIRGIN

*INCREASED INCORE PAGE SIZE TO        VIRGIN

240,000 FROM 60,000.                 VIRGIN

VERS. 2007-2 (DEC.  2007)   *72 CHARACTER FILE NAME.              VIRGIN

VERS. 2010-1 (Apr.  2010)   *General update based on user feedbackVIRGIN

*INCREASED INCORE PAGE SIZE TO        VIRGIN

600,000 FROM 240,000.                VIRGIN

VERS. 2012-1 (Aug.  2012)   *Added CODENAME                       VIRGIN

*32 and 64 bit Compatible             VIRGIN

*Added ERROR stop                     VIRGIN

VERS. 2015-1 (Jan.  2015)   *Extended OUT9.                       VIRGIN

*Replaced ALL 3 way IF Statements.    VIRGIN

*Generalized TART Group Structures.   VIRGIN

*Generalized SAND-II Group Structures.VIRGIN

*Extended SAND-II to 60, 150, 200 MeV.VIRGIN

VERS. 2015-2 (Apr.  2015)   *Changed ALL data to "D" instead of   VIRGIN

"E" to insure it is REAL*8 and avoid VIRGIN

Truncation ERRORS.                   VIRGIN

VERS. 2017-1 (May   2017)   *Added UKAEA 1102 Group Structure.    VIRGIN

*Increased points to 3,000,000        VIRGIN

*Increased groupd to 30,000           VIRGIN

*Updated based on user feedback       VIRGIN

*Defintion of built-in group structureVIRGIN

using SUBROUTINE GROPE is identical  VIRGIN

for GROUPIE and VIRGIN.              VIRGIN

*All floating point parameters changedVIRGIN

to character inout + IN9 conversion. VIRGIN

VERS. 2018-1 (Jan.  2018)   *Decreased PAGE size from 3,000,000   VIRGIN

to 1,500,000                         VIRGIN

*On-line output for ALL ENDERROR      VIRGIN

VERS. 2019-1 (June  2019)   *Additional Interpolation Law Tests   VIRGIN

*Checked Maximum Tabulated Energy to  VIRGIN

insure it is the same for all MTs -  VIRGIN

if not, print WARNING messages.      VIRGIN

VERS. 2020-1 (Feb.  2020)   *Identical to 2019-1.                 VIRGIN

VIRGIN

2015-2 Acknowledgment                                             VIRGIN

=====================                                             VIRGIN

I thank Andrej Trkov (NDS, IAEA) for finding the problem with     VIRGIN

the "E" formatted DATA (this effected both VIRGIN and GROUPIE).   VIRGIN

I also thank Andrej for overseeing the entire PREPRO project      VIRGIN

at IAEA, Vienna; he is part of a truly International team who     VIRGIN

worked together to produce PREPRO-2015-2, and to make it          VIRGIN

available Internationally on-line for FREE to ALL users.          VIRGIN

VIRGIN

OWNED, MAINTAINED AND DISTRIBUTED BY                              VIRGIN

------------------------------------                              VIRGIN

THE NUCLEAR DATA SECTION                                          VIRGIN

INTERNATIONAL ATOMIC ENERGY AGENCY                                VIRGIN

P.O. BOX 100                                                      VIRGIN

A-1400, VIENNA, AUSTRIA                                           VIRGIN

EUROPE                                                            VIRGIN

VIRGIN

ORIGINALLY WRITTEN BY                                             VIRGIN

------------------------------------                              VIRGIN

Dermott E. Cullen                                                 VIRGIN

VIRGIN

PRESENT CONTACT INFORMATION                                       VIRGIN

---------------------------                                       VIRGIN

Dermott E. Cullen                                                 VIRGIN

1466 Hudson Way                                                   VIRGIN

Livermore, CA 94550                                               VIRGIN

U.S.A.                                                            VIRGIN

Telephone  925-443-1911                                           VIRGIN

E. Mail    RedCullen1@Comcast.net                                 VIRGIN

Website    RedCullen1.net/HOMEPAGE.NEW                            VIRGIN

VIRGIN

PURPOSE                                                           VIRGIN

-------                                                           VIRGIN

THIS PROGRAM IS DESIGNED TO CALCULATE UNCOLLIDED (I.E. VIRGIN)    VIRGIN

FLUX AND REACTIONS DUE TO TRANSMISSION OF A MONODIRECTIONAL       VIRGIN

BEAM OF NEUTRONS THROUGH ANY THICKNESS OF MATERIAL. IN ORDER      VIRGIN

TO SIMULATE AN EXPERIMENTAL MEASUREMENT THE RESULTS ARE GIVEN     VIRGIN

AS INTEGRALS OVER ENERGY TALLY GROUPS (AS OPPOSED TO POINTWISE    VIRGIN

IN ENERGY). BY TAKING THE RATIO OF REACTIONS TO FLUX IN EACH      VIRGIN

GROUP AN EQUIVALENT SPATIALLY DEPENDENT GROUP AVERAGED CROSS      VIRGIN

SECTION IS CALCULATED BY THE PROGRAM.                             VIRGIN

VIRGIN

EVALUATED DATA                                                    VIRGIN

--------------                                                    VIRGIN

THE EVALUATED DATA MUST BE IN THE ENDF/B FORMAT. HOWEVER IT       VIRGIN

MUST BE LINEAR-LINEAR INTERPOLABLE IN ENERGY-CROSS SECTION        VIRGIN

BETWEEN TABULATED POINTS. SINCE ONLY CROSS SECTIONS (FILE 3 OR 23)VIRGIN

ARE USED, THIS PROGRAM WILL WORK ON ANY VERSION OF ENDF/B         VIRGIN

(I.E. ENDF/B-I, II, III, IV, V OR VI).                            VIRGIN

VIRGIN

RELATED COMPUTER CODES                                            VIRGIN

----------------------                                            VIRGIN

IN ORDER TO CONVERT ENDF/B DATA TO THE FORM REQUIRED BY THIS CODE VIRGIN

THE FOLLOWING COMPUTER CODES MAY BE USED,                         VIRGIN

VIRGIN

LINEAR  - CONVERT FROM GENERAL ENDF/B INTERPOLATION TO LINEAR-    VIRGIN

LINEAR INTERPOLATION.                                   VIRGIN

RECENT  - ADD THE RESONANCE CONTRIBUTION TO TABULATED BACKGROUND  VIRGIN

CROSS SECTIONS TO OBTAIN LINEAR-LINEAR INTERPOLABLE     VIRGIN

RESULTS.                                                VIRGIN

SIGMA1  - DOPPLER BROADEN CROSS SECTION TO OBTAIN LINEAR-LINEAR   VIRGIN

INTERPOLABLE RESULTS.                                   VIRGIN

MIXER   - MIX INDIVIDUAL MATERIALS TOGETHER TO DEFINE COMPOSITE   VIRGIN

MIXTURES, E.G., COMBINE MATERIALS TO DEFINE STAINLESS   VIRGIN

STELL.                                                  VIRGIN

VIRGIN

IN ORDER TO PLOT THE OUTPUT RESULTS OF THIS CODE USE PROGRAM      VIRGIN

PLOTTAB.                                                          VIRGIN

VIRGIN

COPIES OF ANY OR ALL OF THESE CODES MAY BE OBTAINED FROM D.E.     VIRGIN

CULLEN AT THE ABOVE ADDRESS.                                      VIRGIN

VIRGIN

OUTPUT FORMAT                                                     VIRGIN

-------------                                                     VIRGIN

FOR ALL VERSIONS OF THIS PROGRAM PRIOR TO VERSION 92-1 OUTPUT WAS VIRGIN

IN TABULAR FORM.                                                  VIRGIN

VIRGIN

FOR VERSION 92-1 AND LATER VERSIONS OF THIS CODE ALL OUTPUT IS IN VIRGIN

THE PROGRAM PLOTTAB FORMAT TO ALLOW RESULTS TO BE EASILY PLOTTED. VIRGIN

FOR A COPY OF PROGRAM PLOTTAB CONTACT D.E. CULLEN AT THE ABOVE    VIRGIN

ADDRESS.                                                          VIRGIN

VIRGIN

TALLY GROUPS                                                      VIRGIN

------------                                                      VIRGIN

THE TALLY GROUP STRUCTURE MAY BE ANY SET OF MONONTONICALLY        VIRGIN

INCREASING ENERGY BOUNDARIES. THERE MAY BE UP TO 2000 TALLY       VIRGIN

GROUPS. BY USING THE INPUT PARAMETERS THE USER MAY SPECIFY ANY    VIRGIN

ARBITRARY TALLY GROUP STRUCTURE OR SELECT ONE OF THE FOLLOWING    VIRGIN

BUILT-IN GROUP STRUCTURES.                                        VIRGIN

VIRGIN

(0) TART 175 GROUPS                                               VIRGIN

(1) ORNL  50 GROUPS                                               VIRGIN

(2) ORNL 126 GROUPS                                               VIRGIN

(3) ORNL 171 GROUPS                                               VIRGIN

(4) SAND-II 620 GROUPS - 1.0D-4 eV UP TO 18 MEV                   VIRGIN

(5) SAND-II 640 GROUPS - 1.0D-4 eV UP TO 20 MEV                   VIRGIN

(6) WIMS 69 GROUPS                                                VIRGIN

(7) GAM-I 68 GROUPS                                               VIRGIN

(8) GAM-II 99 GROUPS                                              VIRGIN

(9) MUFT 54 GROUPS                                                VIRGIN

(10) ABBN 28 GROUPS                                                VIRGIN

(11) TART 616 GROUPS TO  20 MeV                                    VIRGIN

(12) TART 700 GROUPS To   1 GeV                                    VIRGIN

(13) SAND-II 665 GROUPS - 1.0D-5 eV UP TO 18 MEV                   VIRGIN

(14) SAND-II 685 GROUPS - 1.0D-5 eV UP TO 20 MEV                   VIRGIN

(15) TART 666 GROUPS TO 200 MeV                                    VIRGIN

(16) SAND-II 725 GROUPS - 1.0D-5 eV UP TO  60 MEV                  VIRGIN

(17) SAND-II 755 GROUPS - 1.0D-5 eV UP TO 150 MEV                  VIRGIN

(18) SAND-II 765 GROUPS - 1.0D-5 eV UP TO 200 MEV                  VIRGIN

(19) UKAEA  1102 GROUPS - 1.0D-5 eV UP TO 1 GeV                    VIRGIN

VIRGIN

INCIDENT SPECTRUM                                                 VIRGIN

-----------------                                                 VIRGIN

THE INCIDENT SPECTRUM MAY BE ANY TABULATED FUNCTION THAT IS       VIRGIN

GIVEN BY A SET OF POINTS THAT IS MONOTONICALLY INCREASING IN      VIRGIN

ENERGY AND LINEAR-LINEAR INTERPOLABLE IN ENERGY-SPECTRUM          VIRGIN

BETWEEN TABULATED POINTS. THERE IS NO LIMIT TO THE NUMBER OF      VIRGIN

POINTS USED TO DESCRIBE THE SPECTRUM. THERE ARE FIVE BUILT-IN     VIRGIN

OPTIONS FOR THE SPECTRUM.                                         VIRGIN

VIRGIN

(1) CONSTANT...ENERGY INDEPENDENT (INPUT 0)                       VIRGIN

(2) 1/E (INPUT 1)                                                 VIRGIN

(3) BLACKBODY - PHOTON SPECTRUM                                   VIRGIN

(4) BLACKBODY - ENERGY SPECTRUM (E TIMES THE PHOTON SPECTRUM)     VIRGIN

(5) TRANSMITTED SPECTRUM FROM PREVIOUS CASE                       VIRGIN

VIRGIN

NORMALIZATION OF SPECTRUM                                         VIRGIN

-------------------------                                         VIRGIN

ANY INCIDENT SPECTRUM, EITHER READ AS INPUT OR ONE OF THE         VIRGIN

BUILT-IN SPECTRA, WILL BE NORMALIZED TO UNITY WHEN INTEGRATED     VIRGIN

OVER THEIR ENTIRE ENERGY RANGE.                                   VIRGIN

VIRGIN

TRANSMITTED SPECTRA WILL NOT BE RE-NORMALIZED, SINCE IT ALREADY   VIRGIN

INCLUDES THE NORMALIZATION OF THE INCIDENT SPECTRUM.              VIRGIN

VIRGIN

NOTE, INCIDENT SPECTRA IS NORMALIZED TO UNITY OVER THEIR ENTIRE   VIRGIN

ENERGY RANGE - NOT OVER THE ENERGY RANGE OF THE GROUPS. IF THE    VIRGIN

ENERGY RANGE OF THE GROUPS IS LESS THAN THAT OF THE SPECTRUM      VIRGIN

ONLY THAT PORTION OF THE SPECTRUM WILL BE USED AND THIS WILL      VIRGIN

NOT BE RE-NORMALIZED TO UNITY.                                    VIRGIN

VIRGIN

COMPOSITION OF A LAYER                                            VIRGIN

----------------------                                            VIRGIN

YOU MAY RUN PROBLEMS INVOLVING                                    VIRGIN

1) A LAYER OF UNIFORM DENSITY - DENSITY FOR ATTENUATION IS THAT   VIRGIN

OF THE TOTAL. DENSITY FOR REACTIONS IS THAT OF THE REACTION.   VIRGIN

2) A LAYER OF UNIFORM DENSITY - DENSITY IS THE SUM OF THE TOTAL   VIRGIN

AND REACTION DENSITIES - THE SUM OF THE CROSS SECTIONS IS      VIRGIN

USED FOR ATTENUATION AND REACTIONS.                            VIRGIN

3) A LAYER OF VARYING DENSITY BASED ON A UNIFORM TOTAL DENSITY    VIRGIN

PLUS A VARIATION BETWEEN 0 AND A MAXIMUM BASED ON THE          VIRGIN

REACTION DENSITY - 0 AT 0 THICKNESS AND MAXIMUM AT MAXIMUM     VIRGIN

THICKNESS. IN THIS CASE THE AVERAGE REACTION DENSITY IS EQUAL  VIRGIN

TO THE INPUT REACTION DENSITY. THE VARIATION IN REACTION       VIRGIN

DENSITY CAN BE LINEAR, SQUARE OR CUBIC.                        VIRGIN

4) A LAYER OF VARYING DENSITY BASED ON A TOTAL DENSITY WHICH      VIRGIN

VARYING FROM MAXIMUM AT 0 THICKNESS TO 0 AT MAXIMUM THICKNESS  VIRGIN

PLUS A REACTION DENSITY WHICH VARIES FROM 0 AT 0 THICKNESS     VIRGIN

TO MAXIMUM AT MAXIMUM THICKNESS. IN THIS CASE THE AVERAGE      VIRGIN

DENSITY OF THE TOTAL AND REACTION WILL BOTH BE EQUAL TO THE    VIRGIN

INPUT TOTAL AND REACTION DENSITIES. THE VARIATION IN TOTAL     VIRGIN

AND REACTION DENSITY CAN BE LINEAR, SQUARE OR CUBIC.           VIRGIN

VIRGIN

IN THE FIRST CASE THE TWO REQUESTED CROSS SECTIONS ARE CONSIDERED VIRGIN

TO BE INDEPENDENT - THE TOTAL CROSS SECTION IS USED TO CALCULATE  VIRGIN

ATTENUATION AND THE REACTION CROSS SECTION IS USED TO CALCULATE   VIRGIN

REACTIONS, E.G., TRANSMISSION THROUGH NATURAL URANIUM (THE TOTAL  VIRGIN

CROSS SECTION SHOULD BE THAT OF NATURAL URANIUM) AND REACTIONS    VIRGIN

IN A U-235 DETECTOR (THE REACTION CROSS SECTION SHOULD BE THAT OF VIRGIN

U-235).                                                           VIRGIN

VIRGIN

IN THE OTHER THREE CASES THE TWO REQUESTED CROSS SECTIONS ARE     VIRGIN

TREATED AS TWO CONSTITUENTS OF A MIXTURE OF TWO MATERIALS AND     VIRGIN

THE TWO CROSS SECTIONS ARE USED BOTH TO DEFINE A TOTAL CROSS      VIRGIN

SECTION FOR ATTENUATION AND A REACTION CROSS SECTION TO DEFINE    VIRGIN

REACTIONS. IN THESE CASES THE MIXTURE WILL VARY CONTINUOUSLY,     VIRGIN

E.G., IN CASE 4) HALF WAY THROUGH THE LAYER THE COMPOSITION WILL  VIRGIN

BE 1/2 THE MATERIAL DEFINED BY THE TOTAL AND 1/2 THE MATERIAL     VIRGIN

BASED ON THE REACTION. IN THESE CASES RATHER THAN THINKING OF     VIRGIN

THE TWO CROSS SECTIONS AS A TOTAL AND REACTION CROSS SECTION,     VIRGIN

IT IS BETTER TO THINK OF THEM AS THE TOTAL CROSS SECTIONS FOR     VIRGIN

MATERIALS A AND B AND THE CALCULATED REACTIONS WILL BE BASED      VIRGIN

ON THESE TWO TOTAL CROSS SECTIONS.                                VIRGIN

VIRGIN

MULTIPLE LAYERS                                                   VIRGIN

---------------                                                   VIRGIN

THIS CODE MAY BE USED TO RUN EITHER A NUMBER OF INDEPENDENT       VIRGIN

PROBLEMS, EACH INVOLVING TRANSMISSION THROUGH A SINGLE LAYER OF   VIRGIN

MATERIAL, OR TRANSMISSION THROUGH A NUMBER OF LAYERS ONE AFTER    VIRGIN

THE OTHER.                                                        VIRGIN

VIRGIN

IN THE CASE OF MULTIPLE LAYERS, ONE LAYER AFTER ANOTHER, THE      VIRGIN

TRANSMITTED ENERGY DEPENDENT SPECTRUM IS USED AS THE INCIDENT     VIRGIN

SPECTRUM FOR THE NEXT LAYER. THERE IS NO LIMIT TO THE NUMBER      VIRGIN

OF LAYERS WHICH MAY BE USED - EACH LAYER IS TREATED AS A          VIRGIN

COMPLETELY INDEPENDENT PROBLEM WITH A DEFINED INCIDENT SOURCE,    VIRGIN

AND AS SUCH THE CYCLE OF TRANSMISSION THROUGH EACH LAYER AND      VIRGIN

USING THE TRANSMITTED SPECTRUM AS THE INCIDENT SPECTRUM FOR THE   VIRGIN

NEXT LAYER MAY BE REPEATED ANY NUMBER OF TIMES.                   VIRGIN

VIRGIN

REMEMBER - THE INCIDENT SPECTRUM IS ASSUMED TO BE LINEARLY        VIRGIN

INTERPOLABLE IN ENERGY AND SPECTRUM BETWEEN THE ENERGIES AT       VIRGIN

WHICH IT IS TABULATED. THE TRANSMITTED SPECTRUM WILL BE TABULATED VIRGIN

AT THE UNION OF ALL ENERGIES OF THE INCIDENT SPECTRUM AND CROSS   VIRGIN

SECTIONS (TOTAL AND REACTION). IN ORDER TO INSURE THE ACCURACY    VIRGIN

OF THE RESULT WHEN PERFORMING MULTIPLE LAYER CALCULATION BE SURE  VIRGIN

TO SPECIFY THE INCIDENT SPECTRUM ON THE FIRST LAYER TO SUFFICIENT VIRGIN

DETAIL (ENOUGH ENERGY POINTS CLOSELY SPACED TOGETHER) IN ORDER TO VIRGIN

ALLOW THE TRANSMITTED SPECTRUM TO BE ACCURATELY REPRESENTED BY    VIRGIN

LINEAR INTERPOLATION BETWEEN SUCCESSIVE ENERGY POINTS - THERE IS  VIRGIN

NO LIMIT TO THE NUMBER OF POINTS ALLOWED IN THE INCIDENT SPECTRUM,VIRGIN

SO IF YOU ARE IN DOUBT, SIMPLY USE MORE ENERGY POINTS TO SPECIFY  VIRGIN

THE INCIDENT SPECTRUM.                                            VIRGIN

VIRGIN

RESULT OUTPUT UNITS                                               VIRGIN

-------------------                                               VIRGIN

FLUX      = EXACTLY AS CALCULATED                                 VIRGIN

REACTIONS = 1/CM OR 1/GRAM                                        VIRGIN

AVERAGE   = 1/CM - MACROSCOPIC UNITS                              VIRGIN

CROSS                                                             VIRGIN

SECTION                                                           VIRGIN

VIRGIN

THICKNESS AND DENSITY                                             VIRGIN

---------------------                                             VIRGIN

THE UNCOLLIDED CALCULATION ONLY DEPENDS ON THE PRODUCT OF         VIRGIN

THICKNESS AND DENSITY (I.E. GRAMS PER CM SQUARED). THIS FACT      VIRGIN

MAY BE USED TO SIMPLIFY INPUT BY ALLOWING THE THICKNESS AND       VIRGIN

DENSITY TO BE GIVEN EITHER AS CM AND GRAMS/CC RESPECTIVELY        VIRGIN

OR ELSE TO GIVE THICKNESS IN GRAMS/(CM*CM) AND INPUT A            VIRGIN

DENSITY OF 1.0 - OR IN ANY OTHER CONVENIENT UNITS AS LONG AS      VIRGIN

THE PRODUCT OF THICKNESS AND DENSITY IS IN THE CORRECT GRAMS      VIRGIN

PER CENTIMETER SQUARED.                                           VIRGIN

VIRGIN

GRAMS/(CM*CM) ARE RELATED TO ATOMS/BARN THROUGH THE RELATIONSHIP  VIRGIN

VIRGIN

GRAMS/(CM*CM)=(ATOMS/BARN)*(GRAMS/MOLE)*(MOLE/ATOM)               VIRGIN

VIRGIN

OR...                                                             VIRGIN

VIRGIN

GRAMS/(CM*CM)=(ATOMS/BARN)*(ATOMIC WEIGHT)/0.602                  VIRGIN

VIRGIN

CROSS SECTIONS AT A SPACE POINT AND OPTICAL THICKNESS             VIRGIN

-----------------------------------------------------             VIRGIN

THIS PROGRAM ALLOWS LAYERS OF EITHER UNIFORM DENSITY OR           VIRGIN

CONTINUOUSLY VARYING DENSITY. THE DENSITY CAN BE ONE OF THE       VIRGIN

FOLLOWING FORMS,                                                  VIRGIN

1) C                   = UNIFORM DENSITY                          VIRGIN

2) C*2*(X/T)           = LINEAR VARIATION FROM 0 TO C             VIRGIN

3) C*(2-2*(X/T))       = LINEAR VARIATION FROM C TO 0             VIRGIN

4) C*3*(X/T)**2        = SQUARE VARIATION FROM 0 TO C             VIRGIN

5) C*(3-3*(X/T)**2)/2  = SQUARE VARIATION FROM C TO 0             VIRGIN

6) C*4*(X/T)**3        = CUBIC VARIATION FROM 0 TO C              VIRGIN

7) C*(4-4*(X/T)**3)/3  = CUBIC VARIATION FROM C TO 0              VIRGIN

VIRGIN

IN ORDER TO CALCULATE REACTIONS AT A POINT THE MICROSCOPIC        VIRGIN

REACTION CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES.  VIRGIN

VIRGIN

IN ORDER TO CALCULATE TRANSMISSION WE MUST DEFINE THE OPTICAL     VIRGIN

PATH LENGTH WHICH MAY BE DEFINED BY INTEGRATING EACH OF THE       VIRGIN

ABOVE DENSITY FORMS TO FIND,                                      VIRGIN

1) C*X                                                            VIRGIN

2) C*X*(X/T)                                                      VIRGIN

3) C*X*(2-(X/T))                                                  VIRGIN

4) C*X*(X/T)**2                                                   VIRGIN

5) C*X*(3-(X/T)**2)/2                                             VIRGIN

6) C*X*(X/T)**3                                                   VIRGIN

7) C*X*(4-(X/T)**3))/3                                            VIRGIN

VIRGIN

IN ORDER TO CALCULATE TRANSMISSION TO A POINT THE MICROSCOPIC     VIRGIN

TOTAL CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES      VIRGIN

TO DEFINE THE OPTICAL PATH LENGTH.                                VIRGIN

VIRGIN

THE VARIATION OF THE DENSITY THROUGH THE LAYER MAY BE DEFINED     VIRGIN

BY SETTING X = 0 OR X = T TO FIND,                                VIRGIN

X = 0      X = T                                               VIRGIN

-----      -----                                               VIRGIN

1) C          C                                                   VIRGIN

2) 0          2*C                                                 VIRGIN

3) 2*C        0                                                   VIRGIN

4) 0          3*C                                                 VIRGIN

5) 3*C/2      0                                                   VIRGIN

6) 0          4*C                                                 VIRGIN

7) 4*C/3      0                                                   VIRGIN

VIRGIN

THE OPTICAL PATH THROUGH A LAYER OF THICKNESS T MAY BE DEFINED    VIRGIN

FROM THE ABOVE EXPRESSIONS BY SETTING X=T TO FIND THAT IN ALL     VIRGIN

CASES THE ANSWER WILL BY C*T. THE CONSTANTS IN THE ABOVE          VIRGIN

EXPRESSIONS HAVE BEEN INTRODUCED IN ORDER TO FORCE THIS RESULT.   VIRGIN

WITH THESE FACTORS THE OPTICAL PATH LENGTH THROUGH THE LAYER      VIRGIN

WILL EXACTLY CORRESPOND TO AN AVERAGE DENSITY CORRESPONDING TO    VIRGIN

THAT INPUT FOR THE TOTAL AND/OR REACTION, I.E., C CORRESPONDS     VIRGIN

TO THE INPUT DENSITY.                                             VIRGIN

VIRGIN

NOTE - FOR THE SAME OPTICAL PATH LENGTHS THROUGH THE LAYER THE    VIRGIN

TRANSMISSION WILL BE EXACTLY THE SAME. HOWEVER, VARYING THE       VIRGIN

DENSITY WILL ALLOW YOU TO MODIFY THE REACTION RATES AT SPECIFIC   VIRGIN

DEPTHS INTO THE LAYER.                                            VIRGIN

VIRGIN

COMPUTATION OF INTEGRALS                                          VIRGIN

------------------------                                          VIRGIN

STARTING FROM TOTAL CROSS SECTIONS, REACTION CROSS SECTIONS AND   VIRGIN

A SOURCE SPECTRUM ALL OF WHICH ARE GIVEN IN TABULAR FORM WITH     VIRGIN

LINEAR INTERPOLATION BETWEEN TABULATED POINTS ALL REQUIRED        VIRGIN

INTEGRALS CAN BE DEFINED BY ANALYTICAL EXPRESSIONS INVOLVING      VIRGIN

NOTHING MORE COMPLICATED THAN EXPONENTIALS. THE INTEGRALS THAT    VIRGIN

MUST BE EVALUATED ARE OF THE FORM...                              VIRGIN

VIRGIN

FLUX                                                              VIRGIN

----                                                              VIRGIN

(INTEGRAL EK TO EK+1) (S(E)* EXP(-XCT(E)*Z)*DE)                   VIRGIN

VIRGIN

REACTIONS                                                         VIRGIN

---------                                                         VIRGIN

(INTEGRAL EK TO EK+1) (S(E)*XCR(E)*EXP(-XCT(E)*Z)*DE)             VIRGIN

VIRGIN

WHERE..                                                           VIRGIN

EK TO EK+1 = LONGEST ENERGY INTERVAL OVER WHICH S(E), XCT(E) AND  VIRGIN

XCR(E) ARE ALL LINEARLY INTERPOLABLE.                VIRGIN

S(E)       = ENERGY DEPENDENT WEIGHTING SPECTRUM                  VIRGIN

XCR(E)     = REACTION CROSS SECTION                               VIRGIN

XCT(E)     = OPTICAL PATH LENGTH (BASED ON TOTAL CROSS SECTION)   VIRGIN

Z          = MATERIAL THICKNESS                                   VIRGIN

VIRGIN

S(E), XCR(E) AND XCT(E) ARE ALL ASSUMED TO BE GIVEN IN TABULAR    VIRGIN

FORM WITH LINEAR INTERPOLATION USED BETWEEN TABULATED POINTS.     VIRGIN

IN OTHER WORDS BETWEEN TABULATED POINTS EACH OF THESE THREE IS    VIRGIN

DEFINED BY A FUNCTION OF THE FORM...                              VIRGIN

VIRGIN

F(E)=((E - EK)*FK+1 + (EK+1 - E)*FK)/(EK+1 - EK)                  VIRGIN

VIRGIN

EACH OF THESE THREE CAN BE CONVERTED TO NORMAL FORM BY THE        VIRGIN

CHANGE OF VARIABLES....                                           VIRGIN

VIRGIN

X=(E - 0.5*(EK+1 + EK))/(EK+1 - EK)                               VIRGIN

VIRGIN

IN WHICH CASE X WILL VARY FROM -1 (AT EK) TO +1 (AT EK+1) AND     VIRGIN

EACH FUNCTION REDUCES TO THE NORMAL FORM...                       VIRGIN

VIRGIN

F(X)=0.5*(FK*(1 - X) + FK+1*(1 + X))                              VIRGIN

=0.5*(FK+1 + FK) + 0.5*(FK+1 - FK)*X                          VIRGIN

VIRGIN

BY DEFINING THE AVERAGE VALUE AND 1/2 THE CHANGE ACROSS THE       VIRGIN

INTERVAL.                                                         VIRGIN

VIRGIN

AVF=0.5*(FK+1 + FK)                                               VIRGIN

DF= 0.5*(FK+1 - FK)                                               VIRGIN

DE= 0.5*(EK+1 - EK)                                               VIRGIN

VIRGIN

EACH OF THE THREE FUNCTIONS REDUCES TO THE SIMPLE FORM...         VIRGIN

VIRGIN

F(X)=AVF+DF*X                                                     VIRGIN

VIRGIN

AND THE TWO REQUIRED INTEGRALS REDUCE TO...                       VIRGIN

VIRGIN

FLUX                                                              VIRGIN

----                                                              VIRGIN

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                            VIRGIN

((AVS+DS*X)*EXP(-DXCT*Z*X)*DX)                                    VIRGIN

VIRGIN

REACTION                                                          VIRGIN

--------                                                          VIRGIN

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                            VIRGIN

((AVS*AVXCR+(AVS*DXCR+AVXCR*DS)*X+DS*DXCR*X*X)*EXP(-DXCT*Z*X)*DX) VIRGIN

VIRGIN

WHERE                                                             VIRGIN

VIRGIN

AVXCT   = AVERAGE VALUE OF THE TOTAL CROSS SECTION                VIRGIN

AVXCR   = AVERAGE VALUE OF THE REACTION CROSS SECTION             VIRGIN

AVS     = AVERAGE VALUE OF THE SOURCE                             VIRGIN

DXCT    = 1/2 THE CHANGE IN THE TOTAL CROSS SECTION               VIRGIN

DXCR    = 1/2 THE CHANGE IN THE REACTION CROSS SECTION            VIRGIN

DS      = 1/2 THE CHANGE IN THE SOURCE                            VIRGIN

DE      = 1/2 THE CHANGE IN THE ENERGY                            VIRGIN

VIRGIN

NOTE THAT IN THIS FORM THE ENERGY ONLY APPEARS IN FRONT OF THE    VIRGIN

INTEGRALS AND THE INTEGRALS ARE EXPRESSED ONLY IN TERMS OF THE    VIRGIN

TABULATED VALUES OF S(E), XCT(E) AND XCR(E). IN PARTICULAR NO     VIRGIN

DERIVATIVES ARE USED, SO THAT THERE ARE NO NUMERICAL INSTABILITY  VIRGIN

PROBLEMS IN THE VACINITY OF DISCONTINUITIES IN S(E), XCT(E) OR    VIRGIN

XCR(E). INDEED, SINCE (EK+1 - EK) APPEARS IN FRONT OF THE INTEGRALVIRGIN

POINTS OF DISCONTINUITY AUTOMATICALLY MAKE ZERO CONTRIBUTION TO   VIRGIN

THE INTEGRALS.                                                    VIRGIN

VIRGIN

THE REQUIRED INTEGRALS CAN BE EXPRESSED IN TERMS OF THE THREE     VIRGIN

INTEGRALS IN NORMAL FORM....                                      VIRGIN

VIRGIN

F(A,N) = (INTEGRAL -1 TO 1) (X**N*EXP(-A*X)*DX), N=0,1 AND 2.     VIRGIN

VIRGIN

THESE THREE INTEGRALS CAN BE EVALUATED TO FIND...                 VIRGIN

VIRGIN

N=0                                                               VIRGIN

---                                                               VIRGIN

F(A,0) = (EXP(A)-EXP(-A))/A                                       VIRGIN

VIRGIN

N=1                                                               VIRGIN

---                                                               VIRGIN

F(A,1) = ((1-A)*EXP(A)-(1+A)*EXP(-A))/(A*A)                       VIRGIN

VIRGIN

N=2                                                               VIRGIN

---                                                               VIRGIN

F(A,2) = ((2-2*A+A*A)*EXP(A)-(2+2*A+A*A)*EXP(-A))/(A*A*A)         VIRGIN

VIRGIN

HOWEVER THESE EXPRESSIONS ARE NUMERICALLY UNSTABLE FOR SMALL      VIRGIN

VALUES OF A. THEREFORE FOR SMALL A THE EXPONENTIAL IN THE         VIRGIN

INTEGRALS ARE EXPANDED IN A POWER SERIES...                       VIRGIN

VIRGIN

EXP(-AX)=1.0-(AX)+(AX)**2/2-(AX)**3/6+(AX)**4/24-........         VIRGIN

=(SUM K=0 TO INFINITY) (-AX)**K/(K FACTORIAL)             VIRGIN

VIRGIN

AND THE INTEGRAL REDUCES TO THE FORM....                          VIRGIN

VIRGIN

(SUM K=0 TO INFINITY) ((-A)**K/(K FACTORIAL)) *                   VIRGIN

(INTEGRAL -1 TO 1) (X**(N+K))*DX                                  VIRGIN

VIRGIN

WHICH CAN BE ANALYTICALLY EVAULATED TO FIND....                   VIRGIN

(K(N) = K FACTORIAL)                                              VIRGIN

VIRGIN

N=0                                                               VIRGIN

---                                                               VIRGIN

F(A,0) = 2*(1+(A**2)/K(3)+(A**4)/K(5)+(A**6)/K(7)+....            VIRGIN

VIRGIN

N=1                                                               VIRGIN

---                                                               VIRGIN

F(A,1) = -2*A*(2/K(3)+4*(A**2)/K(5)+6*(A**4)/K(7)+8*(A**6)/K(9)+..VIRGIN

VIRGIN

N=2                                                               VIRGIN

---                                                               VIRGIN

F(A,2) = 2*(2/K(3)+3*4*(A**2)/K(5)+5*6*(A**4)/K(7)+               VIRGIN

7*8*(A**6)/K(9)+....                                     VIRGIN

VIRGIN

THESE EXPANSIONS ARE USED WHEN THE ABSOLUTE VALUE OF A IS LESS    VIRGIN

THAN 0.1. BY TRUNCATING THE ABOVE SERIES BEFORE A**8 THE ERROR    VIRGIN

RELATIVE TO THE LEADING TERM OF THE SERIES WILL BE 10**(-10),     VIRGIN

YIELDING 10 DIGIT ACCURACY.                                       VIRGIN

VIRGIN

AFTER EVALUATING THE ABOVE FUNCTIONS, EITHER DIRECTLY OR BY USING VIRGIN

THE EXPANSION THE TWO REQUIRED INTEGRALS CAN BE WRITTEN AS...     VIRGIN

VIRGIN

FLUX                                                              VIRGIN

----                                                              VIRGIN

DE*EXP(-AVXCT*Z)*(AVS*F(A,0) + DS*F(A,1))                         VIRGIN

VIRGIN

REACTIONS                                                         VIRGIN

---------                                                         VIRGIN

DE*EXP(-AVXCT*Z)*                                                 VIRGIN

(AVS*AVXCR*F(A,0) + (AVS*DXCR+AVXCR*DS)*F(A,1) + DS*DXCR*F(A,2))  VIRGIN

VIRGIN

INPUT FILES                                                       VIRGIN

-----------                                                       VIRGIN

FILENAME  UNIT  DESCRIPTION                                       VIRGIN

--------  ----  -----------                                       VIRGIN

INPUT       2   INPUT LINES                                       VIRGIN

ENDFIN     10   EVALUATED DATA IN ENDF/B FORMAT                   VIRGIN

VIRGIN

OUTPUT FILES                                                      VIRGIN

------------                                                      VIRGIN

FILENAME  UNIT  DESCRIPTION                                       VIRGIN

--------  ----  -----------                                       VIRGIN

OUTPUT      3   OUTPUT LISTING                                    VIRGIN

VIRGIN

SCRATCH FILES                                                     VIRGIN

-------------                                                     VIRGIN

FILENAME  UNIT  DESCRIPTION                                       VIRGIN

--------  ----  -----------                                       VIRGIN

SCR1       12   REACTION, FLUX AND CROSS SECTION RESULTS (BCD)    VIRGIN

(SORTED AT END OF RUN AND OUTPUT SEPARATELY)      VIRGIN

SCR2       13   TALLY GROUP ENERGY BOUNDARIES (BINARY)            VIRGIN

SCR3       14   SOURCE SPECTRUM (BINARY)                          VIRGIN

SCR4       15   TOTAL CROSS SECTION (BINARY)                      VIRGIN

SCR5       16   REACTION CROSS SECTION (BINARY)                   VIRGIN

VIRGIN

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILEIO2)  VIRGIN

----------------------------------------------------------------  VIRGIN

UNIT  FILE NAME   FORMAT                                          VIRGIN

----  ----------  ------                                          VIRGIN

2   VIRGIN.INP   BCD                                            VIRGIN

3   VIRGIN.LST   BCD                                            VIRGIN

10   ENDFB.IN     BCD                                            VIRGIN

11-15  (SCRATCH)   BINARY                                          VIRGIN

16   PLOTTAB.CUR PLOTTAB OUTPUT FORMAT DATA                      VIRGIN

VIRGIN

INPUT LINES                                                       VIRGIN

-----------                                                       VIRGIN

ANY NUMBER OF CASES MAY BE RUN ONE AFTER THE OTHER. AFTER THE     VIRGIN

FIRST CASE HAS BEEN RUN THE FOLLOWING CASES MAY USE THE SAME      VIRGIN

THICKNESSES, GROUP STRUCTURE AND SPECTRUM AS THE PRECEDING CASE.  VIRGIN

IN ADDITION THE TRANSMITTED SPECTRUM FROM ONE CASE MAY BE USED    VIRGIN

AS THE INCIDENT SPECTRUM IN THE NEXT CASE, TO ALLOW MULTIPLE      VIRGIN

LAYERS OF DIFFERENT MATERIALS.                                    VIRGIN

VIRGIN

LINE  COLS.  FORMAT  DESCRIPTION                                  VIRGIN

----  -----  ------  ----------                                   VIRGIN

1   1-60   ENDF/B INPUT DATA FILENAME                          VIRGIN

(STANDARD OPTION = ENDFB.IN)                      VIRGIN

VIRGIN

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL    VIRGIN

THEN USE STANDARD FILENAMES.                                      VIRGIN

VIRGIN

2-3   1-72   18A4   TWO LINE TITLE DESCRIBING PROBLEM            VIRGIN

4   1- 6    I6    ZA (1000*Z+A) OF TARGET FOR TOTAL            VIRGIN

7-11    I5    MT OF TOTAL                                  VIRGIN

12-22   E11.4  DENSITY FOR TOTAL                            VIRGIN

23-28    I6    ZA (1000*Z+A) OF TARGET FOR REACTION         VIRGIN

29-33    I5    MT OF REACTION                               VIRGIN

= 0 - NO REACTION CALCULATION (ONLY FLUX).   VIRGIN

= GREATER THAN 0 - CALCULATE REACTIONS.      VIRGIN

34-44   E11.4  DENSITY FOR REACTION                         VIRGIN

45-50    I6    NUMBER OF TARGET THICKNESSES                 VIRGIN

= GREATER THAN 0 = READ FROM INPUT           VIRGIN

(1 TO 2000 ALLOWED)                        VIRGIN

=  0 = SAME AS LAST CASE                     VIRGIN

51-55    I5    NUMBER OF TALLY GROUPS                       VIRGIN

(REMEMBER NUMBER OF GROUP BOUNDARIES         VIRGIN

IS ONE MORE THAN THE NUMBER OF GROUPS)       VIRGIN

UP TO 2000 GROUPS ARE ALLOWED                VIRGIN

BUILT-IN GROUP STRUCTURES.                   VIRGIN

= GREATER THAN 0 = READ FROM INPUT           VIRGIN

=  0 TART 175 GROUPS                         VIRGIN

= -1 ORNL  50 GROUPS                         VIRGIN

= -2 ORNL 126 GROUPS                         VIRGIN

= -3 ORNL 171 GROUPS                         VIRGIN

= -4 SAND-II 620 GROUPS..1.0D-4 eV TO 18 MEV VIRGIN

= -5 SAND-II 640 GROUPS..1.0D-4 eV TO 20 MEV VIRGIN

= -6 WIMS 69 GROUPS                          VIRGIN

= -7 GAM-I 68 GROUPS                         VIRGIN

= -8 GAM-II 99 GROUPS                        VIRGIN

= -9 MUFT 54 GROUPS                          VIRGIN

=-10 ABBN 28 GROUPS                          VIRGIN

=-11 TART 616 GROUPS TO  20 MeV              VIRGIN

=-12 TART 700 GROUPS TO   1 GeV              VIRGIN

=-13 SAND-II 665 GROUPS..1.0D-5 eV TO 18 MEV VIRGIN

=-14 SAND-II 685 GROUPS..1.0D-5 eV TO 20 MEV VIRGIN

=-15 TART 666 GROUPS TO 200 MeV              VIRGIN

=-16 SAND-II 725 GROUPS..1.0D-5 eV TO  60 MEVVIRGIN

=-17 SAND-II 755 GROUPS..1.0D-5 eV TO 150 MEVVIRGIN

=-18 SAND-II 765 GROUPS..1.0D-5 eV TO 200 MEVVIRGIN

=-19 UKAEA  1102 GROUPS..1.0D-5 eV to   1 GeVVIRGIN

56-60    I5    NUMBER OF POINTS IN SOURCE SPECTRUM          VIRGIN

(MUST BE AT LEAST TWO POINTS)                VIRGIN

= GREATER THAN 1 = READ FROM INPUT           VIRGIN

=  0 = SAME AS LAST CASE                     VIRGIN

= -1 = CONSTANT (ENERGY INDEPENDENT)         VIRGIN

= -2 = 1/E                                   VIRGIN

= -3 = BLACKBODY - PHOTON SPECTRUM           VIRGIN

= -4 = BLACKBODY - ENERGY SPECTRUM           VIRGIN

= -5 = TRANSMITTED SPECTRUM FROM LAST CASE   VIRGIN

NOTE, ALL SPECTRA, EXCEPT THE TRANSMITTED    VIRGIN

SPECTRUM FROM THE LAST CASE, WILL BE         VIRGIN

NORMALIZED SUCH THAT ITS INTEGRAL OVER       VIRGIN

ENERGY WILL BE UNITY.                        VIRGIN

61-64  1X,3I1  SPATIALLY DEPENDENT OUTOUT                   VIRGIN

= 0 = NO                                     VIRGIN

= 1 = YES                                    VIRGIN

FOR THE 3 QUANTITIES                         VIRGIN

COLUMN 67 FLUX                               VIRGIN

68 REACTIONS                          VIRGIN

69 AVERAGE CROSS SECTION              VIRGIN

65-65    I1    ENERGY DEPENDENT OUTOUT                      VIRGIN

= 0 = NONE                                   VIRGIN

= 1 = INCIDENT SPECTRUM                      VIRGIN

= 2 = TRANSMITTED SPECTRUM                   VIRGIN

= 3 = INCIDENT REACTIONS                     VIRGIN

= 4 = TRANSMIITED REACTIONS                  VIRGIN

= 5 = TOTAL CROSS SECTION                    VIRGIN

= 6 = REACTION CROSS SECTION                 VIRGIN

5   1-11   E11.4  BLACKBODY TEMPERATURE IN eV                  VIRGIN

12-22   E11.4  FLUX NORMALIZATION                           VIRGIN

23-33   E11.4  REACTION NORMALIZATION                       VIRGIN

CALCULATIONS WILL BE BASED ON THE SPECTRUM   VIRGIN

AND CROSS SECTIONS AS READ. AT OUTPUT THE    VIRGIN

RESULTS WILL BE MULTIPLIED BY THESE          VIRGIN

NORMALIZATION FACTORS.                       VIRGIN

34-44    I11   DENSITY PROFILE                              VIRGIN

= 0 - UNIFORM - BASED ON TOTAL DENSITY       VIRGIN

= 1 - UNIFORM - TOTAL + REACTION DENSITY     VIRGIN

= 2 - TOTAL + LINEAR REACTION                VIRGIN

= 3 - LINEAR (TOTAL + REACTION)              VIRGIN

= 4 - TOTAL + SQUARE REACTION                VIRGIN

= 5 - SQUARE (TOTAL + REACTION)              VIRGIN

= 6 - TOTAL + CUBIC REACTION                 VIRGIN

= 7 - CUBIC (TOTAL + REACTION)               VIRGIN

6-N   1-66  6E11.4  TARGET THICKNESSES IN CM                     VIRGIN

IF SAME AS LAST CASE THIS SECTION IS NOT     VIRGIN

INCLUDED IN THE INPUT.                       VIRGIN

VARY   1-66  6E11.4  TALLY GROUP ENERGY BOUNDARIES                VIRGIN

(NUMBER OF BOUNDARIES IS ONE MORE THAN       VIRGIN

THE NUMBER OF TALLY GROUPS)                  VIRGIN

IF THE STANDARD OPTION (-14 TO 0) IS         VIRGIN

SELECTED THIS SECTION IS NOT INCLUDED        VIRGIN

IN THE INPUT                                 VIRGIN

VARY   1-66  6E11.4  SOURCE SPECTRUM IN ENERGY (eV)-SOURCE PAIRS  VIRGIN

(MUST BE AT LEAST TWO POINTS)                VIRGIN

IF STANDARD OPTION (-5 TO 0) IS SELECTED THISVIRGIN

SECTION IS NOT INCLUDED IN THE INPUT         VIRGIN

VIRGIN

ANY NUMBER OF CASES MAY BE RUN ONE AFTER ANOTHER.                 VIRGIN

VIRGIN

EXAMPLE INPUT NO. 1                                               VIRGIN

-------------------                                               VIRGIN

CALCULATE THE UNCOLLIDED FLUX AND CAPTURE (MT=102) THROUGH        VIRGIN

30 CM OF IRON (DENSITY 7.87 G/CC). TALLY THE RESULTS USING        VIRGIN

THE TART 175 GROUP STRUCTURE. THE SOURCE WILL BE CONSTANT         VIRGIN

FROM 1 KEV TO 20 MEV. USE THE STANDARD ENDF/B INPUT DATA          VIRGIN

FILENAME.                                                         VIRGIN

VIRGIN

ENDFB.IN                                                          VIRGIN

IRON 0 TO 30 CM THICK.                                            VIRGIN

CONSTANT SOURCE FROM 1 KEV TO 20 MEV.                             VIRGIN

26000    1 7.8700D+ 0 26000  102 7.8700D+ 0     2    0    2 1100 VIRGIN

0.0000D+ 0 1.0000D+ 0 1.0000D+ 0          0 0.0000D+00           VIRGIN

0.0000D+00 3.0000D+01                                            VIRGIN

1.0000D+03 1.0000D+00 2.0000D+07 1.0000D+00                      VIRGIN

VIRGIN

EXAMPLE INPUT NO. 2                                               VIRGIN

-------------------                                               VIRGIN

CALCULATE THE UNCOLLIDED PHOTON FLUX THROUGH A MIXTURE OF SILICON VIRGIN

AND IRON FOR 100 MEV PHOTONS INCIDENT. THE TRANSMISSION WILL BE   VIRGIN

CALCULATED FOR 21 THICKNESSES VARYING BETWEEN 0 AND 1 CM. THERE   VIRGIN

WILL BE ONLY 1 TALLY GROUP SPANNING A VERY NARROW ENERGY RANGE    VIRGIN

NEAR 100 MEV, AND THE SOURCE SPECTRUM WILL BE CONSTANT OVER THE   VIRGIN

SAME ENERGY RANGE. USE THE STANDARD ENDF/B INPUT DATA FILENAME    VIRGIN

BY LEAVING THE FIRST INPUT LINE BLANK.                            VIRGIN

VIRGIN

(THIS IS A BLANK LINE TO USE THE STANDARD INPUT FILENAME)         VIRGIN

100 MEV PHOTONS                                                   VIRGIN

SILICON + 5 % IRON                                                VIRGIN

14000  521 2.30000+ 0 26000  521 1.15000- 1    21    1    2 1000 VIRGIN

0.00000+ 0 1.00000+ 0 1.00000+ 0          1 0.00000+00           VIRGIN

0.00000+00 5.00000-01 1.00000+00 1.50000+00 2.00000+00 2.50000+00VIRGIN

3.00000+00 3.50000+00 4.00000+00 4.50000+00 5.00000+00 5.50000+00VIRGIN

6.00000+00 6.50000+00 7.00000+00 7.50000+00 8.00000+00 8.50000+00VIRGIN

9.00000+00 9.50000+00 1.00000+01                                 VIRGIN

9.99000+ 7 1.00100+ 8                                            VIRGIN

9.99000+ 7 1.00000+ 4 1.00100+ 8 1.00000+ 4                      VIRGIN

VIRGIN

=======================================================================VIRGIN